3GPP TSG RAN WG2#119-e	R2-2209168

e-Meeting, 17th - 29th August, 2022

3GPP TSG RAN WG4 Meeting #104-e 		R4-2214493

Electronic Meeting, Aug. 15 – Aug. 26, 2022

Title:	Reply LS on the UE/TRP TEG framework

Response to:

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2, RAN WG3

Cc:

Contact person:

#### Name:	Qiuge Guo

#### E-mail Address:	guoqiuge@CATT.cn

Attachments: None

# 1	Overall description

RAN4 thanks RAN1 for the LS R4-2211503(R1-2205382) on UE/TRP TEG framework. In RAN4#104e meeting, RAN4 discussed the LS and provided the following feedbacks:

Issue #2: Applicability of an Rx/RxTx TEG RAN4 feedback: RAN1’s understanding on applicability of reported TEG (issue #2) is correct. Issue #5: Maximum number of changes of reports (or changes) of the TEG-SRS association RAN4 feedback: RAN4 has the same understanding on the maximum number of changes of reports (or changes) of the TEG-SRS association (issue #5). Issue #6: Questions on UE Rx/RxTx TEG margins RAN4 feedback: - UE Rx/RxTx TEG margins are provided as LPP signalling parameters out of UE capability signaling. - A single timing error margin value is provided per Rx TEG/RxTx TEG type per measurement instance in a single LPP message, if it has multiple measurement instances. - The timing error margin values for an Rx TEG/RxTx TEG type in different LPP messages can be different. Issue #7: Difference of timing error margin values for Rx TEG and RxTx TEG RAN4 feedback: RAN4 confirm that RAN1’s understanding is correct.

In addition, RAN4 reached the following agreements on TEG:

Applicability of timing error margin of Rx TEG: - For Rx TEG, the applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margin (dependent on PRS/SRS BW) and frequency drift margin. Candidate timing error margin for RxTx TEG: - (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc.

RAN4 kindly asks RAN1/2/3 to take the above information into account in the following work on NR positioning enhancements.

# 2	Actions

To RAN WG1, RAN WG2 and RAN WG3:

ACTION: 	RAN4 kindly asks RAN1/2/3 to take the above information into account in the following work on NR positioning enhancements.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #104bis-e	October 10 – October 18, 2022	Electronic Meeting

TSG RAN WG4 Meeting #105	November 14 – November 18, 2022	Europe