3GPP TSG-RAN WG2 #119bis-e	R2-2209383

E-meeting, October2022

Agenda Item:	6.24.1

Source:	OPPO, Ericsson, Samsung

Title:	Discussion on R4 LS on dual-PA architecture clarification

Document for:	Discussion, Decision

Introduction

This is to discuss the R4 LS of R4-2206503.

Discussion

In the R4 LS, it is required to

Therefore, RAN4 would like to respectfully ask RAN2 to extend the meaning of dualPA-Architecture capability in TS38.306 from Rel-16 if there is no NBC issue.

I.e., the following clarification should be applicable to Rel-17 without concern, yet the question is just whether/how to handle it in Rel-16.

| dualPA-Architecture For band combinations with single-band with UL CA, this field indicates the support of dual PA and dual LO frequencies for FR1, or dual LO frequencies for FR2. If absent in such band combinations, the UE supports single PA and single LO frequency for all the ULs for FR1, or single LO frequency for FR2. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Essentially, the question is whether there exists a UE implementation of dual-PA + single-LO. Our understanding is no, and R4 preference on reusing the dual-PA capability for dual-LO capability reporting justified that understanding at least from R4 perspective. I.e., if R2 can confirm this understanding, then there would be no inter-operability issue for R16. And if it can be confirmed as a NBC change, there seems no need to prevent this from R15 either.

The related CRs are provided as [2-4].

- RAN2 confirm there is no NBC issue to extend the meaning of dualPA architecture capability in TS38.306, and agree on CR since Rel-15.

Conclusion

We have the following proposals:

Proposal 1	RAN2 confirm there is no NBC issue to extend the meaning of dualPA architecture capability in TS38.306, and agree on CR since Rel-15.

Reference

- R4-2206503	LS on clarification of dualPA-Architecture capability	OPPO	LS out

- R2-2209381	Correction to definition of dualPA-Architecture capability indication	Ericsson, OPPO, Samsung	Rel-16	38.306	16.10.0	NR_RF_FR1_enh	0812		A

- R2-2209382	Correction to definition of dualPA-Architecture capability indication	Ericsson, OPPO, Samsung	Rel-17	38.306	17.2.0	NR_RF_FR1_enh	0811		A

- R2-2210134	Correction to definition of dualPA-Architecture capability indication	Ericsson, OPPO, Samsung	Rel-15	38.306	15.18.0	NR_RF_FR1_enh	0813		F