3GPP TSG-RAN WG2 #119e	Tdoc R2-2209530

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	6.17.2

Source:	Ericsson

Title:	On LS on activating two TCI states for CORESET#0

Document for:	Discussion

# 1	Introduction

RAN2 received an LS in R1-2208203 from RAN1 about activating two TCI states for CORESET 0.

# 1. Overall Description:

Previously in RAN1 LS R1-2200886 to RAN2, RAN1 stated that there is no restriction in RAN1 on whether enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET #0.

However, in RAN1 #110, RAN1 reached the following agreement that UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS. The agreement from RAN1 #110 is listed as below.

Agreement UE does not expect CORESET#0 to be activated with two TCI states when it is associated with SS#0 for Type 0/0A/2 CSS - Send an LS to inform RAN2 of the agreement

# 2. Actions:

To RAN2

ACTION:

RAN1 would like to respectfully request RAN2 to take the above information into account and update the corresponding specification if needed.

# 2	Discussion

RAN1 is changing their previous view on whether two TCI states can be activated for CORESET 0 or not. Respectfully, RAN2 should update the specifications.

The corresponding MAC CE is specified in TS 38.321 as shown below and it allows activating two TCI states for COREST 0. At least the highlighted sentence needs to be deleted. RAN2 could further discuss if there needs to be a restriction stated related to the CORESET 0.

#### 6.1.3.44	Enhanced TCI States Indication for UE-specific PDCCH MAC CE

The Enhanced TCI States Indication for UE-specific PDCCH MAC CE is identified by a MAC PDU subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size of 24 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

-	CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI state IDi: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

NOTE 1:	The Enhanced TCI State Indication for UE specific PDCCH MAC CE is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

NOTE 2:	The Enhanced TCI State Indication for UE specific PDCCH MAC CE is applied only if sfnSchemePdcch is configured.

Figure 6.1.3.44-1: Enhanced TCI States Indication for UE-specific PDCCH MAC CE

Delete “In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5].” From the field description of CORESET ID in Enhanced TCI States Indication for UE-specific PDCCH MAC CE

Discuss if there needs to be a restriction stated related to the CORESET 0.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Delete “In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5].” From the field description of CORESET ID in Enhanced TCI States Indication for UE-specific PDCCH MAC CE

Proposal 2	Discuss if there needs to be a restriction stated related to the CORESET 0.