3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2209533

E-meeting, Oct. 10th – Oct. 18th, 2022

Agenda Item:	8.11.2

Source: 	Canon Research Centre France

Title:	MBS pre-configuration and PTM configuration in RRC_INACTIVE state

Document for:	Discussion

# Introduction

In this paper, we address one of the objectives of the work item Release 18 for enhancements of NR Multicast and Broadcast Services (MBS), as described in the WID [1]:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3]

In the previous RAN2#119e meeting [2], the following agreements have been made:

In Rel-18, multicast reception for UEs in INACTIVE supports at least the following scenarios, with the assumption that the UE already has a valid PTM configuration:

Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception.

Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session

FFS for state changes, e.g. due to service being not provided in INACTIVE anymore etc.

HARQ feedback and PTP are not supported for multicast reception in RRC_INACTIVE.

For PTM configuration delivery, RAN2 further investigates the following solutions:

Option 1: Dedicated signalling

Option 2: Solution based on SIB+MCCH

We do not preclude some “mix” of the options

Multicast service continuity after cell reselection in RRC_INACTIVE state (i.e. without resuming RRC connection) will be supported (if the configuration of the new cell is available for the UE). FFS whether there are cases where the UE needs to resume the connection. FFS RAN3 impacts due to inter-gNB mobility.

Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

# Discussion

In Release 17, the MBS multicast session follows state transitions as defined in 3GPP TS 23.247 v17.3.0 [2], figure 4.3-1. The session management procedures include session creation (7.1.1.2 or 7.1.1.3 of TS 23.247), session activation (7.2.5.2 of TS 23.247), session establishment and join (7.2.1.3 of TS 23.247). It is only when the MBS session is established that the network will configure the UE for the reception of multicast data.

The establishment of an MBS session in a UE is performed when the UE is in RRC_CONNECTED state. Then, the network may decide to switch the UE to RRC_INACTIVE state before the MBS multicast data are available through the session activation. When the session is activated the network will have to notify the UEs that have joined the session, and the network will have to switch the UEs that are in RRC_INACTIVE state to RRC_CONNECTED state to configure them.

Observation 1: The Release 17 specifications do not provide a procedure to configure a UE in RRC_INACTIVE state. A UE in RRC_INACTIVE state shall be resumed to RRC_CONNECTED state when an MBS session is activated.

Based on Release 17, at MBS session activation, all UEs belonging to the multicast group shall be configured at the same time.

Observation 2: For a large number of UEs, the delivery of MBS configuration at MBS multicast session activation may lead to link congestion and delay.

## MBS pre-configuration

To avoid a massive MBS configuration at MBS session activation, it is proposed to pre-configure the UEs before the MBS session activation.

MBS configuration may be delivered at different stages:

- At MBS session establishment (or after) when the UE is in RRC_CONNECTED state,

- When a UE is switched to RRC_INACTIVE state to provide PTM configuration to receive the multicast data in RRC_INACTIVE state,

- When a UE is already in RRC_INACTIVE state, e.g. for PTM configuration update.

Then, at MBS multicast session activation, the UEs are notified by the network and they can apply the stored MBS configuration to start receiving the multicast data.

The UEs being pre-configured at different moments (before the session activation), the network load may be alleviated at the MBS session activation.

Proposal 1: It is preferred that the network provides MBS multicast configuration to the UEs in the multicast group before MBS session activation.

For the UEs that can receive MBS multicast data in RRC_INACTIVE state, the network may provide the PTM configuration(s) for some cells (e.g. neighbor cells) or for all cells in the MBS service area. The advantage for a UE in RRC_INACTIVE state receiving MBS multicast data is to be able to switch to a new cell without notifying the network to request a configuration. After performing the cell reselection process and switching to a new cell, the UE can apply the stored PTM configuration corresponding to the new cell.

Proposal 2: When a UE can receive MBS multicast data in RRC_INACTIVE state, the network may provide to the UE the PTM configuration(s) for several cells in the neighborhood.

## Notification of MBS Session Activation

At MBS session activation, the network may perform group paging indicating the activation of an MBS multicast session. A UE in RRC_INACTIVE state that has been pre-configured may directly apply the stored PTM configuration without notification to the network.

The UE may however notify the network to indicate its presence. For this purpose, the UE may send a RRC Resume Request message and the network may respond with a RRC Release with suspendConfig message to maintain the UE in RRC_INACTIVE state. Besides, this notification from UEs enables a gNB to estimate the number of UEs effectively receiving in RRC_INACTIVE state.

As an alternative method to group paging, the network may indicate the activation of MBS session activation through SIB.

Proposal 3: UEs in RRC_INACTIVE state may receive the notification of MBS session activation through group paging or SIB

It can be noted that whether the multicast session can be received in RRC_INACTIVE state is implicitly indicated to the UE by receiving from the network a PTM configuration applicable in RRC_INACTIVE state. The explicit indication whether multicast can be received in RRC_INACTIVE state through paging may not be necessary.

## PTM configuration delivery in RRC_INACTIVE state

Two options are considered for PTM configuration delivery in RRC_INACTIVE state: using a dedicated signaling or based on SIB plus MCCH. However, providing PTM configurations using MCCH channel presents some drawbacks related to security (about the authorization to receive the multicast data). Furthermore, the MCCH content may be transmitted periodically and thus, it is not efficient in terms of bandwidth.

Proposal 4: PTM configuration should be provided to a UE in RRC_INACTIVE state via a dedicated RRC signaling.

When a UE is switched to RRC_INACTIVE state, the network may provide PTM configuration through the RRC Release with SuspendConfig message. Such PTM configuration delivery can be performed before MBS session activation (i.e. pre-configuration) or after MBS session activation. This message may include the PTM configuration for one or several cells in the MBS service area.

UE

gNB

5GC

RRC_INACTIVE

MBS Session Establishment

Join request

RRC_CONNECTED

RRC Release with SuspendConfig (multicast Pre-configuration)

Switch to RRC_INACTIVE state

Figure 2-1. Illustration of pre-configuration case when switching to RRC_INACTIVE state

The process shown in Figure 2-1 illustrates the PTM configuration delivery when switching to RRC_INACTIVE state.

After MBS session establishment, the gNB may decide to switch a UE to RRC_INACTIVE state (it may be decided with assistance information from the core network). At this stage, the gNB may decide to pre-configure the UE to be able to receive the multicast data while in RRC_INACTIVE state, by sending a RRCRelease message with SuspendConfig including the suitable PTM configuration. The UE is then aware that it is enabled to receive multicast data in RRC_INACTIVE state

In the same way, when the network has to configure a UE already in RRC_INACTIVE state, the RRC Release with SuspendConfig message may also be used to provide PTM configuration(s) while maintaining the UE in RRC_INACTIVE state. This may concern the following cases:

1) when the configuration is triggered by the UE, e.g. at cell reselection when the UE is receiving multicast data and does not have the PTM configuration for a target cell,

2) when the configuration is at the initiative of the network.

In the first case (configuration triggered by the UE), the UE may send a RRC Resume request to the network (before or after cell switching) indicating the need for one or several PTM configuration(s) for a given MBS multicast session. Then, the network may respond with the RRC Release with SuspendConfig message including the requested PTM configuration(s).

In the second case (initiated by the network), the network may perform group paging announcing the availability of PTM configuration(s) for a MBS multicast session. Then, a UE interested in the MBS session may send a RRC Resume Request message to notify its presence and its interest in the MBS session. Finally, the network may send a RRC Release with SuspendConfig message including PTM configuration(s). This procedure may be performed at any time, before or after the MBS session activation (e.g. for an update). It may also be performed at the MBS session activation, meaning that the group paging would contain both the notification of MBS session activation and the notification of PTM configuration.

Proposal 5: PTM configuration(s) may be provided to a UE being switched to RRC_INACTIVE state or already in RRC_INACTIVE state using RRC Release with SuspendConfig message.

In both cases, the network may provide the PTM configuration(s) for some cells (e.g. neighbor cells) or for all cells in the MBS service area.

Also, in both cases, the UEs may be maintained in RRC_INACTIVE state (without switching to RRC_CONNECTED state).

Proposal 6: A UE in RRC_INACTIVE state may receive PTM configuration(s) without being switched to RRC_CONNECTED state.

Before sending PTM configuration(s) to a UE, the network should check that the UE belongs to the multicast group (i.e that it has joined the MBS session and is authorized to receive the multicast data).

As an alternative method to group paging, the network may indicate the activation of MBS session activation through SIB.

Proposal 7: Group paging or SIB may be used to notify UEs in RRC_INACTIVE state about the availability of PTM configuration(s).

# Conclusion

In this contribution, we propose solutions to pre-configure UEs before MBS multicast session activation, and to enable UEs to receive multicast data while staying in RRC_INACTIVE state. The following observations and proposals have been made:

Observation 1: The Release 17 specifications do not provide a procedure to configure a UE in RRC_INACTIVE state. A UE in RRC_INACTIVE state shall be resumed to RRC_CONNECTED state when an MBS session is activated.

Observation 2: For a large number of UEs, the delivery of MBS configuration at MBS multicast session activation may lead to link congestion and delay.

Proposal 1: It is preferred that the network provides MBS multicast configuration to the UEs in the multicast group before MBS session activation.

Proposal 2: When a UE can receive MBS multicast data in RRC_INACTIVE state, the network may provide to the UE the PTM configuration(s) for several cells of the MBS service area.

Proposal 3: UEs in RRC_INACTIVE state may receive the notification of MBS session activation through a group paging or a SIB

Proposal 4: PTM configuration should be provided to a UE in RRC_INACTIVE state via a dedicated RRC signaling.

Proposal 5: PTM configuration may be provided to a UE being switched to RRC_INACTIVE state or already in RRC_INACTIVE state using RRC Release with SuspendConfig message.

Proposal 6: A UE in RRC_INACTIVE state may receive PTM configuration(s) without being switched to RRC_CONNECTED state.

Proposal 7: Group paging or SIB may be used to notify UEs in RRC_INACTIVE state about the availability of PTM configuration(s).

# References

[1] RP-221458 Enhancements of NR Multicast and Broadcast Services - Work Item Description

[2] RP-222388 Status Report on NR MBS