3GPP TSG RAN WG2 Meeting #119bis-e                                                        	R2-2209906

E-Meeting, 10th – 19th October, 2022

Agenda item:	8.16.2

Source:	Samsung

Title:	AI/ML Capability Indication

Document for:	Discussion and Decision

# 1. Introduction

In RAN#96 meeting, RAN agreed the Rel-18 revised study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1]. The following are selected objectives of this SID:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels […] For the use cases under consideration: […] - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - .. - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case

In this contribution we discuss introducing AI/ML capability exchange between the UE and the network.

# 2. Discussion

In RAN1#110 meeting, RAN1 discussed the general aspects of AI/ML framework, terminologies, gNB-UE collaboration levels, and evaluation and other aspects of AI/ML for CSI feedback enhancement, beam management and positioning accuracy enhancement use cases.

More specifically, RAN1 discussed the need to define AI/ML capability for the UE (or network), considering the previously agreed network-UE collaboration levels [2]:

- Level x: No collaboration

- Level y: Signalling-based collaboration without model transfer

- Level z: Signalling-based collaboration with model transfer

In conclusion, RAN1 agreed that more study is need on defining UE capability to support AI/ML operation and the related specification impact [3].

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

Observation 1: RAN1 will study the UE AI/ML capability reporting and any related specification impact.

RAN1 has discussed aspects of AI/ML model transfer, model training and inference location, considering the three use cases. For example, joint training (i.e. two-sided model) or separate training (i.e. training model at network-side or UE-side) for the CSI compression use case [3].

Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

In our understanding, it is beneficial to introduce and exchange AI/ML capability between the UE and network in order to allow different collaboration levels between the UE and the network. Moreover, the AI/ML capability reporting may be indicated as a general AI/ML capability or use case specific AI/ML capability (e.g. CSI feedback enhancement, beam management, or positioning accuracy enhancement). Similarly, the network may indicate to the UE information on it supported AI/ML models, or model training and inference location.

Observation 2: It is beneficial to define and exchange AI/ML capability between the UE and the network.

Observation 3: The AI/ML capability reporting may indicate a general or use case specific AI/ML capability.

Proposal 1: RAN2 to discuss introducing new capability indication for AI/ML.

# 3. Conclusion

In this contribution we discussed introducing AI/ML capability exchange between the UE and the network. The following are the observations and proposal in this document:

Observation 1: RAN1 will study the UE AI/ML capability reporting and any related specification impact.

Observation 2: It is beneficial to define and exchange AI/ML capability between the UE and the network.

Observation 3: The AI/ML capability reporting may indicate a general or use case specific AI/ML capability.

Proposal 1: RAN2 to discuss introducing new capability indication for AI/ML.

# 4. Reference

RP-221348 Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

RAN1#109-e Chairman report.

RAN1#110 Chairman report.

R1-2208178 Summary#1 of General Aspects of AI/ML Framework