3GPP TSG-RAN WG2 #119bis-e 	R2-2210135

Electronic meeting, 10th –19th October 2022

Agenda Item:	8.1.2

Source:	NEC

Title:	Control plane signaling and procedures of network-controlled repeater

Document for:	Discussion, Decision

Introduction

The Rel-18 study item on NR network-controlled repeaters (NCR) has been finished and the report is in TR38.867[1]. Consequently, a WI was agreed in RP-222673 at RAN#97e meeting. It includes following objectives:

Specify control plane signalling and procedures [RAN2, RAN1] - The configuration of signalling for side control information indication - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2] - NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability. Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4]

This contribution provides our views on necessary control plane signaling and procedures to support NCR operations.

Discussion

General

The Network-controlled repeater is modelled as Fig.1 [1], which includes the NCR-MT and NCR-Fwd.

Fig.1 Network-controlled repeater (NCR)

- The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface.

- The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB.

In general, the following two phases of the NCR can be defined based on whether NCR-Fwd starts its amplify-and-forwarding of UL/DL RF signal or not from network perspective.

Initiation phase

- During initiation phase, NCR-MT starts its registration/attach under the cell and NCR-Fwd doesn’t start the amplify-and-forwarding of UL/DL RF signal.

Forwarding phase

- After NCR-MT finishing the initial access phase (including necessary identification, authentication, security procedure), NCR-Fwd starts the amplify-and-forwarding of UL/DL RF signal under the gNB control, i.e., NCR-Fwd starts to amplify and forward the received signal from backhaul link to access link. Forwarding phase also includes two aspects, forwarding of broadcast and cell-specific signals/channels and forwarding of UE-specific data.

During the study phase, 4 solutions on repeater management were proposed and summarized in TR38.867. The simplified call flows of 4 solutions with initiation and forwarding phases are shown in Fig.2. It is observed that solution 1, 2 and 3 have a common initiation phase whereas all 4 solutions have a common forwarding phase.

- Fig.2 The simplified call flows of 4 solutions

In general, two phases of the NCR, initiation phase and forwarding phase can be defined based on whether NCR-Fwd starts amplify-and-forwarding of UL/DL RF signal or not.

A common initiation phase can be applied for NCR management solution 1, 2 and 3 whereas a common forwarding phase can be applied for all 4 NCR management solutions.

Details of initiation phase (excluding necessary identification, authentication, security procedure since they are pending for waiting for other groups reply) and forwarding phase will be discussed in clause 2.2 and 2.3, respectively.

Initiation Phase

During initiation phase, NCR-MT starts its registration/attach under the cell and NCR-FWD doesn’t start amplify-and-forwarding of UL/DL RF signal. In other words, initiation phase is a preparation phase for forwarding phase, NCR-MT needs to set up RRC connection firstly to receive side control information from gNB during the initiation phase.

- 2.2.1 PLMN selection

Once an NCR powers on, NCR-MT will act as a normal UE to execute Idle mode procedures, i.e., PLMN selection (at lease when NAS protocol is supported) and cell (re-) selection for camping on a suitable cell. As a network node deployed by a certain operator, PLMN selection procedure would be useful to limit the NCR only try to access and expand the coverage from the right network nodes.

(At least for solutions supports NAS protocol) Legacy PLMN selection should be supported for NCR-MT.

- 2.2.2 NCR-MT cell (re-)selection

The followings are captured in TR38.867,

Additionally, at least one of the NCR-MT’s carrier(s) should operate in the frequency band forwarded by the NCR-Fwd. And the NCR-MT and NCR-Fwd operating in the same frequency band is prioritized for the study.

We observe that control link (between NCR-MT and gNB) and backhaul link (between NCR-Fwd and gNB) operating in the same frequency band would be a basic scenario for NCR deployment. It means that RX level of control link and backhaul link are highly relevant, i.e, the RX level of backhaul link can be estimated from that of control link.

Since the control link (C-link) between NCR-MT and gNB is based on NR Uu interface, basically the NCR-MT cell (re-)selection follows cell selection criterion S. However, NCR is a kind of network node deployed by operators. Operators may have individual requirements on the NCR cell design. So, it would be beneficial and desirable to have an NCR-specific minimum required RX level in the cell which is different from UE perspective.

RAN2 to discuss whether it is beneficial and desirable to have an NCR-specific minimum required RX level in the cell which is different from UE perspective.

- 2.2.3 Cell Reservations and Access Restrictions

Cell Reservations and Access Restrictions (i.e., unified access control) are two mechanisms which allow an operator to impose cell reservations or access restrictions to UE. The first mechanism uses indication of cell status and special reservations for control of cell selection and reselection procedures, while the second allows preventing selected access categories or access identities from sending initial access messages for load control reasons.

Different from UE, NCR is a network node for coverage extension. It should not be subjected to load control or linked to any specific services. Moreover, whether an NCR node is allowed to access a cell, should be done via NCR validation/authorization. Hence it should be allowed to connect to gNB without considering access restriction and unified access control.

Similar handling is captured for IAB node as below:

NOTE 0: IAB-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse and intraFreqReselection (i.e., treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. IAB-MT also ignores cellReservedForOtherUse for cell barring determination (i.e., NPN capable IAB-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3].

Unified Access Control does not apply to IAB-MTs.

RAN2 to discuss that NCR-MT ignores access restriction and unified access control as same as IAB.

- 2.2.4 NCR Capability Report for forwarding phase

NCR data forwarding specific capabilities must be reported to gNB before gNB delivering NCR side control information, e.g., antenna and or beaming forming capabilities on access link, and simultaneous UL or DL transmission on both C-link and backhaul link which is captured in TR38.867:

For the transmission/reception of C-link and backhaul link by NCR, - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way - The UL of C-link and UL of backhaul link can be performed in TDM way. The multiplexing is under the control of gNB with consideration for NCR capability and simultaneous transmission of the UL of C-link and UL of backhaul link is also subject to NCR capability

Most capabilities themselves would be discussed in RAN1 since it is RF or physical layer relevant, however, RAN2 could discuss how gNB can be aware of these NCR specific or NCR-Fwd specific capabilities, there are two potential options:

- One is to add them into current capability signalling, use the current capability enquire, report, storing and retrieve framework

- Another one is to have a separate signalling framework

As normal UE would frequently transit between RRC connected mode and RRC idle mode, and potential change the serving cells, so it is reasonable to store all its capabilities in CN and retrieve to gNB during initial context setup procedure. However, this is not the case for NCR with following assumptions

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867 - For only single hop stationary network-controlled repeaters - The NCR is transparent to the UE. - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

So, we see it will be simple that NCR always send data forwarding relevant capabilities directly to gNB upon NCR-MT finishing the initial access to network. There is no need to store its capabilities in CN, consequently there is no need to support capability enquire message.

RAN2 to discuss that NCR UL/DL RF signal forwarding capabilities are reported, stored, and used in gNB. gNB will neither enquire nor send them to CN for storing them in CN.

Forwarding phase

- 2.3.1 Forwarding of broadcast and cell-specific signals/channels

Regarding forwarding of broadcast and cell specific signals and channels, TR38.867 says,

Whether/how to handle the forwarding of broadcast and cell-specific signals/channels can be discussed in normative phase.

From higher layer point of view, to provide coverage extension, at least, NCR-Fwd should forward broadcast and cell-specific signals/channels including SSB, SIB1 and SI(s), paging in downlink and PRACH in uplink. These are necessary for a UE under the extended coverage to see the cell, receive necessary system information, paging, and/or initialize RACH procedure. These signals and channels should be forwarded unless the NCR-Fwd is turn-off by the gNB.

Side control information (if needed) relevant to cell-specific signal/channel should be semi-static. It should be sent to NCR-MT at initialization phase. Considering such side control information is not delay sensitive and NCR-MT RRC layer knows all cell-specific signal and channel’s configurations, so RRC signalling could be used to send side control information relevant to cell-specific signal/channel.

RAN2 to discuss to send side control information (if needed) relevant to cell-specific signal/channels via RRC signalling.

- 2.3.2 NCR-MT states at forwarding phase

As defined in TR38.867, NCR-MT is a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g., side control information). Then it could be in RRC-Connected, RRC-Idle and RRC-inactive mode (if support).

On the NCR-Fwd behavior, TR38.867 further clarifies that

The NCR-Fwd is always expected to be “OFF” unless otherwise explicitly or implicitly indicated by gNB. - Note-1: This applies to the case regardless of the RRC state of NCR-MT. - Note-2: Indication (e.g., received when NCR-MT in RRC-connected) or DRX state of NCR-MT to control the ON-OFF behaviour of NCR-Fwd when the NCR-MT is in RRC-idle/inactive is not precluded. - The above is not meant to imply any signalling design for NCR-Fwd ON-OFF.

RAN2 to discuss whether NCR-Fwd can be “On” when NCR-MT is in either RRC-Idle or RRC-inactive mode, i.e., whether NCR-MT must be in RRC-connected mode when NCR-Fwd is “On”.

Once an NCR-MT connects to a gNB and passes the identification and authorization procedure, we assume that NCR-Fwd would be at least asked to amplify and forward cell-specific signals and channels. If there is no UE connects to the cell via repeater, it is technically possible to neither exchange further side control information nor manage C-link/backhaul link, so NCR-MT’s RRC connection with the gNB could be released.

On the other hand, if we keep NCR-MT in connected mode all time as long as NCR-Fwd is “ON”, then it would be faster for gNB to manage the C-link, backhaul link, and potentially quickly for gNB to send further side control information to update the data forwarding behavior.

As an example, when it is detected that a new UE is initiating RRC connection to gNB via the repeater, further side control information may be required. The delay of sending the required side control information would be shorter when NCR-MT keeps its RRC-Connected mode whereas be longer if NCR-MT is in RRC-idle. Such a delay will be added into the UE initial access delay in turn. Moreover, the UE context of the NCR-MT will be released in gNB once NCR-MT enters RRC idle. Considering the NCR context management and the delay mentioned above, it seems not suitable to release NCR-MT’s RRC connection into RRC-idle mode when NCR-Fwd is ON. On the other hand, suspending NCR-MT’s RRC connection into RRC-inactive mode may have benefits.

NCR-Fwd should be OFF when NCR- MT is in RRC Idle mode.

NCR-Fwd could be “ON” when NCR-MT is in RRC-inactive mode, if RRC-inactive mode is supported by NCR-MT.

Summary

This contribution provides our views on necessary control plane signaling and procedures to support NCR operations.

- Observation 1 In general, two phases of the NCR, initiation phase and forwarding phase can be defined based on whether NCR-Fwd starts amplify-and-forwarding of UL/DL RF signal or not.

- Observation 2 A common initiation phase can be applied for NCR management solution 1, 2

- and 3 whereas a common forwarding phase can be applied for all 4 NCR management

- solutions.

Proposal 1 (At least for solutions supports NAS protocol) Legacy PLMN selection should be supported for NCR-MT.

Proposal 2 RAN2 to discuss whether it is beneficial and desirable to have an NCR-specific minimum required RX level in the cell which is different from UE perspective.

- Proposal 3 RAN2 to discuss that NCR-MT ignores access restriction and unified access control as same as IAB.

- Proposal 4 RAN2 to discuss that NCR UL/DL RF signal forwarding capabilities are reported, stored, and used in gNB. gNB will neither enquire nor send them to CN for storing them in CN.

- Proposal 5 RAN2 to discuss to send side control information (if needed) relevant to cell-specific signal/channels via RRC signalling.

- Proposal 6 RAN2 to discuss whether NCR-Fwd can be “On” when NCR-MT is in either RRC-Idle or RRC-inactive mode, i.e., whether NCR-MT must be in RRC-connected mode when NCR-Fwd is “On”.

- Proposal 7 NCR-Fwd should be OFF when NCR- MT is in RRC Idle mode.

- Proposal 8 NCR-Fwd could be “ON” when NCR-MT is in RRC-inactive mode, if RRC-inactive mode is supported by NCR-MT.

- References

[1] TR38.867