3GPP TSG-RAN WG2 #119bis-e 	R2-2210198

Electronic meeting, 10th –19th October 2022

Agenda Item:	8.7.4

Source:	NEC

Title:	NR NTN connected mode mobility enhancement

Document for:	Discussion, Decision

# Introduction

A new Rel-18 WI on NR NTN (Non-Terrestrial Networks) enhancements was agreed. One of the objectives is to work on NTN to NTN/TN mobility and service continuity enhancement as following

4.1.4 NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

In this contribution we will share our initial consideration on NTN-NTN handover enhancement in Rel-18 for LEO system (including both quasi-earth-fixed cell and earth-moving cell) to reduce the signaling overhead.

# Discussion

## Handover scenarios and issues

In LEO system, satellites are constantly moving around the earth, this will lead to frequent and even burst handovers regardless the UEs are moving or stationary. We first review NTN-NTN handover scenarios.

Earth moving cell scenarios:

As shown in figure 1, for an earth moving cell of diameter 50 km and 1000 km, it takes 6.61 seconds and 132.38 seconds respectively to move out of the original coverage due to satellite movement, in another word, all UEs served by earth moving cell will experience a handover about every 6.61 seconds and 132.38 seconds respectively (neglecting UE movement).

Figure 1: Earth moving cell scenario (Figure 7.3.2.1.6-1 from TR38.821)

It is very possible that these UE handovers out of an earth moving cell cells will hand over into another cell of the same gNB on earth.

The main characteristics/issues of these handovers are:

- The handover load is high (all UEs)

- UEs handover out and in to a cell gradually.

- Both inter and intra-gNB handover are possible.

Feeder link switch over:

According to TR38.821, feeder link switch is to switch the feeder link (SRI) between different NTN GWs toward the same satellite. This may be due to e.g. maintenance, traffic offloading, or (for LEO) due to the satellite moving out of visibility with respect to the current NTN GW. Two GWs likely connect to different gNB. All UEs served by GW1/gNB1 has to handover to GW2/gNB2

Figure 2 Feeder link switch over (Figure 8.7.1.1-1 from TR38.821)

The main characteristics/issue of these handovers are:

- The handover load is high (all UEs)

- handover burst: all UEs handover approximately at the same time.

- likely inter gNB handover.

Service link switch over:

As shown in figure 3, with steerable beam, at first the gNB is providing coverage over a sea area via satellite#1, but when satellite#1 is flying far away and cannot be connected to the gNB on ground anymore, another satellite#2 flying over can replace the statellite#1 and to relay the signal from the same gNB to the same/similar coverage. These relay satellite switching from one to another one will happen from time to time.

Figure 2: service link switch over scenario

At each time of the satellite switching over, as we concluded in Rel-17 discussion, different PCI will be assigned to the cell, all UEs under the same coverage will go through handover procedure but to the same gNB node and even the same radio resource but just being labelled by different PCI potentially.

The main characteristics of these handovers are:

- The handover load is high (all UEs)

- handover burst: all UEs handover at approximate at the same time.

- intra-gNB handover

In summary there seems two main issues in NTN-NTN handover:

- Signalling overhead due to large amount of UE’s frequent handovers

- PRACH collision/handover storm due to all UEs in coverage handover at the same time i.e., handover burst

## Potential solutions

Signaling overhead

L1/L2 mobility solution will be introduced in another Rel18 work item “Further NR mobility enhancements”, where gNB can preconfigure a list of candidate target cells, and L1/L2 signal will be used afterwards to trigger handover. This potentially will reduce the signaling overhead and shorten the interruption. As we can noticed in above scenario analysis, in many cases, NTN-NTN handover will be intra-gNB handover, e.g., in service link switch over scenario, or in earth moving cell scenario where UE handover between two cells from same gNB. L1/L2 solution could be reused for NTN intra-gNB handover scenario to save RRC signaling and reduce interruption.

Proposal 1: L1/L2 mobility could be applied to NTN-NTN handover for signaling overhead reduction

with CHO, since UE will execute handover automatically when handover condition is met, gNB can always send the conditional RRC reconfiguration in advance and at different time for different UEs, this will reduce the signaling overhead storm issue.

To reduce the overhead caused by conditional RRC reconfiguration:

- Group handover is proposed as one candidate solution, where all the handover UEs will share part of the RRC reconfiguration with minimized UE-specific RRC configuration. however, considering their different capabilities and on-going services, different UEs potentially have different UE-specific configuration for HO or CHO, there may not be many configurations actually comment for all UEs.

- In another hand, especially for feeder link or service link switch over cases, all the UEs connected to one cell will handover to another cell all together, so potentially all UEs can keep the same configuration as in source cell, e.g., same RB configuration, MAC configuration and C-RNTI configuration so on. Particularly for the service link switch over case, the same gNB will continue manage all the UEs in the coverage after handover, there is no reason to change the configurations, in fact, in this case, even the L2 reset may not be needed.

Proposal 2: RAN2 study NTN-NTN Handover where UE keep all or most configuration same as in source serving cell

RACH collision/handover storm

In both feeder link and or service link switch over scenario, the main issue is the PRACH collision due to handover storm.

One solution proposed for this issue is to support RACH-less handover.

Without RACH procedure, one potential behavior of UE is to continue using the TA used in source serving cell:

- For both service link and feeder link switch over cases, either feeder link or service link is changed after handover, and consequently UE-gNB RTT will be different before and after handover, the TA used in serving cell will not be usable in targe cell.

- For earth moving cell scenario, if a UE handover into a cell from the same satellite and same gNB (it can be called intra-satellite intra-gNB handover), the TA will be kept same after handover. However in this scenario, since UE gradually handover out/in, the handover burst and PRACH collision issue is not so serious.

Without RACH procedure, another potential behavior of UE is use the estimated TA (open loop TA) after handover:

- NTN UE is equipped with GNSS, it can do TA compensation, however RAN1/RAN4 did not agree to not skip RACH in Rel-17. We doubt that RAN1 and RAN4 now can conclude that RACH is not necessary anymore for TA purpose. But in any case, RAN2 cannot make the decision.

If RACH is anyway necessary, to solve the PRACH collision issue, we need to stagger the transmission of PRACH from different UEs for HO. Random backoff does not sound a good idea. some necessary configuration from network is preferred to increase the network control on which UEs could access target cell earlier and which UEs do a bit later.

Proposal 3: RAN2 should discuss to stagger the PRACH transmissions for handover

# Summary

Regarding NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead, we discussed the following scenarios

- Earth moving cell

- Feeder link switch over

- Service link switch over

Furthermore, we observe following main issues in NTN-NTN handover:

- Signalling overhead due to large amount of UE’s frequent handovers

- PRACH collision/handover storm due to all UEs in coverage handover at the same time i.e., handover burst

To tackle these issues, we propose:

Proposal 1: L1/L2 mobility could be applied to NTN-NTN handover for signaling overhead reduction

Proposal 2: RAN2 study NTN-NTN Handover where UE keep all or most configuration same as in source serving cell

Proposal 3: RAN2 should discuss to stagger the PRACH transmissions for handover