3GPP TSG-RAN WG2 Meeting #119bis electronic						R2-2210219

Online, August, 2022

Agenda Item:	8.8.2

Source: 	Sony

Title:	Considerations about UAV mobility and user consent

Document for:	Discussion

# Introduction

RANP revised and earlier agreed WID on UAV in [1] by mainly adding a new objective on CHO:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 2. Study and specify UAV-specific CHO triggering, e.g. using event H1/H2 [RAN2]. 3. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 4. Study and specify, if needed, enhancements for UAV identification broadcast [RAN2, SA2]. Applicable to both LTE and NR Note: This study should consider existing techniques based on Uu or non 3GPP technologies, or unlicensed band as the baseline. Note: This description is a placeholder for a more detailed objective to be drafted once SA2 will have concluded their study on the architectural aspects. 5. Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]: - FR1 with directional antenna at UE side

RAN2_119e agreements:

1 Use LTE principle as a baseline, introduce similar event H1 (aerial UE height become higher than threshold) and H2 (aerial UE height become lower than threshold. FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/benefit of the scaling and how) FFS how to limit excessive measurements and measurement reporting FFS if user consent is needed for location reporting in CONNECTED FFS study the vertical movement and associated mobility for UAV UEs

In this contribution we look into FFS from last meeting and especially the need for separate RRM parameters &CHO, and user consent for location reporting.

# Discussion

UAV mobility and measurement reporting

RAN2_119e agreed to support LTE events H1 and H2 for UAV in NR. There is FFS if further NR enhancements are needed. FFS study scaling of RRM parameters (e.g. which parameters and what is the purpose/ benefit of the scaling and how).

A UAV UE may be handed over to a cell supporting UAV features once UAV is hovering above a certain height (i.e. event H1). Then, UAV UE may be handed over to a terrestrial cell once it is hovering below a certain height (i.e. event H1). This allows flexibility in network deployment and one option to achieve by configuring CHO execution criteria based on event H1/H2.

Proposal 1: allow CHO execution condition based on event H1/H2

Another aspect to consider is if different values of Time-To-Trigger can be used based on the altitude of UAV. In one example, network may configure to use a normal value when UAV is near to earth surface (event H2) and a shorter value when UAV is hovering above a certain altitude (event H1) encountering multiple cells with steady radio conditions and there are less chances of ping-pong.

Proposal 2: allow different values of TTT being configured based on if UE has already triggered event H1/H2

User consent

User consent was discussed at length during NTN discussions in Rel-17 and based on the latest LS from SA3, in our understanding, user consent is left for the network entities to handle. A UE shall provide its location if network configured to provide location information. We think that RAN2 can work on this working assumption unless there is a new input from SA3 in Rel-18.

Proposal 3: RAN2 does not work on User consent i.e. UE provide its location if configured by the network to do so.

# Conclusion

We propose RAN2 to agree on the following proposal:

Proposal 1: allow CHO execution condition based on event H1/H2

Proposal 2: allow different values of TTT being configured based on if UE has already triggered event H1/H2

Proposal 3: RAN2 does not work on User consent i.e. UE provide its location if configured by the network to do so.

# References

[1]	RP-222171: New WID: Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)