3GPP TSG-RAN WG2 Meeting #119bis Electronic	R2-2210353

Elbonia, 10 – 19 October 2022

Agenda item:	8.7.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Further view on Idle- and Connected-mode NTN mobility in Rel-18

WID/SID:	NR_NTN_enh – Rel-18

Document for:	Discussion and Decision

# 1	Introduction

The following WI objective has been agreed for Rel-18 NTN WI [1]:

4.1.4 NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

At RAN2#119 the following decisions have been made [2]:

Agreements: 1. RAN2 to work on a solution so that measurements for TN’s coverage are performed only when relevant (FFS what relevant means). 2. RAN2 to work on assistance information that can be provided to NTN UEs for the above. 3. Cell reselection enhancements (for both NTN-NTN and NTN-TN mobility) are considered for both Earth-moving and (quasi-)Earth-fixed scenarios, at least via the use of system information for broadcasting necessary parameters (dedicated signalling is not precluded). FFS whether the same or different solutions are used for Earth-moving and (quasi-)Earth-fixed scenarios

In this paper we outline our further views regarding Rel-18 mobility/service continuity enhancements, for both TN-NTN and NTN-NTN scenarios and for IDLE and CONNECTED.

# 2	IDLE mode

In the agreement quoted above it is for further study if the same or different solution for cell reselection enhancements is applicable to Earth-moving and Earth-fixed scenarios. The Rel-17 enhancements for quasi-Earth fixed cells has been captured in [3], section 5.2.4.2.

## 2.1 	Intra-NTN case

For quasi-Earth fixed scenario there are already mechanisms available, which allow using UE’s position and time for performing cell reselection-related measurements.

Observation 1: Rel-17 introduced distanceThresh and t-Service which can be used by the UEs in quasi-Earth fixed cell to perform cell reselection measurements.

Similar mechanisms were not supported for Earth-moving cells as it was claimed to be much more difficult to specify these (considering cell footprint’s continuous movement).

Observation 2: using distance threshold or t-Service for cell reselections in Earth-moving NTN scenario is more complex than in quasi-Earth fixed NTN case.

It is difficult to imagine the NTN Earth-moving cell which continuously modifies its reference location (provided in SIB19). Thus, if a similar mechanism to what was agreed in Rel-17 is to be supported, there needs to be different means for that - UE-individual estimations are needed. Each user will have to perform appropriate calculations, considering:

- Satellite’s ephemeris (to know how rapidly and in which direction the satellite moves)

- Cell’s reference location (read from SIB)

- UE’s own location

Proposal 1: UE performs individual estimation, considering satellite’s ephemeris, cell reference location and its own location to enable location-based reselections in Earth-moving scenario.

Similar actions are needed for applying time in cell reselections when Earth-moving NTN scenario is considered. Earth-moving cell may broadcast t-Service, which will be valid for a particular location on Earth. The UE may use such knowledge to compute its own, adjusted t-Service, assuming such IDLE UE is simultaneously capable of assessing its geographical coordinates.

Proposal 2: To enable time-based reselections in Earth-moving scenario, the IDLE UE is capable of computing its own location to adjust t-Service provided in SIB19.

One of comments raised at RAN2#119 indicated such assistance information for the UE does not have to be broadcast, but can be provided via dedicated signalling (see [2]). Dedicated signalling is UE-specific and as we consider here the IDLE mode scenario, such information can be provided only in RRC Release message (when moving the UE from CONNECTED to IDLE). Some assistance information can be included there, but difficult to assume the NW will be able to adjust such cell-specific configuration for each particular UE. For such cases system information is a better choice.

Observation 3: Using RRC Release for providing assistance information for intra-NTN cell reselection is possible, but not optimal (as the configuration is cell-specific).

Proposal 3: System information is the basic means for providing necessary parameters to assist the NTN UE in intra-NTN cell reselection process.

## 2.2 	TN – NTN scenario

A second aspect to be addressed is the cell reselection between TN and NTN. In the previous RAN2 discussions during for NTN Rel 17 it has been recognised that in a scenario with UE which is about to move out from TN coverage, it would be beneficial to start searching for NTN cells only when needed. Vice versa, a UE under NTN coverage could benefit from re-selecting to a TN cell when the UE is located close enough to the TN coverage area. In both scenarios, the unnecessary UE cell search (for TN or NTN) should be avoided to minimize the UE power consumption.

Observation 4: It is beneficial for the power saving purposes if the UE can measure for NTN or TN coverage only when relevant.

One possible solution is to provide assistance information to the UEs. In the simplest form, this assistance information is broadcasted in the camping TN cell for the purpose of reselecting to NTN cells, or in the camping NTN cell for the purpose of reselecting to TN cells.

The assistance information from the camping TN cell could be based on the detection of NTN radio coverage by TN in certain relevant/selected network area. For example, the detection of NTN coverage can be based on simple transmission energy detection and/or reception/decoding of the NTN SIB transmission.

Proposal 4: RAN2 is asked to consider the method of detecting the transmission energy or SIB presence to determine the NTN coverage.

The assistance information from the NTN cell could be based on information about the location of the relevant TN cells. For example, the NTN cell can broadcast one or more distanceThresh values corresponding to relevant TN cells, to indicate to the NTN UEs at which locations they should start searching for TN cells. Another alternative is to use at the NTN side additional information received from the TN in the relevant area, e.g. based on the detection of NTN radio coverage by TN as above.

Proposal 5: RAN2 is asked to consider the method where System Information provides a location-related indication where the search for TN coverage shall be initiated.

# 3	CONNECTED mode

In this section our aim is to cover the following objective [1]:

• Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

In the following subsections we consider various problems for NTN handover management.

## 3.1 	Series of CHOs

One of the techniques to minimize the signalling overhead associated with handover is to allow keeping CHO commands after HO execution. This has been considered in various papers submitted to Rel-17 NTN work item, e.g. in [3]. As in NTN scenario the sequence of next serving cells can be predicted with high probability (assuming the UE does not move significantly, compared to the satellite coverage), we see some value in preparing the UE for multiple cell hops in advance. As shown in Fig. 1 a single satellite may provide coverage of several cells. UE is relatively static, at least compared to the velocity of the satellite, so it is quite likely the UE will be served by cell B directly after being served by cell A.

Fig. 1: Current serving and next prepared cells in the chain of CHOs

In such case, it would make sense if the UE is provided in advance with the cell-specific resources/configurations for one or more next serving cells. Such set of configurations can be also equipped with time validity, in order not to reserve too many resources on the network side much earlier than needed. The expected time of stay can be used for roughly assessing when each UE is expected to access subsequent CHO candidate cells.

Observation 5: UE’s expected time of stay in the cell can be used for avoiding too early resource reservations.

When the UE accesses the next prepared cell (cell B in the example depicted in Fig. 1) it may inform the new serving cell of the presence of CHO chain configuration (e.g. in case the chain has not been properly propagated over the network nodes, including the UE’s unique identifier so that the subsequent cells are able to determine for which users the resources were reserved). That would allow the new serving cell to determine if the chain of CHO configurations is valid or the UE shall be perhaps reconfigured.

Observation 6: When accessing the new cell, UE may report it was configured with the chain of CHO configurations in one of the preceding cells.

We feel this kind of approach is an attractive and efficient addition to NTN system. Thus, it shall be seriously considered in Rel-18 NTN framework as a means to reduce the signalling overhead during the NTN-NTN mobility relying on CHO.

Proposal 6: To reduce the signalling overhead during mobility a mechanism is introduced, where the UE can be provided with CHO configurations for cells beyond the next cell change (future candidate cells).

## 3.2 	Maximize the ToS in the cell

In order to reduce the signalling it is important to maximize the time of stay on the cell/minimize the number of handovers. This is especially challenging when the cells are moving (in addition to UE’s movement). During the Rel-17 discussions, it was underlined that in Earth-fixed scenario, the UE might reselect or handover to a cell which has a larger t-Service (i.e. will provide coverage over certain area for a longer time).

Observation 7: In Earth-fixed case, the UE may reselect or handover to a cell which has a larger t-Service.

However, in Earth-moving case the issue is not straightforward to resolve. If the UE has a choice of multiple cells to which it can reselect (when in IDLE) or handover (when in CONNECTED), it is desirable for the UE to choose a cell whose footprint is not about to disappear shortly after accessing that cell. But in order to be able to do it, the UE must know a bit about the cell layout and how does it move with respect to UE’s current position. This can be in the form of partial/local cell layout data depending on the UE position/cell. Alternatively, the use of a pre-defined set of cell layouts (partial satellite footprint, in simplified form) can be considered. For example, this can be expressed as a list of the relative geometry/location of the beam centers on Earth. Other options are not precluded.

Proposal 7: RAN2 is asked to consider the solution for minimizing the signalling during mobility in EMC, e.g. via informing the UE about the partial cell layout.

## 3.3 	RACH-less Handover

At RAN2#119 it was briefly discussed whether to support RACH-less for NTN [2]. The following has been captured:

R2-2207347 Signaling and congestion reduction in satellite switch Qualcomm Incorporated discussion Rel-18 NR_NTN_enh-Core Proposal 1 RAN2 work on group handover and RACH-less handover to mitigate the signaling overhead in source and target cells. - HW thinks that group HO was considered in the study phase but not further pursued at the end. Also RACH-less HO is not supported in NR NTN - QC thinks there is a cleat benefit from both group HO and RACH-less HO - LGE, vivo, Intel have some concerns with RACH-less - Continue on this in the next meeting

The views were diverged but there are companies clearly favouring RACH-less for NTN, e.g. [5]. The authors of [5] propose to combine RACH-less and group HO, apparently to minimize the overhead related to RACH and HO command signalling. While the goal to minimize the burden related to numerous RA attempts is desirable, we have some doubts how this can be achieved, considering the RACH-less HO, known from LTE Rel-14. As a brief reminder, LTE RACH-less was applicable to small cells only (where Timing Advance of the source and target cell is zero) or to the cases where source’s TA can be reused at target cell.

Observation 8: LTE Rel-14 RACH-less applicability was limited to the cases where TA of the source and target cell is zero or where source’s TA can be reused at target cell.

Naturally, it is a bit challenging to expect TA = 0 scenario to be applicable to NTN, so the only case left (assuming there are no enhancements pursued on top of Rel-14 LTE solution) is the reuse of TA between the source and target. That in turn suggests the only potentially feasible case is to apply RACH-less when source and target cells’ antennas are collocated, i.e. intra-satellite scenario. Alternatively, inter-satellite case with full synchronization between satellites would have to be ensured.

Observation 9: Solution relying on Rel-14 RACH-less can be potentially applicable only to intra-satellite scenario or fully synchronized inter-satellite case.

This may bring gains in selected scenarios (when the UE handovers between the cells from the same satellite), but cannot be applied widely in NTN. As random access is used for multiple purposes besides TA estimation, such as initial UL power adjustment and UL grant reception, the benefits of skipping RA could be even more limited if some other means need to be used for determining UL power and receiving UL grant. As a result, we suggest not to pursue RACH-less HO in Rel-18 NTN and focus on other mobility areas.

Proposal 8: RACH-less HO is not supported in Rel-18 NTN enhancements.

## 3.4 	Group Handover

At RAN2#119 it was also briefly discussed whether to support group HO for NTN [2]. This solution seems to be relevant for the scenario of Earth-fixed cell, where the cell’s coverage disappears at roughly the same time for all UEs served by that cell. In such circumstances, shortly before the cell is switched off, the group of UEs need to be commanded to change the cell. In [5] it is suggested to prepare the UEs in advance (e.g. using the CHO framework) and then trigger CHO using paging or PDCCH with G-RNTI. In our view the problem which group HO is attempting to solve can be already addressed using CHO with time-based triggering. The network can prepare multiple UEs in advance and stagger those preparations to avoid signalling storm. Then, each UE will evaluate corresponding CHO execution conditions, including time-based condition which should reflect the serving cell’s switch-off time. The UE decided to start accessing the target when CHO-related conditions are met (i.e. no further signalling from the source cell is needed). Thus, we suggest the following:

Proposal 9: Group HO is not pursued as a part of Rel-18 NTN enhancements as similar goal can be achieved using CHO with time-based triggering.

# 4	Conclusion

The following observations and proposals have been made in this paper:

Observation 1: Rel-17 introduced distanceThresh and t-Service which can be used by the UEs in quasi-Earth fixed cell to perform cell reselection measurements.

Observation 2: using distance threshold or t-Service for cell reselections in Earth-moving NTN scenario is more complex than in quasi-Earth fixed NTN case.

Proposal 1: UE performs individual estimation, considering satellite’s ephemeris, cell reference location and its own location to enable location-based reselections in Earth-moving scenario.

Proposal 2: To enable time-based reselections in Earth-moving scenario, the IDLE UE is capable of computing its own location to adjust t-Service provided in SIB19.

Observation 3: Using RRC Release for providing assistance information for intra-NTN cell reselection is possible, but not optimal (as the configuration is cell-specific).

Proposal 3: System information is the basic means for providing necessary parameters to assist the NTN UE in intra-NTN cell reselection process.

Observation 4: It is beneficial for the power saving purposes if the UE can measure for NTN or TN coverage only when relevant.

Proposal 4: RAN2 is asked to consider the method of detecting the transmission energy or SIB presence to determine the NTN coverage.

Proposal 5: RAN2 is asked to consider the method where System Information provides a location-related indication where the search for TN coverage shall be initiated.

Observation 5: UE’s expected time of stay in the cell can be used for avoiding too early resource reservations.

Observation 6: When accessing the new cell, UE may report it was configured with the chain of CHO configurations in one of the preceding cells.

Proposal 6: To reduce the signalling overhead during mobility a mechanism is introduced, where the UE can be provided with CHO configurations for cells beyond the next cell change (future candidate cells).

Observation 7: In Earth-fixed case, the UE may reselect or handover to a cell which has a larger t-Service.

Proposal 7: RAN2 is asked to consider the solution for minimizing the signalling during mobility in EMC, e.g. via informing the UE about the partial cell layout.

Observation 8: LTE Rel-14 RACH-less applicability was limited to the cases where TA of the source and target cell is zero or where source’s TA can be reused at target cell.

Observation 9: Solution relying on Rel-14 RACH-less can be potentially applicable only to intra-satellite scenario or fully synchronized inter-satellite case.

Proposal 8: RACH-less HO is not supported in Rel-18 NTN enhancements.

Proposal 9: Group HO is not pursued as a part of Rel-18 NTN enhancements as similar goal can be achieved using CHO with time-based triggering.

# References

- RP-221819 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements 3GPP TSG RAN Meeting #96 Budapest, Hungary, June 6-9, 2022

- R2-2208701 Report from Break-out session on NR-NTN, IoT-NTN, REDCAP and CE 3GPP TSG-RAN WG2 Meeting #119 electronic, 17-26 of August 2022

- 3GPP Technical Specification 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state (Release 17)

- R2-2202467 Remaining Rel-17 NTN open issues for CONNECTED mode 3GPP TSG-RAN WG2 Meeting #117 Electronic, 21 February – 03 March 2022

- R2-2207347 Signaling and congestion reduction in satellite switch 3GPP TSG-RAN WG2 Meeting #119 electronic, 17-26 of August 2022