3GPP TSG-RAN2 Meeting #119-bis-e	R2-2210410

# Online, 10– 19 Oct, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3545 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on validity duration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2 #119-e, it was agreed that the acquisition of SIB19 in RRC_IDLE/RRC_INACTIVE is up to UE implementation: Agreements: - It is left to UE implementation on how UEs in RRC_IDLE/RRC_INACTIVE re-acquire SIB19 for serving cell’s satellite assistance information The current 38.331 has defined UE behaviour for SIB19 acquisition and T430 maintenance in Section 5.2.2.4.21, it should be made clear that the specified behaviour is only for UEs in RRC_CONNECTED. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in 5.2.2.4.21 that the behaviour is only for UEs in RRC_CONNECTED. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: NTN Inter-operability: No interoperability issue, as it only affects the UE behaviour in RRC_IDLE/RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreements are not correctly captured in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The CR is based on the endorsed NTN RRC CR R2-2209247 with all changes accepted. |  |  |  |  |  |  |  |  |

<Start of Change>

##### 5.2.2.4.21	Actions upon reception of SIB19

Upon receiving SIB19, the UE in RRC_CONNECTED shall:

1>	start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime;

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

<End of Change>