3GPP TSG-RAN WG2 #119bis electronic	R2-2210501

10th – 19th Oct 2022

Online

Source:                    	DENSO CORPORATION

Title:  	C-DRX enhancements for XR-specific power saving

Document for:        	Discussion and decision

Agenda Item:         	8.5.3.1		DRX enhancements

- Introduction

In RAN #95, revised SID of XR enhancements was approved [1], and the following objectives are specified regarding XR-specific power savings.

Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

In this contribution, we discuss the potential C-DRX enhancements for XR-specific power saving.

- Discussion

2.1. Non-integer periodicity

As described in TR38.838 [2], one of the XR service characteristics is that for video frames, the packet arrival rate is determined by the frame generation rate. Accordingly, considering the typical frame rate of XR video (e.g., 60fps, 120fps), it is assumed that the packet arrival periodicity is a non-integer value (e.g., 16.67ms, 8.33ms). On the other hand, existing C-DRX cycle can only be configured to integer values. This mismatch between C-DRX cycle and XR traffic periodicity can lead to additional traffic delay. Figure 1 shows that XR packets with a frame rate of 60fps can arrive outside the DRX on-duration if C-DRX cycle is set to 16ms. In this case, the XR packet cannot be sent until the next DRX on-duration time.

Figure 1: mismatch between C-DRX cycle and XR traffic periodicity

Observation 1:	The mismatch between C-DRX cycle and XR traffic periodicity can lead to additional traffic delay.

The simplest solution to this issue is that C-DRX cycle supports non-integer periodicity. However, this solution seems to lack flexibility and backward compatibility as it requires introducing a new candidate value ​​for each frame rate. Another potential solution is semi-static or dynamic adaptation of C-DRX configuration. For example, this method allows the gNB to adjust the start offset of DRX on-duration to align with XR packet arrival. In details, whether the adaptation is implemented based on predefined rules or dynamic indication such as L1 signalling can be further considered.

Proposal 1:	Semi-static or dynamic adaptation of C-DRX configuration can be considered to align XR packet arrival and DRX on-duration.

2.2. SFN wraparound mismatch

Besides the non-integer periodicity, SFN wraparound is another factor that causes the mismatch between XR application packet arrivals and DRX on-duration. In the current specification, the DRX on-duration timer starts when SFN and subframe number satisfy the relationship represented by the following formula:

[(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) or

[(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset.

In this formula, SFN takes the value of 0~1023 frames and wraps around every 10240ms. Therefore, if the C-DRX cycle is set to a value which cannot be divisible by 10240ms (e.g., 50ms), it causes a problem that the start point of DRX on-duration cannot be kept at a constant cycle. As shown in Figure 2, the gap between two DRX on-durations before and after SFN wraps around is 40ms, even though the C-DRX cycle is configured to 50ms.

Figure 2: SFN wraparound mismatch

This may also be true if the C-DRX cycle is configured to match the XR traffic periodicity. In other words, XR packets can arrive outside the DRX on-duration due to the on-duration shift that occurs when SFN returns to 0. Therefore, it is beneficial to study how to align XR packet arrivals and DRX on-duration considering SFN wraparound.

Observation 2:	XR packets can arrive outside the DRX on-duration due to SFN wraparound, even if the C-DRX cycle is set to match the XR traffic periodicity.

Proposal 2:	Study how to align XR packet arrivals and DRX on-duration considering SFN wraparound.

2.3. Multiple traffic flows

XR services may have multiple traffic flows with different characteristics, such as I-frame and P-frame, video and audio/data. The current C-DRX configuration allows only one DRX group per FR, which may not adapt well to those multiple flows. Therefore, it would be beneficial to provide multiple C-DRX configurations at the same time. C-DRX parameters can be separately configured to fit the characteristics (e.g., periodicity) of each traffic flow, which contributes to power savings. However, XR flow-specific parameters could complicate the operation of the C-DRX timers. For example, if each configuration has drx-InactivityTimer value separately, the UE is required to know in some way which timer to start when a new transmission is indicated by PDCCH. Therefore, it may be necessary to study whether some parameters can be common to multiple C-DRX configurations.

Observation 3:	It would be beneficial to provide multiple C-DRX configurations at the same time to fit the characteristics of each traffic flow.

Proposal 3:	RAN2 to study whether some parameters can be common to multiple C-DRX configurations.

2.4. DRX Retransmission timer

In the current C-DRX, a UE needs to start the DRX retransmission timers after PUSCH transmission and monitor PDCCH for potential retransmission while drx-RetransmissionTimerUL is running. Thus, frequent UL transmission causes the UE have less opportunity to sleep and increases the UE power consumption. This case may be true for UL pose/control traffic in XR services. According to TR38.838 [2], it is assumed that a packet for UL pose/control arrives at UE with short periodicity (4ms). In addition, UL pose/control information has a strict latency requirement (10ms PDB). Even if the UE received a grant for UL retransmission, it may not be useful because the packet has exceeded the PDB requirement. Therefore, for UL pose/control traffic, it might be better to study enhancements to disable the DRX retransmission timers in order to reduce unnecessary PDCCH monitoring.

Observation 4:	It is assumed that a packet for UL pose/control arrives at UE with short periodicity (4ms) and has a strict latency requirement (10ms PDB).

Observation 5:	Frequent UL transmission increases the UE power consumption due to PDCCH monitoring for potential retransmission.

Observation 6:	The retransmission may not be useful for the packet with a strict latency requirement.

Proposal 4:	For UL pose/control traffic, RAN2 to study enhancements to disable the DRX retransmission timers in order to reduce unnecessary PDCCH monitoring.

- Summary and proposal

In this contribution, we discussed the potential enhancements for XR-specific power saving and provided the following observations and proposals.

Observation 1:	The mismatch between C-DRX cycle and XR traffic periodicity can lead to additional traffic delay.

Observation 2:	XR packets can arrive outside the DRX on-duration due to SFN wraparound, even if the DRX cycle is set to match the XR traffic periodicity.

Observation 3:	It would be beneficial to provide multiple C-DRX configurations at the same time to fit the characteristics of each traffic flow.

Observation 4:	It is assumed that a packet for UL pose/control arrives at UE with short periodicity (4ms) and has a strict latency requirement (10ms PDB).

Observation 5:	Frequent UL transmission increases the UE power consumption due to PDCCH monitoring for retransmission.

Observation 6:	The retransmission may not be useful for the packet with a strict latency requirement.

Proposal 1:	Semi-static or dynamic adaptation of C-DRX configuration can be considered to align XR packet arrival and DRX on-duration.

Proposal 2:	Study how to align XR packet arrivals and DRX on-duration considering SFN wraparound.

Proposal 3:	RAN2 to study whether some parameters can be common to multiple C-DRX configurations.

Proposal 4:	For UL pose/control traffic, RAN2 to study enhancements to disable the DRX retransmission timers in order to reduce unnecessary PDCCH monitoring.

- References

[1]	RP-220285, Revised SID: Study on XR Enhancements for NR, Nokia

[2] 3GPP TR 38.838 V17.0.0 (2021-12)