3GPP TSG-RAN WG2 #119bis-e	R2-2210533

Online, 10th – 19th October 2022

Source:                    	DENSO CORPORATION

Title:  	Applicable scenarios for dual Tx/Rx MUSIM devices

Document for:        	Discussion and decision

Agenda Item:         	8.17.2.1	Scenarios

- Introduction

In the WID of Dual Tx/Rx Multi-SIM for NR [1], the following is stated as one of objective.

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE

In the MUSIM discussions for Rel-17, applicable UE architecture was mainly focused to Single-Tx/Rx UE. As mentioned in the above WID, for Rel-18 discussion to support Dual Tx/Rx UE architecture, it would be necessary to introduce the mechanism to indicate preference on temporary UE capability restriction and its removal. In accordance with the agenda of RAN2 #119bis, this paper attempts to investigate about applicable scenarios for the mechanism.

- Discussion

  - Possible state transitions and applicable scenarios

We would like to discuss about possible UE states related to Dual Tx/Rx MUSIM before discussing about the applicable scenarios. Then, we think the following states could be listed up as possible UE states related to Dual Tx/Rx MUSIM.

- State 1a: Communicates with NW A only

- State 2a: Communicates with NW A only but monitors NW B within Rel-17 MUSIM gap

- State 3: Communicates with the both NW A and NW B

- State 1b: Communicates with NW B only

- State 2b: Communicates with NW B only but monitors NW A within Rel-17 MUSIM gap

Figure 1. Communication image of each state

Based on the above assumption, Table 1 shows possible usecase scenarios within the each state transition.

|  | Destination State |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source State |  | 1a (Communicates with NW A only) | 2a (Communicates with NW A only but monitors NW B within Rel-17 MUSIM gap) | 3 (Communicates with the both NW A and NW B) | 1b (Communicates with NW B only) | 2b (Communicates with NW B only but monitors NW A within Rel-17 MUSIM gap) |
|  | 1a |  | UE starts NW B paging monitoring due to device configuration change (user operation, inserting SIM B, etc.) | UE starts data communication with NW B due to user operation or inserting SIM B, etc. | Network switching occurs due to device configuration change (user operation, inserting SIM B /removing SIM A, etc.) | Network switching occurs due to device configuration change (user operation, prioritized NW configuration, etc.) |
|  | 2a | UE stops NW B paging monitoring due to device configuration change (user operation, inserting SIM B, etc.) |  | UE starts data communication with NW B due to user operation or inserting SIM B, etc. UE responses to a paging message | Same as 1a→1b | Same as 1a→2b |
|  | 3 | UE stops data communication with NW B due to device configuration change (user operation, prioritized NW configuration, removing SIM B) or no more application data, etc. | UE stops data communication with NW B due to device configuration change (user operation, prioritized NW configuration) or no more application data, etc. |  | Reverse of 3→1a | Reverse of 3→2a |
|  | 1b | Reverse of 1a→1b | Reverse of 1a→2b | Reverse of 1a→3 |  | Same as 1a→2a |
|  | 2b | Reverse of 1a→2b | Reverse of 2a→2b | Reverse of 2a→3 | Same as 2a→1a |  |

Table 1. Possible usecase scenarios within the each state transition

With the above table, it could be observed that there are some cases where UE transits to/from State 3 (Communicates with the both NW A and NW B) directly from/to State 1a/1b (Communicates with single NW only (without Rel-17 MUSIM gap)). Therefore, the following could be observed and proposed:

Observation 1:	To transit to the state that communicates with the both network A and network B, not always Rel-17 MUSIM gap is needed

Proposal 1:	RAN2 to treat Rel-18 MUSIM capability to be isolated from Rel-17 MUSIM gap capability

  - Possible signallings

In accordance with Table 1, we map possible signallings for each state transition below (Table 2).

Note: The message colored in black is legacy. The message colored in blue is Rel-17 MUSIM signalling. The message colored in red is undefined with current specifications (probably to be discussed).

|  | Destination State |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source State |  |  | 1a (Communicates with NW A only) | 2a (Communicates with NW A only but monitors NW B within Rel-17 MUSIM gap) | 3 (Communicates with the both NW A and NW B) | 1b (Communicates with NW B only) | 2b (Communicates with NW B only but monitors NW A within Rel-17 MUSIM gap) |
|  | 1a | A |  | UAI (gap pref.) | temporary UE capability restriction | - UAI (preferred RRC state) - RRCRelease | - UAI (preferred RRC state) - RRCRelease |
|  |  | B |  |  | RRC conn. (Setup/Reestablishment/Resume) | RRC conn. | - RRC conn. - UAI (gap pref.) |
|  | 2a | A | UAI (gap release) |  | temporary UE capability restriction | - UAI (preferred RRC state) - RRCRelease | - UAI (preferred RRC state) - RRCRelease |
|  |  | B |  |  | RRC conn. | RRC conn. | - RRC conn. - UAI (gap pref.) |
|  | 3 | A | removal of restriction | removal of restriction |  | RRCRelease | RRCRelease |
|  |  | B | RRCRelease | RRCRelease |  | removal(?) of restriction | removal(?) of restriction |
|  | 1b |  | Reverse of 1a→1b | Reverse of 1a→2b | Reverse of 1a→3 |  | Same as 1a→2a |
|  | 2b |  | Reverse of 1a→2b | Reverse of 2a→2b | Reverse of 2a→3 | Same as 2a→1a |  |

Table 2. Possible signallings for each state transition

With the above table, it could be observed that legacy and Rel-17 MUSIM signallings can basically cover the scenarios except transition to/from state 3 (Communicates with the both NW A and NW B, cells colored in yellow). Therefore, the following could be observed and proposed:

Observation 2:	Aside from “temporary UE capability restriction” and “removal of restriction”, no necessity could be observed to define new signallings

Proposal 2:	RAN2 to prioritize the discussion on how to accomplish “temporary UE capability restriction” and “removal of restriction”

Additionally, to investigate the mechanism, we would like to raise an aspect to be considered:

Observation 3:	When UE switches to NW B completely after parallel communication is finished, UE cannot "remove" the restriction if the UE’s capability was not indicated as "restricted" explicitly to NW B side at the start of the parallel communication

Proposal 3:	RAN2 to consider the case where the UE switches to NW B completely after parallel communication is finished

- Summary and proposal

This paper discussed about the applicable scenarios for Dual Tx/Rx Milti-SIM devices for NR. In summary, the followings were observed and proposed:

Observation 1:	To transit to the state that communicates with the both network A and network B, not always Rel-17 MUSIM gap is needed

Proposal 1:	RAN2 to treat Rel-18 MUSIM capability to be isolated from Rel-17 MUSIM gap capability

Observation 2:	Aside from “temporary UE capability restriction” and “removal of restriction”, no necessity could be observed to define new signallings

Proposal 2:	RAN2 to prioritize the discussion on how to accomplish “temporary UE capability restriction” and “removal of restriction”

Observation 3:	When UE switches to NW B completely after parallel communication is finished, UE cannot "remove" the restriction if the UE’s capability was not indicated as "restricted" explicitly to NW B side when parallel communication is started

Proposal 3:	RAN2 to consider the case where the UE switches to NW B completely after parallel communication is finished

- References

[1] RP-220955, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”