3GPP TSG-RAN2 Meeting #119bis-e	R2-2210607

# Electronic meeting, 10th - 19th October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the PPW index |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-9-30 |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: |  | According to current specification, the PPW ID field within the PPW Activation/Deactivation Command MAC CE indicates the index in dl-PPW-PreConfigToAddModList-r17. However, current spec supports the multiple-time addition/release of pre-configed PPW, i.e., there may be multiple dl-PPW-PreConfigToAddModList-r17. For example: Step 1: the AddModList is used to add the PPW 1/2/3 (separately corresponding to index 0/1/2 in the list); Step 2: the PPW 1/2 is released via dl-PPW-PreConfigToReleaseList-r17; Step 3: the AddModList is used to add another PPW 1 One question is what is the index value for the PPW 1 in Step 3. That is, the “index” is unclear. We understand the index is based on the list which is maintained in the UE, e.g., the PPWs in the list are ordered by the addition time, or PPW ID value. We prefer that the PPWs in the list are ordered by the addition time. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the PPW index is the index in the list which is maintained in the UE and is ordered by the addition time. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: PPW activation/deactivation Inter-operability analysis: 1. If the network is implemented according to the CR and the UE is not, error PPW may be activated or deactivated. 2. If the UE is implemented according to the CR and the network is not, error PPW may be activated or deactivated. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Different understanding between UE and gNB will cause error PPW is activated or deactivated. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.42 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 6.1.3.42	PPW Activation/Deactivation Command MAC CE

The PPW Activation/Deactivation Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b.

It has variable size defined as follows (Figure 6.1.3.42-1):

-	numEntry: This field indicates the number of entries N-1 in the MAC CE. 00 indicates that N equals to 2; 01 indicates that N equals to 3 and so on. The length of the field is 2 bits;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	PPW ID: This field indicates the index of the PPW configured on active DL BWP of the Serving Cell identified by the above Serving Cell ID. Index 0 corresponds to the first entry within the list of the PPW configuration which is maintained in the UE and is ordered by the addition time in this BWP, index 1 corresponds to the second entry in the list and so on. The length of the field is 2 bits;

-	A/D: This field indicates the activation or deactivation of the PPW. The field is set to 1 to indicate activation, otherwise it indicates deactivation. The length of the field is 1 bit;

-	R: Reserved bit, set to 0.

Figure 6.1.3.42-1: PPW Activation/Deactivation Command MAC CE

End of change