3GPP TSG-RAN WG2 Meeting #119bis Electronic	R2-2210645

Elbonia, 10 – 19 October 2022

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Coverage Enhancements for NR NTN

WID/SID:	NR_NTN_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In the WID for Rel-18 NR NTN enhancement [1], the objective of coverage enhancements is defined, in which RAN2 is expected to study the need for NTN-specific coverage enhancements for Rel-18 on top of the coverage enhancement WI. Specifically, it is expected to support coverage enhancement for VoIP and low-data rate services for commercial handset terminals. For example, how to improve performance of low-rate VoNR codec including RAN protocol overhead reduction.

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). <omit some text> The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. <omit some text> Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the RAN2 study. - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec.

In RAN2-119 meeting, RAN2 further confirmed only the solutions that address the NTN specific characteristics (e.g., related to propagation delays, coverage loss, satellite movement) should be considered in this WI.

1. RAN2 understands that, based on the WID, only solutions that address the NTN specific characteristics (e.g. related to propagation delays, coverage loss, satellite movement) should be considered. But the identified solutions could then also be applicable to other cases (TN networks). In any case this will be discussed case by case (this understanding is not meant to change the WID description)

In this contribution, we would like to provide our view on potential NTN-specific coverage enhancements and the issues to support VoNR in NTN from RAN2 point of view.

# 2	Discussion

## 2.1	Coverage enhancement for Msg3 repetitions

In Rel-17 NR NTN, it was agreed Msg3 repetition defined in Coverage Enhancement WI should be supported for NTN, while any further enhancement should be studied in Rel-18.

3. Msg3 repetition is supported in Rel-17 NTN. The text proposal in R2-2206207 is adopted as baseline and included in the TS 38.321 Rapporteur CR. No need to send an LS to RAN1 on this.

The PUSCH of Msg3 was found to be a coverage bottleneck for an NR system. For this reason, repetitions of Msg3 were introduced in Rel-17, to increase reliability of Msg3 detection and decoding. The signal repetition framework was kept invariant compared to PUSCH repetitions in RRC connected mode, but new signalling and methods for indication of the number of repetitions for Msg3 were specified.

The main agreements for enabling of Msg3 repetitions include:

- UE requests Msg3 repetitions if the measured SS-RSRP is below an RSRP threshold configured through SIB1

- Request occurs through choice of specific preamble and/or RO in Msg1

- gNB indicates the number of Msg3 repetitions by using two repurposed bits of the MCS field of the RAR UL grant carried in Msg2

The framework developed in Rel-17 seems to work fine for an NTN setting. A specific set of preambles or RO can be reserved and configured by gNB to allow UEs at low RSRP to communicate a request for Msg3 repetitions. However, the need for specific enhancements for optimization of Msg3 repetitions framework in NTN could be further investigated, especially considering specific NTN characteristics.

As mentioned above, Msg3 repetitions are requested and scheduled only if the SS-RSRP measured by the single UE is below a configured RSRP threshold. Typically, setting of such RSRP threshold is based on an estimate of the UL link budget when the DL RSRP is equal to the configured threshold. In other words, if the DL RSRP is below the configured threshold, the UE is likely to deliver an UL RSRP below the necessary RSRP (and SNR) for correct detection of the Msg3 content, hence such a UE may benefit from Msg3 repetitions.

In NTN, due to satellite movement, the average SNR (or DL RSRP) experienced by the single UEs strongly depends on the satellite position compared to Earth, with a larger average SNR when the satellite is at nadir (large elevation angles) and lower average SNR when the satellite is at horizon (small elevation angles). This means that, for a certain configured RSRP threshold for requesting of Msg3 repetitions, the probability that the DL RSRP of a UE is below the threshold is larger for satellite at small elevation angles, compared to satellite at larger elevation angles. Therefore, when the satellite is at small elevation angles, a higher request for Msg3 repetitions is to be expected and hence a larger probability of PRACH collisions for a fixed preamble and RO set configured for request of Msg3 repetitions.

Observation 1. Due to satellite movement, the probability of PRACH collision for a fixed preamble and RO set configured for requesting Msg3 repetitions is larger when the satellite is at small elevation angles.

Since the majority of the path loss will come from the satellite-to-earth propagation, with the elevation angle change, many UE may be “in similar conditions” at most of the time which may cause UEs to request Msg3 repetition at the same time. So, the risk of overloading the preamble and RO sets may increase the RACH collisions. A larger probability of PRACH collisions creates undesired delays in initial access that need to be avoided and addressed in Rel-18. Methods for configuration of a more dynamic set of resources for Msg1 transmissions requesting Msg3 repetitions could for example be discussed to alleviate the issue and maintain the collision probability of Msg1 invariant regardless of satellite position.

Proposal 1: RAN2 to discuss methods for configuration of PRACH resources for requesting of Msg3 repetitions for NR NTN.

## 2.2	Blind Msg3 retransmission for initial Msg3 transmission

For Msg3 coverage enhancement, an alternative solution is to support blind Msg3 retransmission. It is up to NW scheduling strategy on whether the blind retransmission will be scheduled for a UE based on e.g., the NW resource status and the UE’s service requirement for latency. Compared to the coverage enhancement based on Msg3 repetitions, blind Msg3 retransmission can be applied to all UEs without special UE capability requirement.

However, in Rel-17 NTN, blind Msg3 retransmission is supported for Msg3 retransmission while it is not supported for initial Msg3 transmission.

Agreement: Blind Msg3 retransmission is not supported for initial Msg3 transmission in Rel-17 NTN.

As explained in [2], due to long UE-gNB RTT, it was agreed to delay the start of ra-ContentionResolutionTimer by UE-gNB RTT after initial Msg3 transmission (in red in Figure1). Hence it is not possible for NW to schedule blind Msg3 retransmission for initial Msg3 transmission since UE will not monitor PDCCH during that period.

Figure1: ra-ContentionResolutionTimer (re)start

Observation 2: Blind retransmission for initial Msg3 transmission is not supported due to the UE-gNB RTT delay before starting the ra-ContentionResolutionTimer.

In the email discussion [3], majority companies would like to support blind Msg3 retransmission for initial Msg3 transmission for NTN and some companies propose to study it in Rel-18 as part of coverage enhancement objective. But there are different options provided by companies on how to address the issue, for example:

- Option1: Do not stop the ra-ResponseWindow after initial RAR reception

- Option2: Start ra-ContentionResolutionTimer immediately after initial Msg3 transmission

- Option3: Introduce a new timer (and short timer) for blind Msg3 retransmission after initial Msg3 transmission

- Option4: Whether the UE will additionally monitor for a blind Msg3 retransmission grant may be configurable by the network

In our understanding, it is reasonable to further investigate the Pros and Cons for different options and support blind Msg3 retransmissions for initial Msg3 transmission.

Proposal 2: RAN2 to address the issues on how to support blind Msg3 retransmission for initial Msg3 transmission for NR NTN.

## 2.3	VoIP codec adaptation

According to TS 26.114, a UE offering speech communication shall support narrowband, wideband and super-wideband communication. The UE may support AMR, AMR WB and EVS in which different codec modes are supported with different bitrate.

Table1: Codec modes with different codec bitrate for AMR and AMR-WB

|  | Codec mode with different codec bitrate (in kbps) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AMR | 4.75 | 5.15 | 5.9 | 6.7 | 7.4 | 7.95 | 10.2 | 12.2 |  |
| AMR-WB | 6.6 | 8.85 | 12.65 | 14.25 | 15.85 | 18.25 | 19.85 | 23.05 | 23.85 |

Since each VoIP packet is generated with periodicity 20ms, it is obvious that the higher codec bitrate used the more RTP payload will be generated. Correspondingly, the total bits per 20ms will increase. Below is an example to illustrate the TBS to accommodate the VoIP payload as well as RTP/UDP/IP headers and MAC headers with the use of bandwidth-efficient format. When UE’s codec rate changes from 4.75 Kbps to 12.2 Kbps, the required TBS will increase from 192 bits to 336 bits.

Table2: TBS for different Codec mode

| Codec Mode | 4.75 | 5.15 | 5.9 | 6.7 | 7.4 | 7.95 | 10.2 | 12.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits per speech frame | 95 | 103 | 118 | 134 | 148 | 159 | 204 | 244 |
| Payload header and ToC | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| RTP payload (bits) | 105 | 113 | 128 | 144 | 158 | 169 | 214 | 254 |
| RTP payload (bytes) | 13.13 | 14.13 | 16 | 18 | 19.75 | 21.13 | 26.75 | 31.75 |
| Rounded-up RTP payload (bytes) | 14 | 15 | 16 | 18 | 20 | 22 | 27 | 32 |
| Rounded-up RTP payload (bits) | 112 | 120 | 128 | 144 | 160 | 176 | 216 | 256 |
| RoHC for RTP/UDP/IP header (40 bits) | 152 | 160 | 168 | 184 | 200 | 216 | 256 | 296 |
| TBS to accommodate VoIP payload, RoHC and PDCP/RLC/MAC (40 bits) | 192 | 200 | 208 | 224 | 240 | 256 | 296 | 336 |

A higher AMR voice codec rate may provide a higher-definition voice call and accordingly a better user experience. However, radio channel conditions may also impact the selection of codec mode and codec rate. When UE is in a link budget limited situation (poor RF conditions), small TBS should be scheduled to extend the coverage.  Hence from UE’s point of view, the low codec bitrate (instead of high codec bitrate) should be selected in UE to generate small RTP payload which can avoid packet RLC segmentation. On the other hand, a higher codec rate can be used in good radio channel conditions to ensure a better user experience of the VoNR call. So, codec bitrate adaptation based on radio conditions is needed.

The voice codec selection and adjustment were discussed in enhancement of VoLTE in SID [4] and WID[5], RAN2 agreed that the RAN-assisted Codec adaptation mechanism in which NW can send codec bitrate recommendation to UE via DL MAC CE. Furthermore, UE may query the recommended codec bitrate from NW via UL MAC CE. The same design was inherited in NR as specified in 38.321 section 5.18.10 and 6.1.3.20.

Observation 3: RAN-assisted voice codec bitrate adaptation mechanism is supported in TN, in which NW can send codec bitrate recommendation to UE via DL MAC CE and UE may query the recommended codec bitrate from NW via UL MAC CE.

For NR NTN, according to the updated WID[1], the VoIP performance is evaluated in LEO-1200km scenario. The legacy MAC-CE based codec bit rate adaptation may not work well since the UE-gNB RTT delay will be 42ms which is quite larger than TN.

For example,  given UE generate VoIP packet every 20ms, the time distance between the point where UE wants to trigger the codec adaptation due to detected coverage change and the point where UE applies new codec can reach up to 80ms. As illustrated in Figure2, when the UE triggers codec rate increase (e.g., it may be triggered by UE’s detection of better RF condition or requested by peer UE of the call), according to TS26.114 UE has to query the recommended codec bitrate from NW via UL MAC CE and then wait DL RBR (Recommended Bit Rate) MAC CE to trigger further codec bitrate increase. The time distance between UE triggers the codec bitrate increase and UE applies new codec can reach up to 80ms. The time delay on application of higher codec bitrate means the delay to provide a better user experience (a higher-definition voice call) which is not good for system performance. The case seems happen frequently due to satellite movement in high speed (e.g. 7.5km/s) where the pathloss between UE and gNB will change accordingly.

Figure 2: Time delay of codec adaptation for bitrate increase

Observation 4: Due to high UE-gNB RTT, the legacy MAC-CE based codec bitrate adaptation may not work well.

In the study report TR36.750 for VoLTE enhancement SID [4], there are several candidate solutions captured. In our view, the proposed solutions can be the baseline for further enhancements for NTN. Specifically, the broadcast-based bit rate adaptation may need to be further studied for NTN to avoid high latency issue. In this approach, NW broadcast a mapping relationship between recommended bit rates and channel conditions via system information. When initiating VoIP call or during an on-going call, UE can determine the recommended bit rate corresponding to the measured channel conditions and negotiates with the peer UE based on SIP procedure. Obviously, this approach is helpful to save RTT delay in-between the MAC CE exchange.

Proposal 3: RAN2 to further discuss whether broadcast-based bitrate adaptation mechanism should be introduced for NR NTN.

Another issue caused by high UE-gNB RTT is the extended delay of E2E codec negotiation. As described in TS26.114 section 10.7, in case an gNB determines to recommend a UE to modify the bit rate due to e.g. good radio conditions in the uplink or downlink, it may signal a recommended bit rate (via DL MAC CE) to the UE. The UE may use the recommended bit rate as an input to initiate an end-to-end rate adaptation by sending an application layer message (e.g., RTCP or RTP CMR) to the peer UE or the concerning media GW. Accordingly, the peer UE may retrieve a recommended bit rate (e.g. UL query MAC CE) from its serving gNB to ensure end-to-end codec adaptation.

Figure3 below (excerpt from TS26.114) gives the detailed signalling diagrams to describe the end-to-end voice codec adaptation based on ANBR (Access Network Bitrate Recommendation) mechanism defined in RAN.  Take the uplink voice codec adaptation as the example, if a voice codec adaptation resulting in an increase of the media bitrate, the remote terminal (peer node) received application signalling for bitrate adaptation will trigger an adaptation decision. It needs at least one RTT for the ANBRQ and ANBR message(i.e. UL and DL MAC CE) transmission between the terminal and network (e.g., step 7 and 8 in Figure3). Due to the large latency in the NTN system, the codec adaptation procedure will take quite long time, especially when the remote terminal cannot accept the increased bitrate, it may take up to three RTTs to successfully negotiate the increased bitrate. The delay of codec adaptation negotiation may cause the voice packet loss to occur for a longer time in NTN system compared with TN system. Furthermore, the delay of codec adaptation may delay when the higher codec rate can be used in good radio conditions to ensure a better user experience of the VoNR call.

Figure 3  Uplink bitrate increase based on ANBR

Observation 5: Due to high UE-gNB RTT, the latency of E2E Codec negotiation procedure will be extended in NTN.

Observation 6: The delay of codec adaptation negotiation may cause the voice packet loss or delay of applying the adjusted codec rate.

Proposal 4: RAN2 to discuss how to reduce the E2E codec negotiation delay for NR NTN.

## 2.4	VoIP Packet Aggregation

Since the VoIP packet is generated with periodicity 20ms, if packet aggregation is not used, UE may request to transmit each packet every 20 ms which is aligned with the RTP payload generation in application layer. However, if the packet delay budget in RAN side is up to 100 ms, it is feasible for UE to perform packet aggregation which means the UE may request to transmit packets every 40ms, 60ms or even 80ms while it can still meet the packet delay budget with proper scheduling by network. In this case, UE may transmit 2 or 3 or 4 speech frames at one transmission. As discussed in RAN2-119 meeting, there may have two methods to perform VoIP packet aggregation in UE (e.g., in application layer or in RAN layers).

If the packet aggregation is performed in application layer, it means multiple speech frames will be encapsulated in one RTP packet. In this case, the protocol overhead will be reduced significantly. As show in Table3 below, the overhead is reduced from 41.67% to 15.15% of TBS when UE applies the packet aggregation level from 1 to 4. Since the application layer operation is not in the scope of RAN2, other WG(s) should be involved (e.g. SA4).

Table3: Overhead reduction for application layer aggregation

| Codec (kbps) | Application layer aggregation | Inter tx time (ms) | Payload (bytes) | RoHC (bytes) | PDCP (bytes) | RLC (bytes) | MAC (bytes) | TBS (bits) | Overhead % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.75 | 1 | 20 | 14 | 5 | 2 | 1 | 2 | 192 | 41.67% |
| 4.75 | 2 | 40 | 14*2 | 5 | 2 | 1 | 2 | 304 | 26.32% |
| 4.75 | 3 | 60 | 14*3 | 5 | 2 | 1 | 2 | 416 | 19.23% |
| 4.75 | 4 | 80 | 14*4 | 5 | 2 | 1 | 2 | 528 | 15.15% |

Observation 7: Packet aggregation in application layer can reduce the VoNR overhead significantly while it needs to involve in other WG(s).

The other alternative is to perform the packet aggregation in RAN layer(s) which means each speech frame will be encapsulated in one RTP packet while RAN layer will aggregate multiple RTP packets for one transmission. However, the protocol overhead reduction may be less than the one in application layer aggregation since at least the overhead of each RTP header (ROHC header) cannot be reduced. Assuming the MAC layer aggregates at most 4 RTP packets in one transmission, the protocol overhead may reduce from 41.67% to 37.78%. However, the RAN layer packet aggregation is under the scope of RAN side since no application layer change is needed.

Observation 8: Packet aggregation in RAN layer limits the solution to RAN scope though the overhead reduction gain is less than the aggregation in application layer.

In either the VoIP packet aggregation in application layer or RAN layer(s), the NW scheduling interval will increase from 20ms to 40/60/80ms as requested by UE. The large scheduling interval enables the PUSCH repetition with number more than 20 to have a further coverage enhancement to fill the coverage gap. Otherwise (i.e., when the scheduling interval is 20ms), the maximum PUSCH repetition should be limited to 20 to transmit the packets in time. We understand RAN1 is discussing the PUSCH using e.g. 32 repetitions with DMRS bundling for coverage enhancement. The packet aggregation may be mandatory to support the high number of PUSCH repetitions.

Observation 9: Packet aggregation enables NW to schedule more than 20 PUSCH repetitions to meet coverage requirements.

With analysis above, to support the VoNR in NTN, we suggest RAN2 to further study the VoIP packet aggregation in RAN layer(s) and/or application layer.

Proposal 5: RAN2 to further study the packet aggregation mechanism in UE’s application layer and/or RAN layer(s).

# 3	Conclusion

This document has made the following observations:

Observation 1. Due to satellite movement, the probability of PRACH collision for a fixed preamble and RO set configured for requesting Msg3 repetitions is larger when the satellite is at small elevation angles.

Observation 2: Blind retransmission for initial Msg3 transmission is not supported due to the UE-gNB RTT delay before starting the ra-ContentionResolutionTimer.

Observation 3: RAN-assisted voice codec bitrate adaptation mechanism is supported in TN, in which NW can send codec bitrate recommendation to UE via DL MAC CE and UE may query the recommended codec bitrate from NW via UL MAC CE.

Observation 4: Due to high UE-gNB RTT, the legacy MAC-CE based codec bitrate adaptation may not work well.

Observation 5: Due to high UE-gNB RTT, the latency of E2E Codec negotiation procedure will be extended in NTN.

Observation 6: The delay of codec adaptation negotiation may cause the voice packet loss or delay of applying the adjusted codec rate.

Observation 7: Packet aggregation in application layer can reduce the VoNR overhead significantly while it needs to involve in other WG(s).

Observation 8: Packet aggregation in RAN layer limits the solution to RAN scope though the overhead reduction gain is less than the aggregation in application layer.

Observation 9: Packet aggregation enables NW to schedule more than 20 PUSCH repetitions to meet coverage requirements.

And proposed the following:

Proposal 1: RAN2 to discuss methods for configuration of PRACH resources for requesting of Msg3 repetitions for NR NTN.

Proposal 2: RAN2 to address the issues on how to support blind Msg3 retransmission for initial Msg3 transmission for NR NTN.

Proposal 3: RAN2 to further discuss whether broadcast-based bitrate adaptation mechanism should be introduced for NR NTN.

Proposal 4: RAN2 to discuss how to reduce the E2E codec negotiation delay for NR NTN.

Proposal 5: RAN2 to further study the packet aggregation mechanism in UE’s application layer and/or RAN layer(s).

# Reference

[1] RP-222654 NR NTN (Non-Terrestrial Networks) enhancements

[2] R2-2208561 On Msg3 blind retransmission and UE behaviour upon validity timer expiry, Nokia, Nokia Shanghai Bell

[3] R2-2208751 Report of [AT119-e][101][NTN] UP corrections, Interdigital

[4] RP-160563 New SI proposal: Study on enhancement of VoLTE

[5] RP-161856 New Work Item on Voice and Video Enhancement for LTE