3GPP TSG-RAN2 Meeting #119bis electronic	R2-2210661

Online, Oct 10th – 19th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0820 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on the ue-PowerClassPerBandPerBC-r17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh |  |  |  |  |  | Date: |  |  | 2022-9-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | - In Rel17, a per band per BC power class was introduced, but its impact on the maximum Tx power determination hasn’t been specified. - The below note has been deleted according to the latest RAN4 feature list (R4-2209173) NOTE: It is not applicable to the case when UL-MIMO and intra-band UL CA are in operation at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that for MR-DC BCs containing only single CC or intra-band CA in NR side, the ue-PowerClassPerBandPerBC-r17 shall be aligned to the powerClassNRPart-r16. - Clarify the maximum Tx power determination as below - If the ue-PowerClassPerBandPerBC-r17 was reported, the minimum value of ue-PowerClassPerBandPerBC-r17 and powerClass(powerClass-v1610) determines maximum TX power available in the corresponding band. - If the ue-PowerClassPerBandPerBC-r17 was not reported, the minimum value of ue-PowerClass(-v1610/1700) and powerClass(powerClass-v1610) determines maximum TX power available in the corresponding band. - Delete the NOTE according to the latest RAN4 feature list (R4-2209173) NOTE: It is not applicable to the case when UL-MIMO and intra-band UL CA are in operation at the same time. Impact analysis Impacted 5G architecture options: NR-DC, NR-CA, MR-DC Impacted functionality: Maximum Tx power determination Inter-operability: If the network implemented this CR but the UE doesn’t, no inter-operability issue; If the UE implemented this CR but the Network doesn’t, the network may configure an unavailable power class to UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network may configure an unavailable power class to UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1/4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of change

#### 4.2.7.1	BandCombinationList parameters

/******************************Omit the Unchanged Part**************************************/

| mrdc-Parameters Contains the band combination parameters for a given (NG)EN-DC/NE-DC band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ne-DC-BC Indicates whether the UE supports NE-DC for the band combination. | BC | No | N/A | N/A |
| powerClass, powerClass-v1610 Indicates power class the UE supports when operating according to this band combination. If the field is absent, the UE supports the default power class. If the per band per band combination power class (PowerClassPerBandPerBC-r17) that the UE supports on the individual bands of this band combination was reported and lower than this power class, the prior one determines maximum TX power available in each band. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClass in BandNR) and the corresponding per band per band combination power class (PowerClassPerBandPerBC-r17) was not reported, the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. This capability is not applicable to IAB-MT. | BC | No | N/A | FR1 only |
| powerClassNRPart-r16 Indicates NR part power class the UE supports when operating according to this band combination. This field only applies for MR-DC BCs containing only single CC or intra-band CA in NR side in this release. | BC | No | N/A | FR1 only |

/******************************Omit the Unchanged Part**************************************/

# Second of change

#### 4.2.7.7	FeatureSetUplink parameters

/******************************Omit the Unchanged Part**************************************/

| tx-Support-UL-GapFR2-r17 Indicates whether the UE supports UL transmission in FR2 bands within an FR2 UL gap when the FR2 UL gap is activated in inter-band UL CA. The UE which indicates support for tx-Support-UL-GapFR2-r17 shall also indicate support for ul-GapFR2-r17 in an FR2 band. | FS | No | No | FR2 only |
| --- | --- | --- | --- | --- |
| ue-PowerClassPerBandPerBC-r17 Indicates the UE power class per band per band combination. NOTE: For MR-DC BCs containing only single CC or intra-band CA in NR side, the ue-PowerClassPerBandPerBC-r17 shall be aligned to the corresponding powerClassNRPart-r16. | FS | No | N/A | FR1 only |
| ul-CancellationCrossCarrier-r16 Indicates whether the UE supports UL cancellation scheme for cross-carrier comprised of the following functional components: - Supports group common DCI (i.e. DCI format 2_4) for cancellation indication on a different DL CC than that scheduling PUSCH or SRS; - UL cancellation for PUSCH. Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions; - UL cancellation for SRS symbols that overlap with the cancelled symbols. | FS | No | N/A | N/A |

/******************************Omit the Unchanged Part**************************************/

# End of change