3GPP TSG-RAN WG2 Meeting #119bis-e                               R2-2210728

Online, October 10-19, 2022

Agenda item:		8.17.2.1

Source:				Samsung

Title:					General considerations on potential scenarios for MUSIM

Document for:		Discussion & Decision

1	Introduction

In Rel-18 WI on MUSIM, the one of objectives in [1] is as follows:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-Tx UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

This contribution provides our views on potential scenarios to be considered/prioritized in this WI.

2	Discussion

Given that the applicable UE architecture is dual-Rx/dual-Tx in this WI, it seems feasible to support dual RRC connection from a UE implementation perspective as follows:

- Dynamic sharing of multiple RF chains: RF chains are shared between the two USIMs in MUSIM device to the extent that all RF chains can be used by one USIM at a time.

- Exclusive use of one set of Tx RF chain and Rx RF chain: Each set of Tx RF chain and Rx RF chain can be solely used by each USIM in MUSIM device at all times.

From a specification perspecitive, however, we do not see any specification efforts to support the latter case i.e. each UE's USIM can operate independently as in legacy. Thus, we suggest to confirm that RAN2 will only focus on the former case.

Proposal 1: RAN2 to specify enhancements on dual-Tx/dual-Rx capable UE supporting dynamic sharing of multiple RF chains between the two USIMs in MUSIM device in Rel-18.

As this is the first meeting dicussing Rel-18 MUSIM WI, it would be good to discuss which scenario(s) to be considered/prioritized in this WI. At the high level, the following cases can be considered:

- Case 1: Both UEs (USIM A and USIM B) are in RRC_IDLE or RRC_INACTIVE in NW A and NW B

- Case 2: Both UEs (USIM A and USIM B) are in RRC_CONNECTED in NW A and NW B

- Case 3: One UE (USIM A) is in RRC_CONNECTED in NW A while another UE (USIM B) is in RRC_IDLE or RRC_INACTIVE in NW B.

For case 1, it is quite logical that each set of Tx RF chain and Rx RF chain can be used by each UE's USIM (e.g. single-Tx/single-Rx UE), respectively. It does not really matter whether one or two UEs in MUSIM device perform idle mode operations (e.g. paging monitoring, serving/neighour cell measurements, on-demand SI request) or perform RRC setup/resume procedure to enter RRC_CONNECTED. Thus, we think that RAN2 does not need to consider this case this WI.

Proposal 2: RAN2 does not work on any enhancements in Rel-18 when both UEs (USIM A and USIM B) in MUSIM device are either in RRC_IDLE or RRC_INACTIVE in NW A and NW B.

For case 2, as mentioned above, it seems possible from some UE implementations that all RF chains are occupied by one UE's USIM in MUSIM device at a time. But considering that both UEs (USIM A and USIM B) are in RRC_CONNECTED, we think that it seems enough to address the following scenarios in case 2:

- Scenario 1: One UE (USIM A) performs transmission and reception in NW A via all operable RF chains in MUSIM device (e.g. NR with CA, NR DC) while another UE (USIM B) just stays in RRC_CONNECTED without performing transmission and reception in NW B for a certain period.

- Scenario 2: Each UE in MUSIM device performs transmission and reception in NW A and NW B, respectively via a set of at least one Tx RF chain and at least one Rx RF chain.

Depending on required operations in UE's USIM B, the following scenarios in case 3 are expected:

- Scenario 3: UE's USIM B needs to perform transmission and reception in NW B to enter RRC_CONNECTED (e.g. RRC connection setup/resume)

- Scenario 4: UE's USIM B needs to perform transmission and/or reception in NW B without entering RRC_CONNECTED (e.g. on-demand SI request, SI acquition, paging monitoring, etc)

Our understanding is that scenario 3 is not curretly supported from a specification perspective. But potential solutions of supporting scenarios 1 and 2 above may be applied directly to scenario 3. Hence, we think that it is preferable to prioritize to study scenarios 1 and 2 first and then discuss whether any additional enhancements are required in scenario 3. With respect to scenario 4, there seems no specification efforts required in Rel-18 given that Rel-17 MUSIM features (e.g. MUSIM gaps or leave indication) are reused as a baseline.

Proposal 3: RAN2 to specify enhancements on supporting the following scenarios in Rel-18

- In case both UEs (USIM A and USIM B) in MUSIM device are in RRC_CONNECTED in NW A and NW B

  - Scenario 1: One UE (USIM A) performs transmission and reception in NW A via all operable RF chains in MUSIM device (e.g. NR with CA, NR DC) while another UE (USIM B) just stays in RRC_CONNECTED without performing transmission and reception in NW B for a certain period

  - Scenario 2: Each UE performs transmission and reception in NW A and NW B, respectively via a set of at least one Tx RF chain and at least one Rx RF chain

- In case one UE (USIM A) in MUSIM device is in RRC_CONNECTED in NW A while another UE (USIM B) in MUSIM device is in RRC_IDLE or RRC_INACTIVE in NW B

  - Scenario 3: UE (USIM B) performs transmission and reception in NW B to enter RRC_CONNECTED (e.g. RRC connection setup/resume)

Proposal 4: RAN2 to prioritize to study potential solutions for supporting scenarios 1 and 2 in Proposal 3 in Rel-18.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposal:

Proposal 1: RAN2 to specify enhancements on dual-Tx/dual-Rx capable UE supporting dynamic sharing of multiple RF chains between the two USIMs in MUSIM device in Rel-18.

Proposal 2: RAN2 does not work on any enhancements in Rel-18 when both UEs (USIM A and USIM B) in MUSIM device are either in RRC_IDLE or RRC_INACTIVE in NW A and NW B.

Proposal 3: RAN2 to specify enhancements on supporting the following scenarios in Rel-18

- In case both UEs (USIM A and USIM B) in MUSIM device are in RRC_CONNECTED in NW A and NW B

  - Scenario 1: One UE (USIM A) performs transmission and reception in NW A via all operable RF chains in MUSIM device (e.g. NR with CA, NR DC) while another UE (USIM B) just stays in RRC_CONNECTED without performing transmission and reception in NW B for a certain period

  - Scenario 2: Each UE performs transmission and reception in NW A and NW B, respectively via a set of at least one Tx RF chain and at least one Rx RF chain

- In case one UE (USIM A) in MUSIM device is in RRC_CONNECTED in NW A while another UE (USIM B) in MUSIM device is in RRC_IDLE or RRC_INACTIVE in NW B

  - Scenario 3: UE (USIM B) performs transmission and reception in NW B to enter RRC_CONNECTED (e.g. RRC connection setup/resume)

Proposal 4: RAN2 to prioritize to study potential solutions for supporting scenarios 1 and 2 in Proposal 3 in Rel-18.

4	Reference

[1] RP-220955, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR