GPP TSG-RAN WG2 Meeting #119bis-e	R2-2210771

# Online, 10th OCT - 19th OCT 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1444 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.321 for SP/AP SRS TCI State Indication MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-10-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 38.321, for suppoting the unified TCI state for SRS resources, the SP/AP SRS TCI state Indication MAC CE is introduced. In the field description of TCI State BWP IDi as shown below: TCI State BWP IDi: This field indicates a UL BWP as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9], on which the TCI State used for SRS resource i is located. The length of the field is 2 bits; Only UL BWP is indicated by such field, however, the unified TCI state type for one serving cell can be indicated as either joint or separate by the IE unifiedTCI-StateType-r17, if this element indicating as joint, the TCI state is configured within the DL BWP, otherwise, the TCI state is configured within the UL BWP then. Therefore, the TCI State BWP IDi can indicate either DL BWP or UL BWP which is dependent on the value of the unifiedTCI-StateType-r17 configured in the serving cell indicated by TCI State Serving Cell IDi |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Clarify the BWP type in the field description of the TCI State BWP IDi: If value of unifiedTCI-StateType in the serving cell indicated by TCI State Serving Cell IDi is joint, this field indicates a DL BWP. Otherwise, this field indicates a UL BWP. Impact analysis Impacted 5G architecture options: SA, NR-DC, NE-DC, (NG)EN-DC Impacted functionality: SRS Inter-operability: If NW implements according to this CR but UE not, UE cannot map the SRS with the correct TCI state by the indication of the MAC CE. If UE implements according to this CR but NW not, NW have no idea how to configure the TCI state which is located in the DL BWP for one SRS resource by AP/SP SRS TCI State Indication MAC CE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Joint/DL TCI state configured on the DL BWP cannot be configured to a SRS resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.59 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | . |  |  |  |  |  |  |  |  |

Start of the change

6.1.3.59	SP/AP SRS TCI State Indication MAC CE

The SP/AP SRS TCI State Indication MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields:

-	A/D: This field indicates whether to activate or deactivate indicated SP SRS resource set. The field is set to 1 to indicate activation, otherwise it indicates deactivation. If the indicated SRS resource set ID is for the AP SRS resource set, MAC entity shall ignore this field;

-	SRS Resource Set's Cell ID: This field indicates the identity of the Serving Cell, which contains the indicated SP/AP SRS Resource Set. If the C field is set to 0, this field also indicates the identity of the Serving Cell associated with all TCI states indicated by the TCI State IDi fields. The length of the field is 5 bits;

-	SRS Resource Set's BWP ID: This field indicates a UL BWP as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9], which contains the indicated SP/AP SRS Resource Set. If the C field is set to 0, this field also indicates the identity of the BWP associated with all TCI states indicated by the TCI State IDi fields. The length of the field is 2 bits;

-	C: This field indicates whether the octets containing TCI State Serving Cell ID field(s) and TCI State BWP ID field(s) are present. If this field is set to 1, TCI State Serving Cell ID field(s) and TCI State BWP ID field(s) are present, otherwise they are not present so MAC entity shall ignore TCI State Serving Cell ID field(s) and TCI State BWP ID field(s);

-	SUL: This field indicates whether the MAC CE applies to the NUL carrier or SUL carrier configuration. This field is set to 1 to indicate that it applies to the SUL carrier configuration, and it is set to 0 to indicate that it applies to the NUL carrier configuration;

-	SRS Resource Set ID: This field indicates the SP/AP SRS Resource Set ID identified by SRS-ResourceSetId as specified in TS 38.331 [5]. The length of the field is 4 bits;

-	TCI State Serving Cell IDi: This field indicates the identity of the Serving Cell on which the TCI State used for SRS resource i is located. The length of the field is 5 bits;

-	TCI State BWP IDi: This field indicates a BWP as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9], on which the TCI State used for SRS resource i is located. If value of unifiedTCI-StateType in the serving cell indicated by TCI State Serving Cell IDi is joint, this field indicates a DL BWP. Otherwise, this field indicates a UL BWP. The length of the field is 2 bits;

-	TCI State IDi: This field contains an identifier of the TCI state used for SRS resource i. TCI State ID0 refers to the first SRS resource within the resource set, TCI State ID1 refers to the second one and so on. If joint/downlink TCI State is used, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If separate downlink and uplink TCI State is used, the most significant bit of TCI state ID is considered as a reserved bit and the remaining 6 bits indicate the UL-TCIState-Id as specified in TS 38.331 [5]. The length of the field is 7 bits. This field is only present if MAC CE is used for activation of SP SRS resource set, i.e. the A/D field is set to 1, or for AP SRS resource set;

-	R: Reserved bit, set to 0.

Figure 6.1.3.59-1: SP/AP SRS TCI State Indication MAC CE

End of the change