3GPP TSG RAN2 Meeting #120	 R2-2211174

# 14th – 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for RA during CG-SDT procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Scenario – Sequence - UE is in RRC_INACTIVE state. - UE initiates CG-SDT procedure. - During the CG-SDT procedure, UE receives network response for initial CG-SDT transmission including CCCH message. - Later while the CG-SDT procedure is ongoing, Cg-SDT-TimeAlignmentTimer expires, UE stops using the CG-SDT resources. TimeAlignmentTimer is also not running (note that TimeAlignmentTimer is not running when CG-SDT procedure is started) - During the CG-SDT procedure, UE initiates RA procedure if there is no valid CG resource (as specified in section 5.8.2) - During the RA procedure, UE transmits C-RNTI MAC CE in MsgA. In this scenario, upon transmitting MsgA, UE expects PDCCH transmission addressed to the C-RNTI including DL assignment MAC PDU contains the Absolute Timing Advance Command MAC CE, so that UE can be UL synchronised for subsequent UL transmissions. However as per current spec while CG-SDT procedure is ongoing, UE can only receive PDCCH transmission addressed to the C-RNTI and this PDCCH contains a UL grant for a new transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified condition in 5.1.4a so that UE can receive Absolute Timing Advance Command MAC CE when CG-SDT is ongoing but the Cg-SDT-TimeAlignmentTimer has expired (i.e. not running). Impacted functionality: SDT Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can not receive the timing info during the RA-procedure while CG-SDT is ongoing and Cg-SDT-TimeAlignmentTimer has expired |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Change>

### 5.1.4a	MSGB reception and contention resolution for 2-step RA type

Once the MSGA preamble is transmitted, regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	start the msgB-ResponseWindow at the PDCCH occasion as specified in TS 38.213 [6], clause 8.2A;

1>	monitor the PDCCH of the SpCell for a Random Access Response identified by MSGB-RNTI while the msgB-ResponseWindow is running;

1>	if C-RNTI MAC CE was included in the MSGA:

2>	monitor the PDCCH of the SpCell for Random Access Response identified by the C-RNTI while the msgB-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers:

2>	if the C-RNTI MAC CE was included in MSGA:

3>	if the Random Access procedure was initiated for SpCell beam failure recovery or for beam failure recovery of both BFD-RS sets of SpCell (as specified in clause 5.17) and the PDCCH transmission is addressed to the C-RNTI:

4>	consider this Random Access Response reception successful;

4>	stop the msgB-ResponseWindow;

4>	consider this Random Access procedure successfully completed.

3>	else if the timeAlignmentTimer associated with the PTAG is running; or

3>	if CG-SDT procedure is ongoing and cg-SDT-TimeAlignmentTimer is running:

4>	if the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission:

5>	consider this Random Access Response reception successful;

5>	stop the msgB-ResponseWindow;

5>	consider this Random Access procedure successfully completed.

3>	else:

4>	if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded:

5>	if the MAC PDU contains the Absolute Timing Advance Command MAC CE:

6>	process the received Timing Advance Command (see clause 5.2);

6>	consider this Random Access Response reception successful;

6>	stop the msgB-ResponseWindow;

6>	consider this Random Access procedure successfully completed and finish the disassembly and demultiplexing of the MAC PDU.

2>	if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the MSGB-RNTI and the received TB is successfully decoded:

3>	if the MSGB contains a MAC subPDU with Backoff Indicator:

4>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

3>	else:

4>	set the PREAMBLE_BACKOFF to 0 ms.

3>	if the MSGB contains a fallbackRAR MAC subPDU; and

3>	if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see clause 5.1.3a):

4>	consider this Random Access Response reception successful;

4>	apply the following actions for the SpCell:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the msgA-PreambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

5>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

6>	consider the Random Access procedure successfully completed;

6>	process the received UL grant value and indicate it to the lower layers.

5>	else:

6>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

6>	if the Msg3 buffer is empty:

7>	obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer;

6>	process the received UL grant value and indicate it to the lower layers and proceed with Msg3 transmission.

NOTE:	If within a 2-step RA type procedure, an uplink grant provided in the fallback RAR has a different size than the MSGA payload, the UE behavior is not defined.

3>	else if the MSGB contains a successRAR MAC subPDU; and

3>	if the CCCH SDU was included in the MSGA and the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU:

4>	stop msgB-ResponseWindow;

4>	if this Random Access procedure was initiated for SI request:

5>	indicate the reception of an acknowledgement for SI request to upper layers.

4>	else:

5>	set the C-RNTI to the value received in the successRAR;

5>	apply the following actions for the SpCell:

6>	process the received Timing Advance Command (see clause 5.2);

6>	indicate the msgA-PreambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP).

4>	deliver the TPC, PUCCH resource Indicator, ChannelAccess-CPext (if indicated), and HARQ feedback Timing Indicator received in successRAR to lower layers.

4>	consider this Random Access Response reception successful;

4>	consider this Random Access procedure successfully completed;

4>	finish the disassembly and demultiplexing of the MAC PDU.

1>	if msgB-ResponseWindow expires, and the Random Access Response Reception has not been considered as successful based on descriptions above:

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	indicate a Random Access problem to upper layers;

3>	if this Random Access procedure was triggered for SI request:

4>	consider this Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1:

4>	set the RA_TYPE to 4-stepRA;

4>	perform initialization of variables specific to Random Access type as specified in clause 5.1.1a;

4>	if the Msg3 buffer is empty:

5>	obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer;

4>	flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer;

4>	discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any;

4>	perform the Random Access Resource selection procedure as specified in clause 5.1.2.

3>	else:

4>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

4>	if the criteria (as defined in clause 5.1.2a) to select contention-free Random Access Resources is met during the backoff time:

5>	perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a).

4>	else:

5>	perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a) after the backoff time.

Upon receiving a fallbackRAR, the MAC entity may stop msgB-ResponseWindow once the Random Access Response reception is considered as successful.

#### <End of Change>