3GPP TSG-RAN WG2 Meeting #120	R2-2211182

Toulouse France, November 14~18, 2022

Agenda item:	8.5.4.1

Source:	Qualcomm Incorporated

Title:	UE feedback enhancements for capacity improvement

WID/SID:	FS_NR_XR_enh

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss enhancements for UE to report its delay statistics, as means to improve capacity.

# Discussion

## Delay status reporting

At RAN2#119bis-e, RAN2 made the following agreement:

Delay information consists of at least “remaining time”. RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

One of the FFS issue is whether PDB/PSDB is sufficient for network to schedule XR traffic efficiently.

We do not think PDD/PSDB is sufficient to help network better schedule delay-sensitive traffic. This is because delay budget starts when a PDU/PDUS arrives in UL buffer, not when network learns the existence of a PDU from BSR. Currently network is not able to know the remaining delay budget of buffered UL data, as BSR does not indicate how much data is buffered for how long. As a result, network is not able to determine when delay budget of a buffered PDU will deplete. Therefore, PDB or PSDB alone cannot provide intended scheduling enhancements.

Observation 1. 	It is the remaining delay budget instead of PSDB that helps network determine how soon it needs to provide UL grants.

Observation 2. 	Currently network is not able to know the remaining delay budget of buffered UL data, as BSR does not indicate how much data is buffered for how long.

For the above reasons, we think it thus is useful for UE to dynamically report remaining delay budget of data its in layer-2 buffer. This remaining delay budget for a layer-2 PDU can be defined as the difference between the PDB or PSDB of its QoS flow and the time elapsed since it was received at SDAP.

Proposal 1.  	Support dynamic reporting of remaining delay budget of data in UE’s layer-2 buffer.

Proposal 2.  	Remaining delay budget of a layer-2 PDU is defined as the difference between the PDB or PSDB of its associated QoS flow and the time elapsed since it was received at SDAP.

We think there are some differences between this delay information reporting and buffer status reporting. For example,

- They are triggered by difference events. Regular BSRs are triggered by data arrivals. However, delay information reports are triggered by data which has been in layer-2 buffer. To reduce overhead of reporting, delay information reports are triggered only when the smallest remaining delay budget in some selected logical channel(s) drops below a threshold.

- To reduce UL overhead, UE may not need to report remaining delay budget of all buffered data. For example, network may only need to select logical channels which have tight delay budgets for UE to report. In addition, this new delay reporting may not be needed all the time. For example, when UL is not congested and most data can be scheduled in time according to their PDB/PSDB, then delay reporting may not be needed.

Observation 3.	Delay information reported by UE does not need to include all buffered data and may not be needed all the time.

Observation 4. 	Triggers for delay information reporting may not be the same as those for BSR.

Based on the above observations, we can conclude that delay information does not need to be reported in enhanced BSR. Instead, it is arguably more efficient to send delay information independently.

Proposal 3.	Delay information reporting does not need to be coupled with BSRs.

## Delay statistics reporting

In some cases (e.g. if a flow has varying frame rate), it is not easy to signal nominal arrival times and delivery deadlines for each PDU or PDU set. Network then has to use a fixed delay budget in its scheduling. As we have explained in Section 2.1.3, that can result in conservative deadlines. Therefore, having UE provide feedback on delay statistics (e.g. average, standard deviations, etc) can help network adapt the delay budgets it configures/applies to compensate the inefficiency. More specifically,

- On downlink, UE can measure the amount of delay budget left before the delivery deadline (aka residual delay budget) of a PDU or PDU set and then provide those statistics to RAN. RAN then can use that information to adjust the delay budget it applies on DL traffic. For example, if residual delay budget is large, RAN can increase the delay budget it applies.

- On uplink, UE can measure the amount of delay experienced by a PDU or PDU set when it is successfully received by RAN (e.g. upon reception of a positive RLC status report). Network then can use the statistics of this delay to estimate the residual delay budget for the rest of the connection, i.e. end-to-end delay budget provisioned for the flow subtracted by the delay reported by UE.

Proposal 4.	Network can configure UE to measure and report DL and/or UL delay statistics for selected DRBs.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1. 	It is the remaining delay budget instead of PSDB that helps network determine how soon it needs to provide UL grants.

Observation 2. 	Currently network is not able to know the remaining delay budget of buffered UL data, as BSR does not indicate how much data is buffered for how long.

Proposal 1.  	Support dynamic reporting of remaining delay budget of data in UE’s layer-2 buffer.

Proposal 2.  	Remaining delay budget of a layer-2 PDU is defined as the difference between the PDB or PSDB of its associated QoS flow and the time elapsed since it was received at SDAP.

Observation 3.	Delay information reported by UE does not need to include all buffered data and may not be needed all the time.

Observation 4. 	Triggers for delay information reporting may not be the same as those for BSR.

Proposal 3.	Delay information reporting does not need to be coupled with BSRs.

Proposal 4.	Network can configure UE to measure and report DL and/or UL delay statistics for selected DRBs.

# References