3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211203

Toulouse, France, Nov 14th – Nov 18th, 2022

Source:	CATT

Title:	Discussion on RRC Aspects for LTM

Agenda Item:	8.4.2.2

Document for:	Discussion and Decision

Introduction

In RAN2#119bise [1], the following agreements are made on RRC aspects for LTM:

A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. a. Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration b. Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration RAN2 to use “LTM” as term for the L1/L2-triggered mobility. Use the term “cell switch” for the procedure of triggering change of cells via the LTM feature Use the term “Subsequent” LTM for the case when cell switch between L1/L2 mobility candidates is done without RRC reconfiguration in between. RAN2 assumes that sequential L1L2 cell change between Candidates without RRC reconfiguration can be supported. RAN2 assumes that candidate cell configuration can only be modified / released by Network (FFS later whether some optimization should be applied e.g. for release). For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration. For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

In this contribution, we give our considerations on the modeling for LTM candidate cell configurations, and discuss some other RRC configuration aspects.

Discussion

RRC modeling for LTM candidate cell configuration

In last meeting, RRC configuration model for LTM candidate cell configuration was discussed and achieved the following agreement.

RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details. - a. Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration - b. Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

From our perspective, if Model 1 is supported, CU will always be involved to composite the message, but it is indeed unnecessary as the configuration is on DU level only. In addition, for Model 1, UE needs to send RRCReconfigurationComplete message to network for confirmation upon LTM execution, which requires longer latency. Besides, modeling in RRC message level causes more signaling and specification overhead, e.g., need to check case by case which field shall be present, and even the optional field is absent, the signaling overhead is still consumed for these optional fields. Furthermore, sequential L1L2 cell change between candidates without RRC reconfiguration may be supported. And since the same RRC reconfiguration message may be applied more than one times, that is corresponding to the same RRC reconfiguration message, multiple RRC reconfiguration complete message may be responded, which go against the current RRC reconfiguration complete mechanism.

For Model 2, it is simpler compared with Model 1, and can be applicable for both intra-DU case and intra-CU inter-DU case. One concern about Model 2 is that it cannot be used to pre-configure RRM measurement configuration. However, we think that this is not a critical issue and can be addressed by some ways, for example, for RRM measurement configuration, in case of intra-frequency, it has high probability that RRM measurement configuration does not need to be updated as the source cell and target cell has same timing. In this case, possibly, only one bit is needed to indicate UE to keep using the source cell RRM measurement configuration in target cell after LTM cell switch. Furthermore, if the RRM measurement configuration needs to be updated after cell switch, the network can only pre-configure the changed parameters per LTM candidate cell, the remaining parameters remain unchanged by default.

Considering above, we prefer Model 2 compared with Model 1, and propose,

Proposal 1: RAN2 to take Model 2 (i.e., One CellGroupConfig IE for each candidate target configuration) as RRC configuration model for LTM candidate cell configuration.

Proposal 2: If model 2 is agreed, RAN2 to further discuss how to configure RRM measurement configuration for LTM candidate cell configuration.

Configuration of L1 measurement/TCI state pool for LTM candidate cells configuration

In R18 L1L2 mobility, so as to assist network to trigger the L1L2 mobility, UE need to perform the L1 measurement and report the corresponding measurement results to network. Further, it is probably that the beam indication need to be included within the L1L2 mobility command, thus UE should know the corresponding configurations of TCI states for each candidate cell in advance. Otherwise, UE shall perform the beam switching (RF tuning), only after UE have already access the target cell and apply the target cell’s configuration, which may introduce additional latency.

Observation 1: UE need to apply the configurations of L1 measurement and TCI states of candidate cells before reception of the LTM command.

In R17 ICBM, except the L1 measurement and TCI states configurations, all other configurations about the non-serving cells with additionalPCI are same. So RAN2 finally decided to model the non-serving cells with additional PCI per TCI level within the serving cell configuration, i.e., adding the additionalPCI for TCI state and SSB configuration so as to distinguish the L1 measurement and TCI for different non-serving cells with additionalPCI.

Observation 2: In R17 ICBM, the L1 measurement and TCI states for non-serving cells with additionalPCI are configured within the serving cell configuration identified with additionalPCI.

On the configuration of these L1 measurement configuration and TCI states, the following signaling structure can be considered:

- Option 1: The L1 measurement configurations and TCI states for candidate cells are configured outside the modeling of the candidate cells,

- Option 1-a: R17 ICBM similar like mechanism are reused for the configurations of L1 measurement configurations and TCI states for candidate cells;

- Option 1-b: A common pool is configured for the L1 measurement configurations and TCI states for candidate cells;

- Option 2: The L1 measurement configurations and TCI states for candidate cells are only configured within the modeling of the candidate cells, i.e., within the CellGroupConfig (based on mode 2) for each candidate cell;

For option 2, although only one target cell will be accessed by UE finally, UE still has to decode the configuration of all candidate cells even the candidate cells are not be the target cell, so as to obtain the L1 measurement and TCI state configuration, while in legacy, only the configuration of target cell shall be applied by UE. That is, if option 2 is adopted, more UE complexity and spec impact will be introduced.

Proposal 3: RAN2 to agree that the L1 measurement configurations and TCI states for candidate cells are configured outside the modeling of the candidate cells.

Proposal 4: RAN2 to further discuss the details of signaling structure of L1 measurement configurations and TCI states for candidate cells, based on the following candidate options:

- Option 1-a: R17 ICBM-like signaling structure is reused for the configurations of L1 measurement configurations and TCI states for candidate cells;

- Option 1-b: A common pool is used to configure the L1 measurement configurations and TCI states for candidate cells.

Subsequent LTM without RRC Reconfiguration

    - Delta configuration for subsequent LTM

Last meeting we have agreed that the sequential LTM shall be supported among candidate cells without RRC reconfiguration, and also delta configuration is used to configure the candidate cells by the NW:

For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration.

Regarding how to achieve the delta configuration, there is an FFS on whether to use a separate reference configuration or the current configuration. If the candidate cell configuration is on the top of current serving cell configuration, then the update of the candidate cell configuration shall be required when UE access to the target cell, which may cause large signaling load and preparation latency.

Observation 3: The sequential LTM without NW reconfiguration cannot be achieved, if the reference configuration is based on the current serving cell configuration.

For the way of a separate reference configuration, this issue may be avoided since the reference configuration is constant and could be not changed with cell switch. For the details of the separate reference configuration, one way is to specify one candidate cell configuration as the reference configuration, and all other candidate configuration is configured based on this specified candidate cell configuration. Another way is to define a new reference configuration. The reference configuration is only used as reference for candidate cell configuration, which is separated from the candidate cell configuration.

Proposal 5: Use a separate reference configuration to perform delta configuration for the candidate cells.

Proposal 6: As for the separate reference configuration, RAN2 to down-select from the following options:

- Specify one of the candidate cell as the reference configuration

- Define one reference configuration independent from the candidate cell configuration

    - On handling of current serving cell

A possible scenario in the subsequent LTM is that UE moves from cell 0 (current serving cell) to candidate cell 1, and further wants to go back to cell 0. In this situation, if the cell 0 (current serving cell) is not configured by NW as the candidate cell in the pre-configured candidate cell configurations, UE cannot switch back to the cell 0.  To support this scenario in LTM, the current serving cell should also be configured as a candidate cell.

Proposal 7:  NW configures the current serving cell as one of the candidate configurations if the NW wants to use it for sequential LTM.

    - On configuration of subsequent LTM

As RAN2 agreed, the subsequent LTM is an enhancement, but not mandatory to be supported for all UEs supporting R18 LTM. That means, whether to support subsequent LTM is separate UE capability. Moreover, different like legacy handover, the configuration of the candidate cells for subsequent LTM should be maintained by UE even after the cell switch is executed.  Thus to distinguish the normal LTM (one-shot) from the subsequent LTM, NW should control the configuration of subsequent LTM, i.e., by providing indication on whether the subsequent LTM is enabled, so that UE can decide whether need to maintain the configurations after cell switch.

Proposal 8: The configuration of subsequent LTM is controlled by NW, i.e., NW provides explicit indication on whether the configuration of candidate cell is used for subsequent LTM.

    - On L1 reporting for subsequent LTM

For subsequent LTM, the L1 measurement should be reported to the target cell after cell switch, so that NW can make the switch decision. However, different like L3 measurement, the resource for legacy L1 measurement reporting is fixed and pre-configured, i.e., the measurement RS, measurement quantity, reporting type as well as the reporting resources are one-to-one mapped. That is, after UE access the target cell, the L1 measurement and reporting will be invalid, since the reporting resource is for the serving cell. However, since the L1 measurement and reporting is within RAN1 scope, so RAN2 can just wait for RAN1 progress.

Proposal 9: As for L1 measurement and reporting for subsequent LTM, RAN2 wait for RAN1 progress.

On BWP handling

In last meeting, the BWP handling was discussed and the following agreements were made.

- FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

In legacy handover, the BWP to be active at target cell is configured via RRC, i.e., firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id. Similarly, for the normal (one-shot) LTM, the legacy BWP handling mechanism can be reused. However, since RAN2 agreed to support the subsequent LTM in last meeting, that means the pre-configured RRC reconfiguration shall be used and maintained by UE and NW for a long time. For this case, it is probably the pre-configured BWP to be used upon execution of L1/L2 inter-cell mobility is not the suitable BWP, but it could be update by DCI after UE accessing the target cll. And the active BWP should be determined by target BWP, if it is included in LTM command, the interaction between source cell and target cell is needed, which may introduce latency. Considering that we can support the legacy mechanism upon LTM execution, the active BWP used in target cell should be the firstActiveDownlinkBWP-Id/ firstActiveUplinkBWP-Id configured by RRC.

Proposal 10: The legacy BWP mechanism, i.e., the firstActiveDownlinkBWP-Id/firstActiveUplinkBWP-Id, can be reused for UE to determine the BWP (for DL/UL) to be used upon the execution of LTM.

Conclusion

Based on the previous analysis in section 2, our main contributions are summarized as follows:

On RRC modeling for LTM candidate cell configuration:

Proposal 1: RAN2 to take Model 2 (i.e., One CellGroupConfig IE for each candidate target configuration) as RRC configuration model for LTM candidate cell configuration.

Proposal 2: If model 2 is agreed, RAN2 to further discuss how to configure RRM measurement configuration for LTM candidate cell configuration.

On Configuration of L1 measurement/TCI state pool for LTM candidate cells configuration:

Observation 1: UE need to apply the configurations of L1 measurement and TCI states of candidate cells before reception of the LTM command.

Observation 2: In R17 ICBM, the L1 measurement and TCI states for non-serving cells with additionalPCI are configured within the serving cell configuration identified with additionalPCI.

Proposal 3: RAN2 to agree that the L1 measurement configurations and TCI states for candidate cells are configured outside the modeling of the candidate cells.

Proposal 4: RAN2 to further discuss the details of signaling structure of L1 measurement configurations and TCI states for candidate cells, based on the following candidate options:

- Option 1-a: R17 ICBM-like signaling structure is reused for the configurations of L1 measurement configurations and TCI states for candidate cells;

- Option 1-b: A common pool is used to configure the L1 measurement configurations and TCI states for candidate cells.

On Subsequent LTM without RRC Reconfiguration:

Observation 3: The sequential LTM without NW reconfiguration cannot be achieved, if the reference configuration is based on the current serving cell configuration.

Proposal 5: Use a separate reference configuration to perform delta configuration for the candidate cells.

Proposal 6: As for the separate reference configuration, RAN2 to down-select from the following options:

- Specify one of the candidate cell as the reference configuration

- Define one reference configuration independent from the candidate cell configuration

Proposal 7:  NW configures the current serving cell as one of the candidate configurations if the NW wants to use it for sequential LTM.

Proposal 8: The configuration of subsequent LTM is controlled by NW, i.e., NW provides explicit indication on whether the configuration of candidate cell is used for subsequent LTM.

Proposal 9: On L1 measurement and reporting for subsequent LTM, RAN2 wait for RAN1 progress.

On BWP handling:

Proposal 10: The legacy BWP mechanism, i.e., the firstActiveDownlinkBWP-Id/firstActiveUplinkBWP-Id, can be reused for UE to determine the BWP (for DL/UL) to be used upon the execution of LTM.

Reference

[1] R2 119bis-e Chair Notes EOM rev1