3GPP TSG-RAN2 Meeting # 120	R2-2211214

# Toulouse, France,  November-14 - November-18

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3571 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR SL discovery capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core, NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2022-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Based on R1 LS reply in R1-2210492, “In case a band combination supporting both NR Sidelink communication and NR Sidelink discovery, the UE capability of these per-FS L1 features would be the same for NR Sidelink communication and NR Sidelink discovery.”. It is natural to avoid duplicating the per-FS capability for BC supporting both NR sidelink communication and NR sidelink discovery, and rely on the legacy NR sidelink communication BC list BandCombinationListSidelinkEUTRA-NR to report the per-FS capability for such BC. However, there is no way for gNB to differentiate between the BC supporting NR sidelink Communication only and the BC supporting NR sidelink discovery as well. - Based on the R1 LS reply in R1-2210492, R1 confirmed that ‘these per-FS L1 features above also apply to the band-combination supporting discovery’. Yet UE cannot report those parameters for the BC supporting NR sidelink discovery only (i.e., not supporting NR sidelink communication so not included in BandCombinationListSidelinkEUTRA-NR). - During R2#119, there was a proposal (R2-2208195) on extending the network filter of sidelink per-FS capability reporting to discovery case as well, yet was not concluded due to the pending R1 LS reply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In BandCombinationListSidelinkEUTRA-NR, add IEs of supportnonRelayDiscovery, and supportRelayDiscovery, to indicate the BC that supports both NR sidelink Communication/Discovery. - For supportedBandCombinationListSL-RelayDiscovery and supportedBandCombinationListSL-nonRelayDiscovery, introduce reporting of 15-25, 32-4 and 32-5a-1. - Apply the network filter to NR sidelink capability as well. Impact analysis Impacted 5G architecture options: SA Impacted functionality NR Sidelink discovery Inter-operability: If network implements the change but not the UE, 1) for change-1 and 2, the problem remains, i.e., the UE cannot report for NR sidelink discovery related per-FS capability, 2) for change-3, there is no inter-operability issue since there is no NR sidelink discovery related per-FS capability to be reported by UE. If the UE implements the change but not the network. 1) for change-1 and change-2, there is no inter-operability issue, i.e., the network would not make use of the per-FS NR sidelink discovery capability, 2) for change-3, here is no inter-operability issue since the network would not make use of the per-FS NR sidelink discovery capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - There is no way for gNB to differentiate between the BC supporting NR sidelink Communication only and the BC supporting NR sidelink discovery as well. - UE cannot report those parameters for the BC supporting NR sidelink discovery only (i.e., not supporting NR sidelink communication so not included in BandCombinationListSidelinkEUTRA-NR). - Network filter of per-FS sidelink capablity only applicable to NR sidelink Communication but not to NR sidelink discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.4, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.306 CR 0824 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

#### 5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, and requestedCapabilityCommon) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

NOTE 2:	In EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations.

NOTE 3:	If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

The UE shall:

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell or SUL according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

NOTE 5:	Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many NR-only band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

NOTE 6:	This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container.

2>	if sidelinkRequest is received:

3>	for a sidelink band combination the UE included in supportedBandCombinationListSidelinkEUTRA-NR, supportedBandCombinationListSL-RelayDiscovery or supportedBandCombinationListSL-NonRelayDiscovery:

4>	if the UE supports partial sensing for a band of the sidelink band combination, include the partial sensing capabilities for the band using the sl-TransmissionMode2-PartialSensing-r17;

3>	set sidelinkRequested to true;

2>	include into featureSets the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received;

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many E-UTRA-NR band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

1>	else (if the requested rat-Type is eutra):

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

NOTE 7:	This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSetsEUTRA (in the UE-EUTRA-Capability) the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	if the network included ue-CapabilityEnquiryExt:

2>	include the received ue-CapabilityEnquiryExt in the field receivedFilters;

Next Change

#### –	BandCombinationListSidelinkEUTRA-NR

The IE BandCombinationListSidelinkEUTRA-NR contains a list of V2X sidelink and NR sidelink band combinations.

BandCombinationListSidelinkEUTRA-NR information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-START

BandCombinationListSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-r16

BandCombinationListSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1630

BandCombinationListSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1710

BandCombinationListSidelinkEUTRA-NR-v17xy ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v17xy

BandCombinationParametersSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-r16

BandCombinationParametersSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1630

BandCombinationParametersSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1710

BandCombinationParametersSidelinkEUTRA-NR-v17xy ::= SEQUENCE {

supportNonRelayDiscovery-r17           ENUMERATED {supported}                          OPTIONAL,

supportRelayDiscovery-r17              ENUMERATED {supported}                          OPTIONAL

}

BandParametersSidelinkEUTRA-NR-r16 ::= CHOICE {

eutra                                  SEQUENCE {

bandParametersSidelinkEUTRA1-r16       OCTET STRING                         OPTIONAL,

bandParametersSidelinkEUTRA2-r16       OCTET STRING                         OPTIONAL

},

nr                                     SEQUENCE {

bandParametersSidelinkNR-r16           BandParametersSidelink-r16

}

}

BandParametersSidelinkEUTRA-NR-v1630 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

tx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

rx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

sl-CrossCarrierScheduling-r16            ENUMERATED {supported}                          OPTIONAL

}

}

BandParametersSidelinkEUTRA-NR-v1710 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

--32-4

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--32-2a:  Receiving NR sidelink of PSFCH

rx-sidelinkPSFCH-r17                     ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL,

--32-5b-1

tx-IUC-Scheme2-Mode2Sidelink-r17         ENUMERATED {n4, n8, n16}                          OPTIONAL

}

}

BandParametersSidelink-r16 ::= SEQUENCE {

freqBandSidelink-r16           FreqBandIndicatorNR

}

BandCombinationListSL-Discovery-v17xy ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkDiscovery-v17xy

BandParametersSidelinkDiscovery-v17xy ::= SEQUENCE {

sl-CrossCarrierScheduling-r16            ENUMERATED {supported}                            OPTIONAL,

--32-4

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL

}

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-STOP

-- ASN1STOP

BandParametersSidelinkEUTRA-NR field descriptions

bandParametersSidelinkEUTRA1, bandParametersSidelinkEUTRA2 This field includes the V2X-BandParameters-r14 and V2X-BandParameters-v1530 IE as specified in 36.331 [10]. It is used for reporting the per-band capability for V2X sidelink communication.

Next Change

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                                   SEQUENCE {

supportedBandListNR                                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList                        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter                           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540                  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested                          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550                  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560                  BandCombinationList-v1560                   OPTIONAL

]],

[[

supportedBandCombinationList-v1610                  BandCombinationList-v1610                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-r16    BandCombinationListSidelinkEUTRA-NR-r16     OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16     BandCombinationList-UplinkTxSwitch-r16      OPTIONAL

]],

[[

supportedBandCombinationList-v1630                  BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-v1630  BandCombinationListSidelinkEUTRA-NR-v1630   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1630   BandCombinationList-UplinkTxSwitch-v1630    OPTIONAL

]],

[[

supportedBandCombinationList-v1640                  BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1640   BandCombinationList-UplinkTxSwitch-v1640    OPTIONAL

]],

[[

supportedBandCombinationList-v1650                  BandCombinationList-v1650                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1650   BandCombinationList-UplinkTxSwitch-v1650    OPTIONAL

]],

[[

extendedBand-n77-r16                                ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-UplinkTxSwitch-v1670   BandCombinationList-UplinkTxSwitch-v1670    OPTIONAL

]],

[[

supportedBandCombinationList-v1680                  BandCombinationList-v1680                   OPTIONAL

]],

[[

supportedBandCombinationList-v1690                  BandCombinationList-v1690                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1690   BandCombinationList-UplinkTxSwitch-v1690    OPTIONAL

]],

[[

supportedBandCombinationList-v1700                  BandCombinationList-v1700                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1700   BandCombinationList-UplinkTxSwitch-v1700    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-r17   OCTET STRING                                OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSL-NonRelayDiscovery-r17 OCTET STRING                               OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSidelinkEUTRA-NR-v1710  BandCombinationListSidelinkEUTRA-NR-v1710   OPTIONAL,

sidelinkRequested-r17                               ENUMERATED {true}                           OPTIONAL,

extendedBand-n77-2-r17                              ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-v1720                  BandCombinationList-v1720                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1720   BandCombinationList-UplinkTxSwitch-v1720    OPTIONAL

]],

[[

supportedBandCombinationListSL-RelayDiscovery-v17xy BandCombinationListSL-Discovery-v17xy       OPTIONAL,

supportedBandCombinationListSL-NonRelayDiscovery-v17xy BandCombinationListSL-Discovery-v17xy    OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-v17xy  BandCombinationListSidelinkEUTRA-NR-v17xy   OPTIONAL

]]

}

RF-Parameters-v15g0 ::=                   SEQUENCE {

supportedBandCombinationList-v15g0        BandCombinationList-v15g0                   OPTIONAL

}

RF-Parameters-v16a0 ::=                            SEQUENCE {

supportedBandCombinationList-v16a0                 BandCombinationList-v16a0                    OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v16a0  BandCombinationList-UplinkTxSwitch-v16a0     OPTIONAL

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]],

[[

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

]],

[[

asymmetricBandwidthCombinationSet     BIT STRING (SIZE (1..32))           OPTIONAL

]],

[[

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-r16 SharedSpectrumChAccessParamsPerBand-r16 OPTIONAL,

-- R1 11-7b: Independent cancellation of the overlapping PUSCHs in an intra-band UL CA

cancelOverlappingPUSCH-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-1: Multiple LTE-CRS rate matching patterns

multipleRateMatchingEUTRA-CRS-r16       SEQUENCE {

maxNumberPatterns-r16               INTEGER (2..6),

maxNumberNon-OverlapPatterns-r16    INTEGER (1..3)

}                                                                               OPTIONAL,

-- R1 14-1a: Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier

overlapRateMatchingEUTRA-CRS-r16        ENUMERATED {supported}                  OPTIONAL,

-- R1 14-2: PDSCH Type B mapping of length 9 and 10 OFDM symbols

pdsch-MappingTypeB-Alt-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-3: One slot periodic TRS configuration for FR1

oneSlotPeriodicTRS-r16                  ENUMERATED {supported}                  OPTIONAL,

olpc-SRS-Pos-r16                        OLPC-SRS-Pos-r16                        OPTIONAL,

spatialRelationsSRS-Pos-r16             SpatialRelationsSRS-Pos-r16             OPTIONAL,

simulSRS-MIMO-TransWithinBand-r16       ENUMERATED {n2}                         OPTIONAL,

channelBW-DL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                          SEQUENCE {

scs-60kHz                           ENUMERATED {supported}              OPTIONAL,

scs-120kHz                          ENUMERATED {supported}              OPTIONAL

}

}                                                                               OPTIONAL,

channelBW-UL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                              SEQUENCE {

scs-60kHz                               ENUMERATED {supported}          OPTIONAL,

scs-120kHz                              ENUMERATED {supported}          OPTIONAL

}

}                                                                               OPTIONAL,

rasterShift7dot5-IAB-r16                ENUMERATED {supported}                  OPTIONAL,

ue-PowerClass-v1610                     ENUMERATED {pc1dot5}                    OPTIONAL,

condHandover-r16                        ENUMERATED {supported}                  OPTIONAL,

condHandoverFailure-r16                 ENUMERATED {supported}                  OPTIONAL,

condHandoverTwoTriggerEvents-r16        ENUMERATED {supported}                  OPTIONAL,

condPSCellChange-r16                    ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeTwoTriggerEvents-r16    ENUMERATED {supported}                  OPTIONAL,

mpr-PowerBoost-FR2-r16                  ENUMERATED {supported}                  OPTIONAL,

-- R1 11-9: Multiple active configured grant configurations for a BWP of a serving cell

activeConfiguredGrant-r16               SEQUENCE {

maxNumberConfigsPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n12},

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 11-9a: Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell

jointReleaseConfiguredGrantType2-r16    ENUMERATED {supported}                  OPTIONAL,

-- R1 12-2: Multiple SPS configurations

sps-r16                                 SEQUENCE {

maxNumberConfigsPerBWP-r16                  INTEGER (1..8),

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 12-2a: Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell

jointReleaseSPS-r16                     ENUMERATED {supported}                  OPTIONAL,

-- R1 13-19: Simultaneous positioning SRS and MIMO SRS transmission within a band across multiple CCs

simulSRS-TransWithinBand-r16            ENUMERATED {n2}                         OPTIONAL,

trs-AdditionalBandwidth-r16             ENUMERATED {trs-AddBW-Set1, trs-AddBW-Set2}  OPTIONAL,

handoverIntraF-IAB-r16                  ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R1 22-5a: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for intra-band UL CA

-- R1 22-5c: Simultaneous transmission of SRS for antenna switching and SRS for antenna switching for intra-band UL CA

simulTX-SRS-AntSwitchingIntraBandUL-CA-r16  SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-v1630   SharedSpectrumChAccessParamsPerBand-v1630   OPTIONAL

]],

[[

handoverUTRA-FDD-r16                      ENUMERATED {supported}                       OPTIONAL,

-- R4 7-4: Report the shorter transient capability supported by the UE: 2, 4 or 7us

enhancedUL-TransientPeriod-r16            ENUMERATED {us2, us4, us7}                   OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1640 SharedSpectrumChAccessParamsPerBand-v1640    OPTIONAL

]],

[[

type1-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

type2-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

pusch-RepetitionMultiSlots-v1650          ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType1-v1650             ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType2-v1650             ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1650 SharedSpectrumChAccessParamsPerBand-v1650    OPTIONAL

]],

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

-- R1 36-1: Support of 1024QAM for PDSCH for FR1

pdsch-1024QAM-FR1-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R4 22-1 support of FR2 HST operation

ue-PowerClass-v1700                       ENUMERATED {pc5, pc6, pc7}                   OPTIONAL,

-- R1 24: NR extension to 71GHz (FR2-2)

fr2-2-AccessParamsPerBand-r17             FR2-2-AccessParamsPerBand-r17                OPTIONAL,

rlm-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

bfd-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

cg-SDT-r17                                ENUMERATED {supported}                       OPTIONAL,

locationBasedCondHandover-r17             ENUMERATED {supported}                       OPTIONAL,

timeBasedCondHandover-r17                 ENUMERATED {supported}                       OPTIONAL,

eventA4BasedCondHandover-r17              ENUMERATED {supported}                       OPTIONAL,

mn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

sn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3a: PDCCH skipping

pdcch-SkippingWithoutSSSG-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3b: 2 search space sets group switching

sssg-Switching-1BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3c: 3 search space sets group switching

sssg-Switching-2BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3d: 2 search space sets group switching with PDCCH skipping

pdcch-SkippingWithSSSG-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3e: Support Search space set group switching capability 2 for FR1

searchSpaceSetGrp-switchCap2-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 26-1: Uplink Time and Frequency pre-compensation and timing relationship enhancements

uplinkPreCompensation-r17                 ENUMERATED {supported}                       OPTIONAL,

-- R1 26-4: UE reporting of information related to TA pre-compensation

uplink-TA-Reporting-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-5: Increasing the number of HARQ processes

max-HARQ-ProcessNumber-r17                ENUMERATED {u16d32, u32d16, u32d32}          OPTIONAL,

-- R1 26-6: Type-2 HARQ codebook enhancement

type2-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6a: Type-1 HARQ codebook enhancement

type1-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6b: Type-3 HARQ codebook enhancement

type3-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-9: UE-specific K_offset

ue-specific-K-Offset-r17                  ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1f: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1

multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1g: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1

multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 14-4: Parallel PRS measurements in RRC_INACTIVE state, FR1/FR2 diff

parallelPRS-MeasRRC-Inactive-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 27-1-2: Support of UE-TxTEGs for UL TDOA

nr-UE-TxTEG-ID-MaxSupport-r17             ENUMERATED {n1, n2, n3, n4, n6, n8}          OPTIONAL,

-- R1 27-17: PRS processing in RRC_INACTIVE

prs-ProcessingRRC-Inactive-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-3-2: DL PRS measurement outside MG and in a PRS processing window

prs-ProcessingWindowType1A-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType1B-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType2-r17             ENUMERATED {option1, option2, option3}       OPTIONAL,

-- R1 27-15: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP

srs-AllPosResourcesRRC-Inactive-r17       SRS-AllPosResourcesRRC-Inactive-r17          OPTIONAL,

-- R1 27-16: OLPC for positioning SRS in RRC_INACTIVE state - gNB

olpc-SRS-PosRRC-Inactive-r17              OLPC-SRS-Pos-r16                             OPTIONAL,

-- R1 27-19: Spatial relation for positioning SRS in RRC_INACTIVE state - gNB

spatialRelationsSRS-PosRRC-Inactive-r17   SpatialRelationsSRS-Pos-r16                  OPTIONAL,

-- R1 30-1: Increased maximum number of PUSCH Type A repetitions

maxNumberPUSCH-TypeA-Repetition-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-2: PUSCH Type A repetitions based on available slots

puschTypeA-RepetitionsAvailSlot-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3: TB processing over multi-slot PUSCH

tb-ProcessingMultiSlotPUSCH-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3a: Repetition of TB processing over multi-slot PUSCH

tb-ProcessingRepMultiSlotPUSCH-r17        ENUMERATED {supported}                       OPTIONAL,

-- R1 30-4: The maximum duration for DM-RS bundling

maxDurationDMRS-Bundling-r17              SEQUENCE {

fdd-r17                                   ENUMERATED {n4, n8, n16, n32}            OPTIONAL,

tdd-r17                                   ENUMERATED {n2, n4, n8, n16}             OPTIONAL

}                                                                                      OPTIONAL,

-- R1 30-6: Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

pusch-RepetitionCRC-r17                   ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1710 SharedSpectrumChAccessParamsPerBand-v1710    OPTIONAL,

-- R4 25-2: Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions

-- on normal operations with the serving cell

parallelMeasurementWithoutRestriction-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 25-5: Parallel measurements on multiple NGSO satellites within a SMTC

maxNumber-NGSO-SatellitesWithinOneSMTC-r17 ENUMERATED {n1, n2, n3, n4}                 OPTIONAL,

-- R1 26-10: K1 range extension

k1-RangeExtension-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R1 35-1: Aperiodic CSI-RS for tracking for fast SCell activation

aperiodicCSI-RS-FastScellActivation-r17   SEQUENCE {

maxNumberAperiodicCSI-RS-PerCC-r17        ENUMERATED {n8, n16, n32, n48, n64, n128, n255},

maxNumberAperiodicCSI-RS-AcrossCCs-r17    ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024}

}                                                                                      OPTIONAL,

-- R1 35-2: Aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth

aperiodicCSI-RS-AdditionalBandwidth-r17   ENUMERATED {addBW-Set1, addBW-Set2}          OPTIONAL,

-- R1 28-1a: RRC-configured DL BWP without CD-SSB or NCD-SSB

bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 28-3: Half-duplex FDD operation type A for RedCap UE

halfDuplexFDD-TypeA-RedCap-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 OPTIONAL,

-- R4 15-3 UE support of CBW for 480kHz SCS

channelBWs-DL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 15-4 UE support of CBW for 960kHz SCS

channelBWs-DL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 17-1 UL gap for Tx power management

ul-GapFR2-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 25-4: One-shot HARQ ACK feedback triggered by DCI format 1_2

oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 25-5: PHY priority handling for one-shot HARQ ACK feedback

oneShotHARQ-feedbackPhy-Priority-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 25-6: Enhanced type 3 HARQ-ACK codebook feedback

enhancedType3-HARQ-CodebookFeedback-r17   SEQUENCE {

enhancedType3-HARQ-Codebooks-r17          ENUMERATED {n1, n2, n4, n8},

maxNumberPUCCH-Transmissions-r17          ENUMERATED {n1, n2, n3, n4, n5, n6, n7}

}                                                                                      OPTIONAL,

-- R1 25-7: Triggered HARQ-ACK codebook re-transmission

triggeredHARQ-CodebookRetx-r17              SEQUENCE {

minHARQ-Retx-Offset-r17                     ENUMERATED {n-7, n-5, n-3, n-1, n1},

maxHARQ-Retx-Offset-r17                     ENUMERATED {n4, n6, n8, n10, n12, n14, n16, n18, n20, n22, n24}

}                                                                                      OPTIONAL

]],

[[

-- R4 22-2 support of one shot large UL timing adjustment

ue-OneShotUL-TimingAdj-r17                        ENUMERATED {supported}               OPTIONAL,

-- R1 25-2: Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8

pucch-Repetition-F0-2-r17                         ENUMERATED {supported}               OPTIONAL,

-- R1 25-11a: 4-bits subband CQI for NTN and unlicensed

cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17   ENUMERATED {supported}               OPTIONAL,

-- R1 25-16: HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH

mux-HARQ-ACK-DiffPriorities-r17                   ENUMERATED {supported}               OPTIONAL,

-- R1 25-20a: Propagation delay compensation based on legacy TA procedure for NTN and unlicensed

ta-BasedPDC-NTN-SharedSpectrumChAccess-r17        ENUMERATED {supported}               OPTIONAL,

-- R1 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17  ENUMERATED {supported}               OPTIONAL,

-- R1 33-2e: Multiple G-RNTIs for group-common PDSCHs

maxNumberG-RNTI-r17                               INTEGER (2..8)                       OPTIONAL,

-- R1 33-2f: Dynamic multicast with DCI format 4_2

dynamicMulticastDCI-Format4-2-r17                 ENUMERATED {supported}               OPTIONAL,

-- R1 33-2i: Supported maximal modulation order for multicast PDSCH

maxModulationOrderForMulticast-r17                CHOICE {

fr1-r17                                           ENUMERATED {qam256, qam1024},

fr2-r17                                           ENUMERATED {qam64, qam256}

}                                                                                                                          OPTIONAL,

-- R1 33-3-1: Dynamic Slot-level repetition for group-common PDSCH for TN and licensed

dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17  ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-3-1a: Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed

dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-4-1: DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast

nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17                ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

maxNumberG-CS-RNTI-r17                                          INTEGER (2..8)                                             OPTIONAL,

-- R1 33-10: Support group-common PDSCH RE-level rate matching for multicast

re-LevelRateMatchingForMulticast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 36-1a: Support of 1024QAM for PDSCH with maximum 2 MIMO layers for FR1

pdsch-1024QAM-2MIMO-FR1-r17                                     ENUMERATED {supported}                                     OPTIONAL,

-- R4 14-3 PRS measurement without MG

prs-MeasurementWithoutMG-r17                                    ENUMERATED {cpLength, quarterSymbol, halfSymbol, halfSlot} OPTIONAL,

-- R4 25-7: The number of target LEO satellites the UE can monitor per carrier

maxNumber-LEO-SatellitesPerCarrier-r17                          INTEGER (3..4)                                             OPTIONAL,

-- R1 27-3-3 DL PRS Processing Capability outside MG - buffering capability

prs-ProcessingCapabilityOutsideMGinPPW-r17    SEQUENCE (SIZE(1..3)) OF PRS-ProcessingCapabilityOutsideMGinPPWperType-r17   OPTIONAL,

-- R1 27-15a: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS

srs-SemiPersistent-PosResourcesRRC-Inactive-r17                 SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17                       ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}                                                                                                                          OPTIONAL,

-- R2: UE support of CBW for 120kHz SCS

channelBWs-DL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL,

channelBWs-UL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationList A list of band combinations that the UE supports for NR (and NR-DC, if requested). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationListSidelinkEUTRA-NR A list of band combinations that the UE supports for NR sidelink communication only, for joint NR sidelink communication and V2X sidelink communication, or for V2X sidelink communication only. The UE does not include this field if the UE capability is requested by E-UTRAN (see TS 36.331[10]) and the network request includes the field eutra-nr-only.

supportedBandCombinationListSL-NonRelayDiscovery A list of band combinations that the UE supports for NR sidelink non-relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationListSL-RelayDiscovery A list of band combinations that the UE supports for NR sidelink relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationList-UplinkTxSwitch A list of band combinations that the UE supports dynamic uplink Tx switching for NR UL CA and SUL. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

End of Change