3GPP TSG-RAN WG2 #120	R2-2211218

Toulouse, France, November 2022

Agenda Item:	5.2.2

Source:	OPPO

Title:	Discussion on resource pool Index definition

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue for R1-2210585.

Left issue from R1-2210585

Based on R1 agreement

Agreement

For determining the scheduled Tx resource pool corresponding to the 'Resource pool index' in a DCI format 3_0 when both sl-TxPoolScheduling and sl-DiscTxPoolScheduling are configured, it is up to RAN2 to decide whether/how to specify the indexing of the configured Tx resource pools.

RAN1 has also discussed whether a UE capability is needed to support DCI format 3_0 capable of scheduling a dedicated discovery pool, and decides to leave this decision to RAN2.

It seems good to clarify whether there is additional aspect to clarify in R2.

Our understanding here is that in R16, when multiple pools are configured in sl-TxPoolScheduling, the 'Resource pool index' in a DCI format

- Does NOT equal to sl-ResourcePoolID-r16, considering it is in the range of [1..16], which can be larger than the number of pools configured (no larger than 8 = maxNrofTXPool-r16)

- Should be based on the number of pools configured

-	Resource pool index –log2I  bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured.

So the index definition, in our view, is: the value is indexed sequentially from 0 in the same ascending order (based on the value of sl-ResourcePoolID-r16) of pools configured in sl-TxPoolScheduling.

E.g., 5 pools are configured, with

- sl-ResourcePoolID-r16 = 2, for exceptional pool

- sl-ResourcePoolID-r16 = 0/1/3/4, for pools in sl-TxPoolScheduling

So 'Resource pool index' in a DCI format should be defined as

| sl-ResourcePoolID | 'Resource pool index' in a DCI format 3_0 |
| --- | --- |
| 0 | 000 |
| 1 | 001 |
| 3 | 010 |
| 4 | 011 |

- R2 clarify that the resource pool index in DCI format 3_0 is defined as the value is indexed sequentially from 0 in the same ascending order (based on the value of sl-ResourcePoolID-r16) of pools configured in sl-TxPoolScheduling.

Conclusion

We have the following proposals:

Proposal 1	R2 clarify that the resource pool index in DCI format 3_0 is defined as the value is indexed sequentially from 0 in the same ascending order (based on the value of sl-ResourcePoolID-r16) of pools configured in sl-TxPoolScheduling.