3GPP TSG RAN WG3 Meeting #120			R2-2211222

Toulouse, France, 14th – 18th Nov, 2022

Source: 	CATT

Title: 	Discussion on the PRU LS from SA2

Agenda item:	8.2.1

Document for: 	Discussion

Introduction

SA2 sent an LS on Positioning Reference Units in R2-2211130(S2-2209590). SA2 has two questions to RAN1 and CC the LS to RAN2. This contribution will discuss the two questions on PRU in Rel-18 from RAN2’s perspective.

Discussion

SA2 has studied the issue of how to support the Positioning Reference Units (PRU) in Rel-18 FS_eLCS_Ph3 study in TR 23.700-71. SA2 concluded that the PRU could be selected by an LMF to obtain measurements of RAN nodes to help improve location accuracy for all UEs and/or to assist the positioning of specific other UEs.

The PRU information stored in core network at least includes PRU location, mobility state (i.e. fixed or mobile) and PRU capability. The PRU capability indicates the capability of a PRU for supporting positioning activities, which includes the positioning signal transmission capability and positioning measurement capability on Uu and possibly PC5.

Regarding the conclusion, SA2 has following questions to RAN1:

- Question#1: Whether there is additional information that needs to be stored in the core network? If yes, what is the information?

- Question#2: Whether a PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5?

PRU was raised by RAN1 in Rel-17 which brings potential protocol impacts to SA2. So SA2 plan to support PRU in their Rel-18 which was now under discussion. RAN2 may further discuss how to enable to support PRU in Rel-18 based on the progress and requirement from SA2 and conclusion of RAN1.

RAN2 has reached below agreement on the LPP procedures when a PRU works as a UE in Rel-17.

Agreement:

RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer.

Agreements:

Proposal 3:	RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for PRU capability transfer.

Observation 1: RAN2 has confirmed that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer, location information transfer, and PRU capability transfer in RAN2#115-e and RAN2#116bis-e.

Proposal 1: RAN2 should enable the support of PRU in normative work following the progress and requirement of SA2 and RAN1.

As for SA2’s second question about “Whether a PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5”, in our view, an anchor UE at a known location can serve as a PRU for SL positioning. It should be beneficial for all SL-positioning methods, especially SL-TDOA. However it depends on the conclusion from RAN1. So we propose:

Proposal 2: RAN2 should enable the support of PRU on PC5 in normative work if RAN1 decides PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5 .

Conclusion

In this contribution, we provided our views on the PRU LS from SA2 in R2-2211130(S2-2209590), including observations and proposals as below:

Observation 1: RAN2 has confirmed that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer, location information transfer, and PRU capability transfer in RAN2#115-e and RAN2#116bis-e.

Proposal 1: RAN2 should enable the support of PRU in normative work following the progress and requirement of SA2 and RAN1.

Proposal 2: RAN2 should enable the support of PRU on PC5 in normative work if RAN1 decides PRU can be allowed to support the positioning signal transmission capability and signal measurement capability on PC5 .

References

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, December 2021.

[2] R1-2204805, Study on expanded and improved NR positioning, Intel, CATT, Ericsson, RAN1 #109-e, May 2022.

[3] Report from session on positioning and sidelink relay, RAN2 #115-e, Aug 2021.

[4] Report from session on positioning and sidelink relay, RAN2 #116-e, Nov 2021.

[5] Report from session on positioning and sidelink relay, RAN2 #116bis-e, Jan 2022.