3GPP TSG RAN WG2 #120                            		             R2-2211227

Toulouse 14th– 19th, 2022

Source:	CATT

Title:	Discussion on RAT dependent integrity

Agenda item:	8.2.3

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to further discuss the RAT-dependent positioning integrity with consideration of the following issues:

- FFS to reuse the DNU in RAT-dependent positioning integrity;

- Signaling procedure for LMF-based integrity;

- Draft TP for TR38.859 on RAT-dependent positioning integrity

Discussion

On the usage of DNU

In RAN2#119e, the following agreements were made on the concept of RAT-dependent integrity [2]. However, some companies doubt that whether all parameters should be used, e.g., DNU flag. From our perspective, LMF may provide the updated integrity assistance data to UE when ‘unhealthy’ happens, so there is no need to reuse DNU in RAT-Dependent integrity.

Agreements: Proposal 1: RAN2 to confirm the integrity principle of operation defined in the section 8.1.1a of TS38.305, including integrity definition (e.g., Error, Bound, Time to Alert, DNU, Residual Risk, irMinimum, irMaximum and Correlation Times; FFS if all parameters are needed in the RAT-dependent case), Equations for the GNSS integrity are reused for RAT dependent positioning methods.

Proposal 1: RAN2 to agree DNU is not introduced for RAT-dependent integrity.

LMF-based integrity

- LMF-based integrity for DL positioning

For LMF-based integrity for DL positioning, the positioning integrity calculation is performed by LMF. To assist the LMF to make positioning integrity calculation, the UE originated error sources as well as the NG-RAN node originated error sources should be provided to LMF.

Proposal 2: For LMF-based integrity for DL positioning, RAN2 to agree that UE provide the UE originated errors to LMF, i.e., the UE measurement errors which align with the agreement in RAN1.

Proposal 3: For LMF-based integrity for DL positioning, RAN2 to agree that NG-RAN node provide the NG-RAN node originated errors originated from NG-RAN to LMF, i.e., error in assistance data which align with the agreement in RAN1.

An example of the general procedure for LMF-based integrity for DL positioning procedure is given as the following.

Figure 1: LMF-based integrity for DL positioning procedure

- LMF-based integrity for UL positioning

For LMF-based integrity for UL positioning, the positioning integrity calculation is performed by LMF. To assist the LMF to make positioning integrity calculation, the UE originated error sources as well as the NG-RAN node originated error sources should be provided to LMF. However, since there is not any UL LPP assistance signalling, thus the UE should provide the UE originated error sources to its serving gNB, then the serving gNB provide the obtained UE originated error sources to LMF.

Proposal 4: For LMF-based integrity for UL positioning, RAN2 to agree that UE provide the UE originated errors which is defined by RAN1 to its serving NG-RAN node, and then the serving NG-RAN node forward such UE originated errors to LMF.

Proposal 5: For LMF-based integrity for UL positioning, RAN2 to agree that NG-RAN node provide the NG-RAN node originated errors originated from NG-RAN to LMF, e.g., TRP measurements errors.

An example of the general procedure for LMF-based integrity for UL positioning is given as the following.

Figure 2: LMF-based integrity for UL positioning procedure

- LMF-based integrity for DL&UL positioning

For LMF-based integrity for DL&UL positioning, the positioning integrity calculation is performed by LMF. To assist the LMF to make positioning integrity calculation, the UE originated error sources as well as the NG-RAN node originated error sources should be provided to LMF. Further, the UE originated error sources can be directly provided to LMF via the current LPP provide location information message directly.

Proposal 6: For LMF-based integrity for DL&UL positioning, RAN2 to agree that UE provide the UE originated errors to the LMF.

Proposal 7: For LMF-based integrity for DL&UL positioning, RAN2 to agree that NG-RAN node provide the NG-RAN node originated errors from NG-RAN to LMF.

An example of the general procedure for LMF-based integrity for DL&UL positioning procedure is given as the following.

Figure 3: LMF-based integrity for DL&UL positioning procedure

TP for TR38.859

Based on above analysis, as well as the agreements made by RAN1 [4][5], the TP on TS38.859 is given.

Proposal 8: For RAT-dependent integrity, RAN2 to agree the TP in annex 1 can be used as baseline for TR38.859.

Conclusions

In this contribution, we discussed the solutions for the integrity of RAT dependent positioning techniques. Based on the discussion, we have the following proposals:

On the usage of DNU

Proposal 1: RAN2 to agree DNU is not introduced for RAT-dependent integrity.

LMF-based integrity

Proposal 2: For LMF-based integrity for DL positioning, RAN2 to agree that UE provide the UE originated errors to LMF, i.e., the UE measurement errors which align with the agreement in RAN1.

Proposal 3: For LMF-based integrity for DL positioning, RAN2 to agree that NG-RAN node provide the NG-RAN node originated errors originated from NG-RAN to LMF, i.e., error in assistance data which align with the agreement in RAN1.

Proposal 4: For LMF-based integrity for UL positioning, RAN2 to agree that UE provide the UE originated errors which is defined by RAN1 to its serving NG-RAN node, and then the serving NG-RAN node forward such UE originated errors to LMF.

Proposal 5: For LMF-based integrity for UL positioning, RAN2 to agree that NG-RAN node provide the NG-RAN node originated errors originated from NG-RAN to LMF, e.g., TRP measurements errors.

Proposal 6: For LMF-based integrity for DL&UL positioning, RAN2 to agree that UE provide the UE originated errors to the LMF.

Proposal 7: For LMF-based integrity for DL&UL positioning, RAN2 to agree that NG-RAN node provide the NG-RAN node originated errors from NG-RAN to LMF.

TP for TR38.859

Proposal 8: For RAT-dependent integrity, RAN2 to agree the TP in annex 1 can be used as baseline for TR38.859.

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning.”

- Report of 3GPP TSG RAN WG2 meeting #119-e, Online

- MCC Support, “Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0”.

Annex: Text Proposal for TR 38.859

/****Start of the changes****/

6.1.2	Methodologies, procedures and signalling for determination of positioning integrity

- 6.1.2.1	Integrity Principle of Operation

For integrity operation, the network will ensure that:

P(Error > Bound for longer than TTA | NOT DNU) <= Residual Risk + IRallocation             (Equation 6.1.2.1-1)

for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum

for all the errors in Table 6.1.4-1, which are available.

Reuse the concepts and principles developed for RAT-Independent GNSS positioning integrity, FFS the usage of DNU.

Both UE-based and LMF-based integrity for RAT-dependent cases are studied and supported in integrity for RAT-dependent positioning.

- 6.1.2.2	Procedures and signalling for UE-Based positioning integrity

R17 UE-based integrity mode signaling can be used as baseline with the following aspects:

-	UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure

-	LMF sends the assistance data for integrity calculation to UE. LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode

-	UE sends integrity result to LMF using LPP location information Transfer message

Table 6.1.2-1: Identified procedures in UE-based positioning integrity mode

| UE-Based Positioning Methods | Errors indentified by RAN1 | LPP Procedures | NRPPa procedures |
| --- | --- | --- | --- |
| DL TDOA | - TRP location (e.g., NR-TRP-LocationInfo in [16]) Inter-TRP synchronization (e.g., NR-RTD-Info in [16]) | Assistance data from LMF to UE | From gNBs to LMF |
| DL AoD | - TRP location (e.g., NR-TRP-LocationInfo in [16]) - FFS: boresight direction of DL-PRS (e.g., NR-DL-PRS-BeamInfo in [16]) FFS: beam information of DL-PRS (e.g., NR-TRP-BeamAntennaInfo in [16]) | Assistance data from LMF to UE | From gNBs to LMF |

- 6.1.2.3	Procedures and signalling for LMF-Based positioning integrity

The LMF-Based positioning integrity includes the procedures as below:

-	UE sends capability info to LMF on integrity using LPP capability transfer procedure

-	UE provides the UE originated errors which is defined by RAN1 to LMF in DL and DL&UL positioning methods

-	UE provides the UE originated errors which are defined by RAN1 to gNB in UL positioning methods and gNB forwards the errors to LMF;

-	gNB provides the gNB originated errors which align with the agreement in RAN1 to LMF in DL, UL and DL&UL positioning methods

Table 6.1.2-2: Identified procedures in LMF-based positioning integrity mode

| LMF-Based Positioning Methods | Errors indentified by RAN1 | LPP Procedures | NRPPa procedure |
| --- | --- | --- | --- |
| DL-TDOA | - RSTD measurement - TRP location FFS: Inter-TRP synchronization | From UE to LMF | From gNBs to LMF |
| UL-TDOA | - RTOA measurement - TRP location - Inter-TRP synchronization | NA | From gNBs to LMF |
| Multi-RTT | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement - TRP location | From UE to LMF | From gNBs to LMF |
| UL-AoA | - Angle of arrival measurement - TRP location ARP location (e.g., ARPLocationInformation in [17]) | NA | From gNBs to LMF |
| DL-AoD | - TRP location | NA | From gNBs to LMF |

/****End of the changes****/