3GPP TSG RAN WG2#120		R2-2211230

Toulouse, France, 14th - 18th November, 2022

Source:	vivo

Title:          Discussion on sidelink positioning

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

At the last two RAN2 meetings, some agreements on remaining issues for further study were reached regarding the sidelink positioning.

Agreements: (RAN2#119e)

Introduce a new protocol for sidelink positioning procedures between UEs (name FFS, e.g., RSPP, SLPP).  FFS where it is specified.

The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355).

RAN2 will study the question of cast type for positioning signalling.  For SL-PRS, follow RAN1 decision and consider cast type if something arises in RAN2 scope.

Agreement: (RAN2#119bis-e)

Proposal 4(14/20) (modified): RAN2 do not decide to support the role of assistant UE for now.  FFS if there is spec impact in RAN2 from the assistant UE.

Indicate in the reply to SA2 that RAN2 have not concluded on the server UE functionalities but have agreed to follow SA2 decision on the definition of the server UE, and discussion continues.

Proposal 6 (modified): RAN2 shall study applicability of at least the following positioning signaling for groupcast/broadcast (in addition to unicast), including addressing any security aspects (involving SA3 where needed). FFS the specific use case:

•	SL positioning capability transfer

•	SL positioning assistance data

•	FFS SL location information transfer

Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of groupcast/broadcast to SLPP/RSPP group operation.  FFS if groupcast/broadcast operation, if supported, would be session-based or sessionless.

At the SA2#153 meeting, SA2 sent an LS [5] to RAN1/2/3 to ask for feedback on some issues. We think that the issues 1), 3), 4), 5) and 7) are related to RAN2.

In this contribution, we will discuss how to reply the SA2’s issues in the LS and would try to address the remaining issues in RAN2.

- Protocol stack

- Assistant UE

- Positioning parameters of SL Positioning discovery

- SL Positioning Server functionalities

- Cast types of SL positioning

- Partial coverage scenario

- General signaling procedure

- Discussion

- 2.1	Protocol stack

At the RAN2#119e meetings, RAN2 agreed to introduce a new protocol for sidelink positioning procedures between UEs and the name is FFS, e.g., RSPP (Ranging/Sidelink Positioning Protocol), SLPP (Sidelink Positioning Protocol).

According to SA2 TR [6], sidelink positioning includes relative positioning, ranging and absolute positioning.

Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information.

We understand that Ranging is one of sub-use cases of Sidelink Positioning. Therefore, we prefer the name SLPP.

Proposal 1: Use the SLPP (Sidelink Positioning Protocol) as the name of the new protocol for sidelink positioning procedures between UEs.

In the SA2’s LS[5], the issue 1) is related to the protocol used between UEs for sidelink positioning.

- SA2 concluded a Ranging/SL Positioning layer is introduced under Application layer; however, whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer is open. SA2 concluded that a new Ranging/Sidelink Positioning protocol (i.e. RSPP) will be used for SR5 over the PC5 reference point between the UEs (i.e. Target UE, Reference UE, Assistant UE, Located UE), which can be over PC5-S or PC5-U or (possibly partially) over PC5-D. The Pros & Cons are evaluated based on the following technical considerations: - PS5-S is currently designed for unicast link management. PC5-U supports all the cast types. However, security aspect on PC5-U and PC5-S for broadcast and group-cast modes need to be re-evaluated. - Impact to existing protocols: a standalone extension of PC5-S is expected if PC5-S is used, or RSPP is transported over PC5-U as the payload. Whether it is feasible or desirable to carry RSPP as payload (e.g. metadata) in PC5-D could not yet be concluded, given the lack of information on the potential size of RSPP messages. - QoS of RSPP transportation: AS layer needs to guarantee RSPP QoS in case of PC5-S is used, or V2X/ProSe layer can explicitly request per Application RSPP QoS in case of PC5-U is used. SA2 can’t reach consensus between PC5-S or PC5-U or PC5-D, and SA2 expects the RAN WG evaluation as the input to help making a decision in the conclusion.

SA2 concluded that whether the Ranging/SL Positioning layer is over V2X/ProSe layer or AS layer is open. We want to emphasize that SA2 only discussed the options over V2X/ProSe layer because the options over AS layer are in the scope of the RAN group.

For the options over V2X/ProSe layer, we understand that PC5-D option is not a real option because only the SL Positioning discovery is related to PC5-D. Therefore, there are two alternatives for the options over V2X/ProSe layer:

-	Alt 1: CP-based solution, new protocol over PC5-S;

-	Alt 2: UP-based solution, new protocol over PC5-U;

For the options over AS layer, the following alternatives should be considered:

-	Alt 3: CP-based solution, with new protocol over PC5-RRC.

-	Alt 4: CP-based solution, with new protocol over PDCP.

Figure 2.1-1: SLPP over PC5 RRC or PDCP

For the UP-based solution, one issue is that the transmission priority of SLPP messages transmitted via SL DRB cannot be guaranteed since the priority of SL DRB is always lower than that of SL SRB. Especially, when the UE has plenty of data to transfer and/or the sidelink resources are scarce, the time delay may be quite significant. However, the time delay is very important for positioning service since it is one of the positioning QoS parameters. Also, SLPP messages are most likely defined by RAN2. It is a little strange that SLPP messages defined by RAN2 are transmitted as user data. By comparison, the transmission priority of SLPP messages can be guaranteed in the CP-based solutions.

Among the CP-based solutions, the lower layer the SLPP is located on, the less the signaling overhead, processing overhead and positioning time delay will be. Therefore, we prefer Alt 4.

Proposal 2: Inform SA2 in the reply LS that SLPP layer is located over PDCP.

In the SA2’s LS[5], the issue 5) is related to the protocol used between UE and LMF for sidelink positioning.

- SA2 concluded that LMF may be involved when the Target UE and the Reference UE are both in network coverage, and the protocol used between UE and LMF can be a standalone extension of LPP, a new protocol or both, such that only this extension needs to be supported for UEs supporting only SL Positioning/Ranging. This extension and RSPP should be defined as common as possible. SA2 would like to understand whether this is feasible from RAN perspective?

In the last RAN2 meeting, RAN2 discussed the issue and achieved the conclusion that RAN2 will determine during normative work. We understand that RAN2 only needs to inform the conclusion to SA2 and no further discussion is needed.

Proposal 3: Inform SA2 in the reply LS that RAN2 will down-select protocol options for sidelink positioning procedures between UE and LMF during normative work.

- 2.2 Assistant UE

In the SA2’s LS[5], the issue 3) is related to the assistant UE.

- To support Ranging/SL Positioning using Assistant UE, how the determination of using assistant UE and the assistant UE selection/reselection is performed from RAN perspective?

In the RAN2#119bis meeting, RAN2 did not decide to support the role of assistant UE for now. We think that, from the view of RAN2, assistant UE relates to one complex Ranging/Sidelink positioning scenario, which can be broken down into two simple positioning scenarios without assistant UE. The scenario with assistant UE is more like something that the higher layer needs to consider. RAN2 should focus on the basic positioning procedure.

Proposal 4: Inform SA2 in the reply LS that RAN2 does not decide to support the role of assistant UE and has no information to feedback for the issue 3).

- 2.3	Positioning parameters of SL Positioning discovery

SA2 concluded to reuse 5G ProSe Discovery procedures and V2X Communication procedures with the additional Ranging/SL Positioning parameters. In the SA2’s LS[5], the issue 4) is related to whether those Ranging/SL Positioning parameters are transparent to ProSe/V2X layer or not.

- On Ranging/SL Positioning discovery, SA2 concluded to reuse 5G ProSe Discovery procedures and V2X Communication procedures with the additional Ranging/SL Positioning parameters; however, it is not decided whether those Ranging/SL Positioning parameters are transparent to ProSe/V2X layer or not, and SA2 would like to understand the views from RAN perspective.

The detailed parameters for positioning in discovery message are determined by RAN2 and are likely to be enhanced (e.g., introducing new parameters by RAN2) in the future. To reduce the coordination work between RAN2/SA2, we think that those Ranging/SL Positioning parameters should be designed as the format of container, which is transparent to ProSe/V2X layer.

Proposal 5: Inform SA2 in the reply LS that the sidelink positioning parameters in the discovery procedure are transparent to ProSe/V2X layer.

- 2.4 SL Positioning Server functionalities

In the last RAN2 meeting, RAN2 discussed the server UE functionalities and reached the following agreement:

Agreement:

Indicate in the reply to SA2 that RAN2 have not concluded on the server UE functionalities but have agreed to follow SA2 decision on the definition of the server UE, and discussion continues.

SA2 has defined the role of SL Positioning Server UE and is able to calculate the positioning result. In the SA2’s LS[5], the issue 4) is related to whether the SL Positioning Server functionalities can support more functionalities in addition to result calculation, from RAN perspective.

- A SL Positioning Server UE can be discovered and selected for result calculation for the case of partial coverage and out of coverage, in case a constrained UE is not able to support all SL Positioning/Ranging features. Whether the SL Positioning Server functionalities can support more functionalities, e.g. SL Positioning/Ranging method determination, operation coordination, resource coordination and scheduling, in addition to result calculation is FFS. SA2 would like to understand whether this is reasonable from RAN perspective.

At the RAN2#119bis-e meeting, the following server functionalities were discussed in an e-mail discussion [7]:

a) : managing the overall co-ordination and scheduling of resources

b): determining type and number of position methods

c): determine how many and which UEs act as anchor UEs

20 companies participated in the discussion and more than half of the companies (11 out of 20 companies) thought that the SL Positioning Server functionalities need to support more functionalities. Anyway, SA2 has defined the role of SL Positioning Server UE and the above 3 functionalities are needed for SL positioning. We think that the SL Positioning Server UE should have all three of these functionalities.

Proposal 6: Inform SA2 in the reply LS that the sidelink positioning server should support the following additional functionalities from RAN2 perspective:

a): managing the overall co-ordination and scheduling of resources

b): determining type and number of position methods

c): determine how many and which UEs act as anchor UEs

We understand that if having the related capability, target UE and anchor UE can act as location server UE, as below:

Figure 2.4-1: locate server UE

According to the SID [4], the sidelink positioning feature needs to support multiple use cases, including V2X, public safety, commercial and IIOT. Therefore, there may be various UE capabilities. Depending on UE capability, the above 3 cases may be used. For example, for the IIOT case, the capability of the target UE may be limited and Case 2 may be not applicable.

Proposal 7: With specific capability, target UE or anchor UE can act as sidelink positioning server.

- 2.5	Cast types of SL positioning

For sidelink positioning, RAN1 has agreed to support multiple positioning methods:

Agreement With regards to the Positioning methods supported using SL measurements study further the following methods: - RTT-type solutions using SL - Study both single-sided (also known as one-way) and double-sided (also known as two-way) RTT - SL-AoA - Include both Azimuth of arrival (AoA) and zenith of arrival (ZoA) in the study - SL-TDOA - SL-AoD - Corresponds to a method where RSRP and/or RSRPP measurements similar to the DL-AoD method in Uu. - Include both Azimuth of departure (AoD) and zenith of departure (ZoD) in the study

It is likely that one anchor UE supports some methods, but doesn’t support others. Anchor UE’s capability should be obtained before being selected. If the capability exchange is only via a sidelink unicast connection, the sidelink unicast connection should be set up first. If the anchor UE’s capability does not fulfil the requirements, the anchor UE cannot be selected and the unicast connection should be released. Broadcast/groupcast of positioning capability benefits for anchor UE selection to avoid unnecessary sidelink unicast connection setup. Therefore, we propose to support the broadcast/groupcast for UE sidelink positioning capability.

If anchor UE can autonomously determine its SL PRS configuration, broadcast/groupcast of SL positioning assistance data is similar to posSIB and benefits for signaling overhead reduction.

If the location information of anchor UE is broadcast besides the SL positioning assistance data, the capable target UE can perform the positioning measurement and calculation without a sidelink unicast connection setup. The time delay is also quite short since there is no signaling exchange via unicast.

Proposal 8: To improve the efficiency of sidelink positioning, the broadcast/groupcast for SL positioning assistance data and UE sidelink positioning capability shall be supported.

Besides,at RAN2#119bis-e meeting, RAN2 reached the following agreements:

Agreements:

Proposal 1 (modified): RAN2 agrees to support unicast SLPP/RSPP session-based operation and to study the applicability of groupcast/broadcast to SLPP/RSPP group operation.  FFS if groupcast/broadcast operation, if supported, would be session-based or sessionless.

Proposal 3 (modified): RAN2 agrees to support at least unicast SLPP/RSPP “centralized” operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs.  RAN2 will follow SA2 on which UE(s) can perform the calculation and related RAN1 definitions.

To our understanding, the existing session establishment is between AMF and LMF, which is in the scope of SA2. Thus, we think a similar procedure between UEs shall be discussed by SA2. From the view of RAN2, the term session-less operation seems to be a similar concept with the broadcast-based SL positioning without SL unicast connection setup, i.e., target UE receives assistant information broadcasted by anchor UEs, measures the SL PRSs broadcasted by anchor UEs and then calculates its location. If so, we shall focus on the solution for different cast types instead of introducing another new term. Moreover, so far it is unclear whether to support the SL positioning scenario without SL unicast connection setup.

Proposal 9: RAN2 does not consider the term “Session-based” or “Sessionless”, and just focuses on the cast type of SL positioning, i.e., unicast/broadcast/groupcast.

- 2.6 Partial coverage scenario

In the last meetings, some companies considered using the UE-to-NW relay to cope with the partial coverage scenario. In Rel-17, the sidelink relay is introduced to provide connectivity to the network for U2N Remote UE(s). Both L2 and L3 U2N Relay architectures are supported. For L3 U2N Relay, the architecture is transparent to the serving RAN of the U2N Relay UE, except for controlling sidelink resources. According to TS23.304 [2], the control plane protocol stack is as below:

Figure 2.6-1: Control plane protocol stacks for L3 U2N relay

To our understanding, the U2N relay feature was developed mainly for (plenty of) user data relay. If the U2N relay is used for sidelink positioning in the partial coverage scenario, there are the following drawbacks:

- It requires that the related UEs must support the relay feature and also the operator must deploy the relay in the network (RAN and core network). Otherwise, the SL positioning in the partial coverage scenario can not work. This greatly restricts the applicable scenario.

- The relay connection establishment procedure includes relay connection setup, relay SRB/DRB setup and relay security activation, which will increase the signaling overhead and the positioning time delay.

- For L3 U2N relay, the transmission path for NAS messages is quite long (Remote UE -> Relay UE -> RAN -> UPF -> N3IWF -> AMF), which increases the positioning time delay.

The sidelink positioning procedure is only related to control signaling interaction. The amount of exchanged messages between LMF and UE is quite limited. It is completely unnecessary to use the U2N relay feature. We understand that another solution is message forwarding via control signaling. Figure 1 shows an example that LMF sends the message 1 to UE2 via UE1:

Figure 2.6-2: message forwarding via control signaling

Proposal 10: Instead of U2N relay, message forwarding via control signaling can be considered for positioning in partial coverage scenario.

- 2.7	General signaling procedures

- 2.7.1 LMF-dependent sidelink positioning signaling procedures

The following Figure 2.7-1 illustrates a typical signaling procedure for LMF-dependent sidelink positioning.

Figure 2.7-1: LMF-dependent sidelink positioning signaling procedures

1.	The LMF receives a Location Service Request from the AMF.

2.	The LMF obtains the sidelink positioning capability of the target UE from the target UE, e.g., via the LPP Request Capabilities message.

3.	The LMF determines to perform the sidelink positioning, e.g., when the UE is not able to perform Uu-based positioning (not support, or not able to measure on enough TRP to perform Uu-based positioning).

4.	The LMF obtains the information of Anchor UE(s), e.g., the target UE may initiate the discovery procedure over the PC5 interface to find the Anchor UE(s) and sends the information of Anchor UE(s) to the LMF.

5.	The LMF may perform the exchange of LPP messages for sidelink positioning between the LMF and Anchor UE(s), e.g., Assistance Data Transfer and Location Information Transfer.

6.	The LMF performs the exchange of LPP messages for sidelink positioning between the LMF and the target UE, e.g., Assistance Data Transfer and Location Information Transfer.

7.	The LMF obtains the location information of the target UE based on the information from Step 5 or 6.

8.	The LMF sends Location Service Response to the AMF.

Proposal 11: For LMF-dependent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Potential standard impact:

-	New or enhanced existing signaling procedure to exchange capability for sidelink positioning.

-	New or enhanced existing discovery procedure to obtain the information of Anchor UEs. (in SA2 scope, if reuse the ProSe discovery procedure)

-	New or enhanced existing signaling procedure to exchange the assistant information and measurement result/position information for sidelink positioning.

Proposal 12: For LMF-dependent sidelink positioning, the potential standard impact includes enhancing the LPP messages about Capability transfer, Assistance Data transfer, and Location Information Transfer.

- 2.7.2 LMF-independent sidelink positioning signaling procedures

The following Figure 2.7-2 illustrates a typical signaling procedure for LMF-independent sidelink positioning.

Figure 2.7-2: LMF-independent sidelink positioning signaling procedures

1.	The Location Server UE receives a Sidelink Location Request message, or the Location Server UE itself determines the need for some location service.

2.	The Location Server UE may initiate the discovery and/or sidelink unicast link establishment with target UE and Anchor UEs.

3.	The Location Server UE initiates the sidelink positioning capability exchange procedure with the target UE and Anchor UEs.

4.	The Location Server UE performs the assistance data transfer procedure with target UE and Anchor UEs.

5.	The target UE transmits the SL PRS and the Anchor UEs measure the SL PRS, or the Anchor UEs transmit the SL PRS and the target UE measures the SL PRS.

6.	The Location Server UE performs the location information transfer procedure with the target UE or Anchor UEs.

7.	The LMF obtains the location information of the target UE based on the information from Step 6.

8.	The LMF provides the location estimate via Sidelink Location Response.

Proposal 13: For LMF-independent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Potential standard impact:

-	New protocol for sidelink positioning may need to be introduced, including new signaling procedures for sidelink positioning capability exchange, assistance data transfer and location information transfer.

-	New signaling procedure for Sidelink Location Request/Response. (in SA2 scope, if the request/response are LCS messages)

-	New or enhanced existing discovery procedure. (in SA2 scope, if reuse the ProSe discovery procedure)

Proposal 14: For LMF-independent sidelink positioning, the potential standard impact includes new SLPP messages for the procedures between UEs, including Capability transfer, Assistance Data transfer, and Location Information Transfer.

- Conclusion

In the contribution, we have the following proposals:

Protocol name

Proposal 1: Use the SLPP (Sidelink Positioning Protocol) as the name of the new protocol for sidelink positioning procedures between UEs.

Reply LS to SA2

Proposal 2: Inform SA2 in the reply LS that SLPP layer is located over PDCP.

Proposal 3: Inform SA2 in the reply LS that RAN2 will down-select protocol options for sidelink positioning procedures between UE and LMF during normative work.

Proposal 4: Inform SA2 in the reply LS that RAN2 does not decide to support the role of assistant UE and has no information to feedback for the issue 3).

Proposal 5: Inform SA2 in the reply LS that the sidelink positioning parameters in the discovery procedure are transparent to ProSe/V2X layer.

Proposal 6: Inform SA2 in the reply LS that the sidelink positioning server should support the following additional functionalities from RAN2 perspective:

a): managing the overall co-ordination and scheduling of resources

b): determining type and number of position methods

c): determine how many and which UEs act as anchor UEs

SL Positioning Server

Proposal 7: With specific capability, target UE or anchor UE can act as sidelink positioning server.

Cast types of SL positioning

Proposal 8: To improve the efficiency of sidelink positioning, the broadcast/groupcast for SL positioning assistance data and UE sidelink positioning capability shall be supported.

Proposal 9: RAN2 does not consider the term “Session-based” or “Sessionless”, and just focuses on the cast type of SL positioning, i.e., unicast/broadcast/groupcast.

Partial coverage scenario

Proposal 10: Instead of U2N relay, message forwarding via control signaling can be considered for positioning in partial coverage scenario.

General signaling procedures

Proposal 11: For LMF-dependent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Proposal 12: For LMF-dependent sidelink positioning, the potential standard impact includes enhancing the LPP messages about Capability transfer, Assistance Data transfer, and Location Information Transfer.

Proposal 13: For LMF-independent sidelink positioning, capture the above signaling procedures into the TR as the baseline.

Proposal 14: For LMF-independent sidelink positioning, the potential standard impact includes new SLPP messages for the procedures between UEs, including Capability transfer, Assistance Data transfer, and Location Information Transfer.

- Reference

- 3GPP TS 23.304 v17.4.0

- 3GPP TS 38.300 v17.1.0

- 3GPP TR 23.700-71 V0.4.0

- RP-213588		Revised SID on Study on expanded and improved NR positioning

- R2-2211139 	LS on RAN dependency for Ranging/Sidelink Positioning	SA2

- 3GPP TR 23.700-86 Study on Architecture Enhancement to support Ranging based services and sidelink positioning

- R2-2210909	Report of [AT119bis-e][423][POS] LS to SA2 on SL positioning terminology (Xiaomi)