3GPP TSG-RAN WG2 #120 	R2-2211270

Toulouse, France, 14th – 18th November, 2022

Title:		Discussion on RedCap Positioning

Source:	Huawei, HiSilicon

Agenda item:	8.2.5

Document for:	Discussion and Decision

# Introduction

RedCap positioning was discussed in RAN2-119bis-e [3] and the following agreements were made:

- Information on RedCap UE capability from the UE to the LMF (e.g., restricted bandwidth capability) can be discussed in WI phase - RAN2 wait for RAN1 conclusion on the signalling design for assistance data for frequency hopping.

We discuss the progress in RAN1 and present the enhancements needed in RAN2.

# Discussion

## 2.1	Summary of RAN1’s progress and discussion

RAN1 agreements from RAN1#110bis-e [2] are listed below:

Agreement For the evaluation of TX/RX frequency hopping for positioning of redcap UEs, the value of the gap between two consecutive hops includes at least from 100us to 5ms. - Companies should indicate if other smaller values are used in their evaluations, and justify the feasibility of smaller values Agreement Study the potential enhancement of the UL SRS for positioning to enable Tx frequency hopping, including but not limited to partial overlapping between hops, hopping bandwidth, time gap between frequency hopping. Agreement Study the potential enhancement of the DL PRS to enable Tx or Rx frequency hopping, including but not limited to impact on processing capability, hopping bandwidth in the positioning frequency layer, time gap between frequency hopping, measurement period, partial overlapping between hops. Agreement For the evaluation of TX/RX frequency hopping for positioning of redcap UEs, the value of UE speed includes 3 km/h, 30 km/h, 60km/h. - Other values are not precluded

For better understanding, we explain frequency hopping transmission and reception as proposed by companies in RAN1 in the following sub-sections.

### 2.1.1	Frequency Hopping Transmission

The two potential candidate solutions are:

Solution 1a: Non-overlapped Frequency Hopping Transmission

UE transmits the SRS in a frequency hopping way, then the TRP performs TOA measurements utilizing several consecutive hops. For example, the TRP could combine the SRSs from several consecutive hops (e.g. 5 hops) into a “wide-band” (e.g. 100MHz or close to 100MHz) SRS and obtains a TOA result. Another way may be the TRP performs TOA measurement using the received “narrow-band” (e.g. 20MHz) SRS for each hop and averages all results across several hops.

Solution 1b: Overlapped Frequency Hopping Transmission

Similarly, UE transmits the SRS in a frequency hopping way. The main difference is we consider an overlapped part in the frequency domain for two adjacent hops to solve the random phase offset issue introduced by the frequency hopping process. Then the TRP performs phase estimation and compensation for every two adjacent hops. The final positioning result is obtained by utilizing the “wide-band” SRS spliced by the SRS from several consecutive hops.

Solution 1a. Non-overlapped Frequency Hopping Transmission

Solution 1b. Overlapped Frequency Hopping Transmission

Figure 1: Potential candidate solutions with Frequency Hopping Transmission for RedCap positioning

### 2.1.2	Frequency Hopping Reception

The two potential candidate solutions for PRS reception in a frequency hopping way were discussed and are shown in Figure 2. For instance, the TRP can transmit a “wide-band” (e.g., 100MHz) PRS, while the receiver (RedCap UEs) could perform the TOA measurement. Similarly, two sub-solutions are proposed in RAN1 meetings, i.e., Non-overlapped Frequency Hopping Reception (Solution 2a) and Overlapped Frequency Hopping Reception (Solution 2b).

Solution 2a. Non-overlapped Frequency Hopping Reception

Solution 2b. Overlapped Frequency Hopping Reception

Figure 2: Potential candidate solutions with Frequency Hopping Reception for RedCap positioning

As seen from RAN1’s agreements, RAN1 will evaluate/study Tx/Rx frequency hopping, potential enhancement of the UL SRS for positioning to enable Tx frequency hopping and potential enhancement of the DL PRS to enable Tx/Rx frequency hopping.

Observation 1: RAN1 will study potential enhancement of

- the UL SRS for positioning to enable Tx frequency hopping

- the DL PRS to enable Tx or Rx frequency hopping

## 2.2	Potential Enhancements in RAN2

With the assumption that RAN1 agrees frequency hopping for UL SRS and/or DL PRS, the configuration and switching mechanism of the hops may be specified in RAN2.

BWP switching allows activating an inactive BWP and deactivating an active BWP simultaneously. BWP switching can be triggered by any of the following:

- DCI based switching: DCI commands (DCI 1-1 and DCI 0-1) are used to schedule data and allows switching pre-configured BWPs.

- RRC signalling: With RRC Reconfiguration or SCell activation, BWP can be switched for transmission and reception.

- bwp-InactivityTimer: If there is no data transmission or reception at the UE for a long time, switching to a BWP with a relatively small bandwidth happens at the UE.

As the bandwidth capability of RedCap UE is reduced to 20MHz, the frequency hopping solution may be performed by BWP switching.

Proposal 1: RAN2 confirms that switching of frequency hopping for UL SRS and/or DL PRS is enabled by BWP switching.

3GPP TS 38.133 [4] specified BWP switch delay requirements. For RRC-based BWP switching, the BWP switch delay is 6ms. For DCI and timer-based BWP Switching mechanisms, BWP switch delay requirements are given in Table 1.

Table 1: DCI and timer-based BWP switch delay requirements

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

We can observe that the current BWP switch time is slot-level. Assuming 5 hops for the frequency hopping, the total BWP switching delay will be higher and may have a negative impact on positioning accuracy. So RAN2 may need to study enhancements to BWP switching mechanisms to enable “short” switching time between adjacent hops.

Observation 2: Enhancements to BWP switching mechanisms may be needed to fulfil the requirements of positioning accuracy.

Proposal 2: RAN2 to study enhancements to BWP switching mechanisms to enable “short” switching time between adjacent hops for frequency hopping in the WI phase.

# 3.	Conclusion

In this contribution, we discussed the progress of RedCap positioning in RAN1 and made the following observations and proposals:

Observation 1: RAN1 will study potential enhancement of

- the UL SRS for positioning to enable Tx frequency hopping

- the DL PRS to enable Tx or Rx frequency hopping

Proposal 1: RAN2 confirms that switching of frequency hopping for UL SRS and/or DL PRS is enabled by BWP switching.

Observation 2: Enhancements to BWP switching mechanisms may be needed to fulfil the requirements of positioning accuracy.

Proposal 2: RAN2 to study enhancements to BWP switching mechanisms to enable “short” switching time between adjacent hops for frequency hopping in the WI phase.

# 4.	References

- 3GPP RP-213561, “New SID on Study on expanded and improved NR positioning”

- RAN1 Chair’s Notes RAN1#110bis-e, RAN1#110bis-e, e-Meeting Oct, 2022.

- RAN2 Chair’s Notes RAN2#119bis-e, RAN2#119bis-e, e-Meeting Oct, 2022.

- 3GPP TS 38.133 v16.1.0, “Requirements for support of radio resource management”