3GPP TSG-RAN WG2 Meeting #120	R2- 2211275

14 – 18 November 2022, Toulouse, France

Agenda item:	8.5.4.1

Source:	Dell Technologies

Title:	BSR feedback enhancements for XR

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

Based on the Rel-18 study item description ‘’NR enhancements for XR‘’, RAN2 is to study potential scheduling and resource allocation improvements for XR services as follows.

- Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements. - Dynamic scheduling/grant enhancements.

During the last RAN2 meeting, the following agreements have been reached, on the major directions of technical improvements to be considered, where enhancements of the existing buffer status report (BSR) formulation and reporting procedures are included.

- As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before). - Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling which should be considered as baseline. Should justify why enhancements are needed. - RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR. FFS if SPS has some power consumption benefits.

Therefore, this contribution presents our views on potential BSR enhancements, on BSR report formulation, table formats, and report triggering criteria, taking the XR-specific traffic profile characteristics into consideration.

# Discussions

Buffer status report (BSR) is vital for dynamic uplink scheduling. With BSR reports, UEs signal serving cells with an indication of the currently buffered uplink traffic size. Per [1], there are two BSR tables defined as short and long BSR reports. A short BSR report is 5 bit long while the long BSR is 8 bit long. As shown by the below Table 6.1.3.1-2 of the long BSR report, each BSR index is associated wit a predefined uplink traffic size interval in the uplink buffers. The larger the BSR index, the larger the range of the buffered traffic size.

In the current specifications, the BSR quantization follows an exponential function as follows:

Bk = Bmin (1+p) k, with p = (Bmax / Bmin)1 / (N-1) – 1

where  Bmin  and  Bmax  are the minimum and maximum possible buffer size which a UE can report. As can be observed, the quantization precision reduces for the larger buffer size intervals. Accordingly, the network will not be exactly aware of how much traffic is being buffered at the reporting device with a considerable standard deviation over the larger buffered traffic sizes. Accordingly, this leads the network to either schedule and allocate too much or too little uplink resource for the actual buffered uplink traffic size. Therefore, this results in extended buffering delays for potential latency critical XR traffic or a degraded spectral efficiency due to the resource over scheduling.

Existing BSR tables and reporting criteria were not problematic for eMBB and URLLC traffic since,

- For eMBB traffic: traffic size is large; however, there is not a tight latency budget as XR traffic, accordingly, it is reasonable for the network to schedule uplink resource as of multiple times corresponding to several BSR transmissions.

- For URLLC traffic: traffic size is typically very small which falls in the high BSR precision quantization area, and hence, the existing BSR tables are suitable for offering a good reporting accuracy of URLLC payload.

Although, for the large XR traffic size, there is a tight latency budget, i.e., packet delay budget (PDB) associated with the XR payload. Accordingly, current BSR tables and reporting procedures need to be further optimized to offer a further faster and more fine grained quantization precision.

Observation 1: Current BSR tables and reporting criteria are suitable for conventional eMBB services since those are not tied with a limited radio latency budget. Thus, the larger quantization errors of the larger buffer range are reduced in time by the multiple gNB scheduling instants.

Observation 2: Current BSR tables and reporting criteria should be further optimized to match the XR-specific traffic profile characteristics and latency targets. Thus, a more precise BSR quantization and reporting procedures are needed.

Proposal 1: New BSR tables, formats, and reporting criteria are needed for XR traffic.

Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 560 | 128 | ≤ 31342 | 192 | ≤ 1754595 |
| 1 | ≤ 10 | 65 | ≤ 597 | 129 | ≤ 33376 | 193 | ≤ 1868488 |
| 2 | ≤ 11 | 66 | ≤ 635 | 130 | ≤ 35543 | 194 | ≤ 1989774 |
| 3 | ≤ 12 | 67 | ≤ 677 | 131 | ≤ 37850 | 195 | ≤ 2118933 |
| 4 | ≤ 13 | 68 | ≤ 720 | 132 | ≤ 40307 | 196 | ≤ 2256475 |
| 5 | ≤ 14 | 69 | ≤ 767 | 133 | ≤ 42923 | 197 | ≤ 2402946 |
| 6 | ≤ 15 | 70 | ≤ 817 | 134 | ≤ 45709 | 198 | ≤ 2558924 |
| 7 | ≤ 16 | 71 | ≤ 870 | 135 | ≤ 48676 | 199 | ≤ 2725027 |
| 8 | ≤ 17 | 72 | ≤ 926 | 136 | ≤ 51836 | 200 | ≤ 2901912 |
| 9 | ≤ 18 | 73 | ≤ 987 | 137 | ≤ 55200 | 201 | ≤ 3090279 |
| 10 | ≤ 19 | 74 | ≤ 1051 | 138 | ≤ 58784 | 202 | ≤ 3290873 |
| 11 | ≤ 20 | 75 | ≤ 1119 | 139 | ≤ 62599 | 203 | ≤ 3504487 |
| 12 | ≤ 22 | 76 | ≤ 1191 | 140 | ≤ 66663 | 204 | ≤ 3731968 |
| 13 | ≤ 23 | 77 | ≤ 1269 | 141 | ≤ 70990 | 205 | ≤ 3974215 |
| 14 | ≤ 25 | 78 | ≤ 1351 | 142 | ≤ 75598 | 206 | ≤ 4232186 |
| 15 | ≤ 26 | 79 | ≤ 1439 | 143 | ≤ 80505 | 207 | ≤ 4506902 |
| 16 | ≤ 28 | 80 | ≤ 1532 | 144 | ≤ 85730 | 208 | ≤ 4799451 |
| 17 | ≤ 30 | 81 | ≤ 1631 | 145 | ≤ 91295 | 209 | ≤ 5110989 |
| 18 | ≤ 32 | 82 | ≤ 1737 | 146 | ≤ 97221 | 210 | ≤ 5442750 |
| 19 | ≤ 34 | 83 | ≤ 1850 | 147 | ≤ 103532 | 211 | ≤ 5796046 |
| 20 | ≤ 36 | 84 | ≤ 1970 | 148 | ≤ 110252 | 212 | ≤ 6172275 |
| 21 | ≤ 38 | 85 | ≤ 2098 | 149 | ≤ 117409 | 213 | ≤ 6572925 |
| 22 | ≤ 40 | 86 | ≤ 2234 | 150 | ≤ 125030 | 214 | ≤ 6999582 |
| 23 | ≤ 43 | 87 | ≤ 2379 | 151 | ≤ 133146 | 215 | ≤ 7453933 |
| 24 | ≤ 46 | 88 | ≤ 2533 | 152 | ≤ 141789 | 216 | ≤ 7937777 |
| 25 | ≤ 49 | 89 | ≤ 2698 | 153 | ≤ 150992 | 217 | ≤ 8453028 |
| 26 | ≤ 52 | 90 | ≤ 2873 | 154 | ≤ 160793 | 218 | ≤ 9001725 |
| 27 | ≤ 55 | 91 | ≤ 3059 | 155 | ≤ 171231 | 219 | ≤ 9586039 |
| 28 | ≤ 59 | 92 | ≤ 3258 | 156 | ≤ 182345 | 220 | ≤ 10208280 |
| 29 | ≤ 62 | 93 | ≤ 3469 | 157 | ≤ 194182 | 221 | ≤ 10870913 |
| 30 | ≤ 66 | 94 | ≤ 3694 | 158 | ≤ 206786 | 222 | ≤ 11576557 |
| 31 | ≤ 71 | 95 | ≤ 3934 | 159 | ≤ 220209 | 223 | ≤ 12328006 |
| 32 | ≤ 75 | 96 | ≤ 4189 | 160 | ≤ 234503 | 224 | ≤ 13128233 |
| 33 | ≤ 80 | 97 | ≤ 4461 | 161 | ≤ 249725 | 225 | ≤ 13980403 |
| 34 | ≤ 85 | 98 | ≤ 4751 | 162 | ≤ 265935 | 226 | ≤ 14887889 |
| 35 | ≤ 91 | 99 | ≤ 5059 | 163 | ≤ 283197 | 227 | ≤ 15854280 |
| 36 | ≤ 97 | 100 | ≤ 5387 | 164 | ≤ 301579 | 228 | ≤ 16883401 |
| 37 | ≤ 103 | 101 | ≤ 5737 | 165 | ≤ 321155 | 229 | ≤ 17979324 |
| 38 | ≤ 110 | 102 | ≤ 6109 | 166 | ≤ 342002 | 230 | ≤ 19146385 |
| 39 | ≤ 117 | 103 | ≤ 6506 | 167 | ≤ 364202 | 231 | ≤ 20389201 |
| 40 | ≤ 124 | 104 | ≤ 6928 | 168 | ≤ 387842 | 232 | ≤ 21712690 |
| 41 | ≤ 132 | 105 | ≤ 7378 | 169 | ≤ 413018 | 233 | ≤ 23122088 |
| 42 | ≤ 141 | 106 | ≤ 7857 | 170 | ≤ 439827 | 234 | ≤ 24622972 |
| 43 | ≤ 150 | 107 | ≤ 8367 | 171 | ≤ 468377 | 235 | ≤ 26221280 |
| 44 | ≤ 160 | 108 | ≤ 8910 | 172 | ≤ 498780 | 236 | ≤ 27923336 |
| 45 | ≤ 170 | 109 | ≤ 9488 | 173 | ≤ 531156 | 237 | ≤ 29735875 |
| 46 | ≤ 181 | 110 | ≤ 10104 | 174 | ≤ 565634 | 238 | ≤ 31666069 |
| 47 | ≤ 193 | 111 | ≤ 10760 | 175 | ≤ 602350 | 239 | ≤ 33721553 |
| 48 | ≤ 205 | 112 | ≤ 11458 | 176 | ≤ 641449 | 240 | ≤ 35910462 |
| 49 | ≤ 218 | 113 | ≤ 12202 | 177 | ≤ 683087 | 241 | ≤ 38241455 |
| 50 | ≤ 233 | 114 | ≤ 12994 | 178 | ≤ 727427 | 242 | ≤ 40723756 |
| 51 | ≤ 248 | 115 | ≤ 13838 | 179 | ≤ 774645 | 243 | ≤ 43367187 |
| 52 | ≤ 264 | 116 | ≤ 14736 | 180 | ≤ 824928 | 244 | ≤ 46182206 |
| 53 | ≤ 281 | 117 | ≤ 15692 | 181 | ≤ 878475 | 245 | ≤ 49179951 |
| 54 | ≤ 299 | 118 | ≤ 16711 | 182 | ≤ 935498 | 246 | ≤ 52372284 |
| 55 | ≤ 318 | 119 | ≤ 17795 | 183 | ≤ 996222 | 247 | ≤ 55771835 |
| 56 | ≤ 339 | 120 | ≤ 18951 | 184 | ≤ 1060888 | 248 | ≤ 59392055 |
| 57 | ≤ 361 | 121 | ≤ 20181 | 185 | ≤ 1129752 | 249 | ≤ 63247269 |
| 58 | ≤ 384 | 122 | ≤ 21491 | 186 | ≤ 1203085 | 250 | ≤ 67352729 |
| 59 | ≤ 409 | 123 | ≤ 22885 | 187 | ≤ 1281179 | 251 | ≤ 71724679 |
| 60 | ≤ 436 | 124 | ≤ 24371 | 188 | ≤ 1364342 | 252 | ≤ 76380419 |
| 61 | ≤ 464 | 125 | ≤ 25953 | 189 | ≤ 1452903 | 253 | ≤ 81338368 |
| 62 | ≤ 494 | 126 | ≤ 27638 | 190 | ≤ 1547213 | 254 | > 81338368 |
| 63 | ≤ 526 | 127 | ≤ 29431 | 191 | ≤ 1647644 | 255 | Reserved |

# BSR Table enhancements

## Improvements of existing BSR tables

Existing BSR reporting procedures, with XR traffic, may mislead gNBs about the actual buffered uplink traffic size, and accordingly, in case of resource over scheduling, the overall spectral efficiency is degraded, which leads to a lower XR device capacity.

Observation 3: The larger buffer size interval of the larger BSR indices lead to the network to either schedule too much or too little resources for the corresponding uplink traffic, resulting in a degraded radio latency and/or spectral efficiency.

In case RAN2 is not to introduce new BSR tables during Rel-18, existing BSR tables and reporting conditions should be further optimized and updated for XR traffic. One simple solution, to further refine the problematic large quantization errors over the larger buffered traffic size intervals, is to use a fixed and more fine-grained quantization step across the entire BSR size range. This way, the quantization precision is unified across all intervals of the uplink buffered traffic. Although, this simple solution leads to significantly increasing the BSR signalling overhead and required number of BSR reporting bits.

Proposal 2: Adopt a fixed and precise quantization step across all BSR traffic size intervals.

Proposal 3: Increase number of BSR reporting bits (> 5 & 8 bits) to allow for more fine-grained quantitation and a larger buffer size range.

To ensure compatibility, UEs may be required to report indications, towards serving gNBs, of the supported BSR tables, and accordingly, the gNBs are able to configure those with the appropriate BSR table which best matches its expected traffic profile, from a list of available BSR tables.

Proposal 4: RAN2 is kindly to study UEs reporting supported BSR tables, and accordingly, for XR capable devices, new and finer BSR tables can be dynamically adopted.

## Introduce new BSR tables for XR traffic

As adopting a fixed BSR quantization step seems to be a simple solution, with a minimal standardization effort, for enhancing the BSR quantization precision, it may significantly increase the BSR reporting overhead, especially for UEs that may not require such high BSR precision, e.g., URLLC or eMBB devices.

Observation 4: For XR uplink video uploading, the expected traffic size is large and thus, the higher BSR indices are to be used for most of the time. With lower BSR granularity over the higher traffic size ranges, the gNB may need several BSR reports for fully scheduling existing uplink traffic in the buffer, extending the exhibited traffic buffering latency. This problem is likely not applicable for XR uplink traffic of pose and/or control updates.

Therefore, RAN2 may discuss and agree on introducing a new BSR table format, which can be dynamically constructed at UEs based on a configured set of quantization steps versus the various BSR traffic size intervals. In particular, a list of BSR step sizes, i.e., a step size group, is configured to UEs, where each corresponds to a certain range the buffered traffic size. Correspondingly, the network is able to configure a certain BSR step size, and hence, a certain BSR precision, for each buffered traffic size interval.

Thus, as shown by Fig. 1, several BSR aggregate traffic size ranges are defined, where for each, a specifically configured quantization step size is used. This gives the network the control over which buffered traffic size range to assign a higher BSR quantization precision and which to relax the BSR quantization step. Based on the configured step size group and respective traffic size range set, UEs dynamically construct the corresponding BSR table and hence, similarly to conventional BSR reporting, UEs report the BSR index which corresponds to the actual traffic size interval.

In principle, the proposed BSR table format can be suitable for various types of traffic profiles. The network can still trade-off the BSR reporting overhead versus the desired BSR reporting precision. For example, for critical XR traffic, the network configures XR devices with a BSR step group that dictates a precise and fine-grained quantization step over the larger buffered traffic size ranges while adopting a relaxed, i.e., larger, step size for the lower traffic sizes. Hence, the BSR overhead can dynamically controlled while still offering precise BSR reporting over the traffic size that is most likely to be always experienced.

Figure 1: Dynamic BSR table format.

Proposal 5: For generality of BSR tables, RAN2 to kindly study that BSR step sizes and corresponding buffer size intervals can be dynamically configured and signalled from gNBs such as the appropriate BSR table is dynamically constructed at the UEs.

Proposal 6: Network configures devices with a BSR step size group, which indicates multiple BSR quantization steps, each corresponding to a certain buffered traffic size range. UEs accordingly dynamically construct the dynamic BSR tables.

# BSR reporting enhancements

Existing BSR can be triggered based on three triggering criteria as follows:

- Regular BSR: BSR reporting is triggered when a new and higher-priority PDU is available in the buffer and retxBSR-Timer is expired.

- Periodic BSR: BSR reporting is triggered when periodicBSR-Timer is expired.

- Padding BSR: BSR reporting is triggered when the padding bits are equal or larger than the size of the BSR MAC CE, and where the BSR can be piggybacked within an allocated uplink resource set.

Even if we assumed that XR traffic is always considered as higher priority than other traffic in the uplink buffers, still, multiple BSR transmissions may be needed to carry information on the actual available traffic in the buffers, which may jeopardize the radio latency target. Hence, new triggering criteria are needed for XR BSR reporting.

Proposal 6: RAN2 to study new triggering criteria of BSR reporting.

In one simple solution, the longer BSR format, i.e., 8-bit BSR table, is only triggered when there are data flows of multiple logical channels (LCH) in the buffers. When a single LCH is active, the short BSR report is triggered instead. Since the longer BSR format offers a more precise BSR accuracy and extended BSR traffic size ranges, we suggest triggering  the longer BSR format even when there is a single active LCH in the buffer, for XR traffic.

Proposal 7: Change the long BSR reporting condition to be always reported even in case of a single LCH -presence.

The XR traffic is characterized by a target tight PDB, unlike eMBB traffic. However, existing BSR reports only include information of the buffered traffic size, regardless of the timing or latency limitations associated with this buffered traffic. So, in one option, a BSR report is triggered when the XR traffic buffering delay exceeds a predefined threshold. Or, in another option, a BSR reporting is triggered when the remaining PDB of the available buffered payload is below a threshold. This way, the reporting trigger of the BSR is dynamically tied to the critical delay targets of XR traffic.

Proposal 8: Consider buffered packet timing or latency information as triggering conditions for BSR reporting. In particular, a UE triggers a BSR transmission when there is an XR PDU set available in the buffer and for which the buffering delay exceeds a predefined threshold.

Proposal 9: RAN2 to study and agree on potential latency/delay criteria for triggering BSR reporting. Examples include remaining PDB of an XR PDU set available in the buffer.

# Conclusions

This contribution has presented Dell’s views on enhancing the existing BSR reporting procedures, for XR capacity enhancements. The following observations and proposals are made:

Observation 1: Current BSR tables and reporting criteria are suitable for conventional eMBB services since those are not tied with a limited radio latency budget. Thus, the larger quantization errors of the larger buffer range are reduced in time by the multiple gNB scheduling instants.

Observation 2: Current BSR tables and reporting criteria should be further optimized to match the XR-specific traffic profile characteristics and latency targets. Thus, a more precise BSR quantization and reporting procedures are needed.

Proposal 1: New BSR tables, formats, and reporting criteria are needed for XR traffic.

Observation 3: The larger buffer size interval of the larger BSR indices lead to the network to either schedule too much or too little resources for the corresponding uplink traffic, resulting in a degraded radio latency and/or spectral efficiency.

Proposal 2: Adopt a fixed and precise quantization step across all BSR traffic size intervals.

Proposal 3: Increase number of BSR reporting bits (> 5 & 8 bits) to allow for more fine-grained quantitation and a larger buffer size range.

Proposal 4: RAN2 is kindly to study UEs reporting supported BSR tables, and accordingly, for XR capable devices, new and finer BSR tables can be adopted.

Observation 4: For XR uplink video uploading, the expected traffic size is large and thus, the higher BSR indices are to be used. With lower BSR granularity over higher traffic size ranges, the gNB may need several BSR reports for fully scheduling existing uplink traffic in the buffer, extending the exhibited traffic buffering latency. This problem is likely not applicable for XR uplink traffic of pose and/or control updates.

Proposal 5: For generality of BSR tables, RAN2 to kindly study that BSR step sizes and corresponding buffer size intervals can be dynamically configured and signalled from gNBs such as the appropriate BSR table is dynamically constructed at the UEs.

Proposal 6: Network configures devices with a BSR step size group, which indicates multiple BSR quantization steps, each corresponding to a certain buffered traffic size range. UEs accordingly dynamically construct the dynamic BSR tables.

Proposal 7: RAN2 to study new triggering criteria of BSR reporting.

Proposal 8: Change the long BSR reporting condition to be always reported even in case of a single LCH -presence.

Proposal 9: Consider buffered packet timing or latency information as triggering conditions for BSR reporting. In particular, a UE triggers a BSR transmission when there is an XR PDU set available in the buffer and for which the buffering delay exceeds a predefined threshold.

Proposal 10: RAN2 to study and agree on potential latency/delay criteria for triggering BSR reporting. Examples include remaining PDB of an XR PDU set available in the buffer.

# References

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".