3GPP TSG-RAN WG2 Meeting #120	R2- 2211276

14 – 18 November 2022, Toulouse, France

Agenda item:	8.5.4.2

Source:	Dell Technologies

Title:	CG scheduling enhancements for XR

Document for:	Decision, Discussion

# 1	Introduction

Based on the Rel-18 study item description ‘’NR enhancements for XR‘’, RAN2 is to study potential scheduling and resource allocation improvements for XR services as follows.

- Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements. - Dynamic scheduling/grant enhancements.

Therefore, this paper presents our views on potential configured grant (CG) scheduling enhancements, by taking into account the XR traffic special characteristics.

# 2	Discussion

XR traffic [1] entails various types of streams that may require different quality of service (QoS) targets. For example, video or audio traffic streams may arrive with a periodicity of 16.66 ms while pose or control traffic streams may arrive with a periodicity of 4 ms. Furthermore, packets of different XR streams may have different packet delay budgets (PDBs), and accordingly, from the scheduling perspective, those should be handled differently.

# 3         Enhancement on configured grant (CG) scheduling

Dynamic scheduling (DG) for uplink traffic requires a transmission cycle of a scheduling request, buffer status reporting, and accordingly, an uplink grant, respectively. Therefore, it exhibits additional scheduling delay from the moment there is a buffered uplink traffic at UEs. Thus, configured grant (CG) scheduling is a more latency friendly scheduling strategy for latency critical traffic. With CG scheduling, multiple uplink resource occasions, with predetermined resource size, are pre-allocated for UEs to transmit their buffered uplink traffic without triggering the SR and BSR transmission cycle. However, CG is deemed best suitable when the traffic periodicity almost follows a predictable periodicity and respective average stream traffic size is not highly time varying.

For XR traffic, the XR traffic volume, and hence, expected buffered XR traffic size, can considerably change in time due to the adaptive selection of different encoding strategies or video codecs. Hence, the amount of resources in the uplink directions required for carrying buffered traffic changes considerably as well. Therefore, current CG scheduling procedures, with pre-configured uplink resource size, may not be efficient for XR traffic. This leads to UEs needing several consecutive CG occasions for transmitting their full uplink buffers, increasing the XR traffic buffering delay.

Observation 1: The typical XR traffic size is time-varying due to the dynamic change of adopted XR codecs or encoding strategies. Therefore, conventional preconfigured CG allocations may not be sufficient to carry the full buffered uplink XR traffic.

One straight forward solution is to configure multiple periodic CG occasions where they offer more successive uplink transmission opportunities for UEs, without the need for SR and BSR transmission or exhibiting extended uplink traffic buffering delays. However, this clearly not a resource efficient strategy, especially for XR streams with different arrival periodicities.

Observation 2: For XR traffic, multiple CG allocations with different allocation sizes should be active to carry the time varying buffered XR traffic size, which is a resource inefficient.

A hybrid scheduling procedure, with DG and CG, can be an efficient way of overcoming the CG drawback for XR traffic. Specifically, CG scheduling can be adopted for a first uplink buffer transmission and followed by a DG scheduling instant for the remaining traffic in the buffer. For the DG scheduling part, and to avoid the delay for transmitting the SR-BSR-grant, the triggering conditions of the BSR can be updated such that a BSR transmission is immediately triggered when the available buffered traffic required an uplink resource size that is exceeding the configured CG occasion. The network can proactively configure UEs with advance DG scheduling grants for the excess buffered CR traffic, as shown by Fig. 1.

Figure 1: Hybrid CG and DG scheduling.

Proposal 1: RAN2 is kindly to study dynamic CG allocation size adaptation with respect to buffered XR traffic size, where a hybrid dynamic and configured grant scheduling is adopted.

Proposal 2: Discuss and agree on triggering a BSR report transmission from UEs when the buffered XR traffic size should require more uplink resources than the configured CG occasions.

Furthermore, as various XR streams require different PDBs and QoS targets, those that are latency critical must be guaranteed to scheduled faster than others which are latency tolerant or not equally important as the former. Despite that existing BSR reporting procedures enable a BSR feedback per LCG; however., the received uplink grant from the gNB is always understood as an aggregate grant for al available LCG in the uplink buffer. Thus, there is no guarantee that UEs will schedule the LCGs of the critical streams first. Thus, it can beneficial for the uplink grant to include indications of the LCG to be transmitted over the allocated resources. This requires introducing new IEs in the existing downlink control information (DCI) formats for uplink scheduling.

Observation 3: Uplink scheduling grant does not specify a single LCH for transmission in the uplink. Thus, UEs assume the granted uplink transmissions for all available LCGs in the uplink buffers.

Proposal 3: RAN2 to kindly study and agree on possible LCG indications in the scheduling uplink grants to guarantee that UEs are offloading their uplink buffers of the indicated latency critical LCGs.

# 4	Conclusions

This contribution discusses potential enhancements on scheduler for XR, the following are the observations and proposals that may be taken into consideration.

Observation 1: The typical XR traffic size is time-varying due to the dynamic change of adopted XR codecs or encoding strategies. Therefore, conventional preconfigured CG allocations may not be sufficient to carry the full buffered uplink XR traffic.

Observation 2: For XR traffic, multiple CG allocations with different allocation sizes should be active to carry the time varying buffered XR traffic size, which is a resource inefficient.

Proposal 1: RAN2 is kindly to study dynamic CG allocation size adaptation with respect to buffered XR traffic size, where a hybrid dynamic and configured grant scheduling is adopted.

Proposal 2: Discuss and agree on triggering a BSR report transmission from UEs when the buffered XR traffic size should require more uplink resources than the configured CG occasions.

Observation 3: Uplink scheduling grant does not specify a single LCH for transmission in the uplink. Thus, UEs assume the granted uplink transmissions for all available LCGs in the uplink buffers.

Proposal 3: RAN2 to kindly study and agree on possible LCG indications in the scheduling uplink grants to guarantee that UEs are offloading their uplink buffers of the indicated latency critical LCGs.

# References

- TR 38.838, V17.0.0, Technical Specification Group Radio Access Network; Study on XR (Extended Reality) Evaluations for NR (Release 17), 3GPP