3GPP TSG-RAN WG2 Meeting #120                                              	R2-2211280

Toulouse, France, 14th - 18th November 2022

Source:	CATT

Title:	Consideration on Service Continuity Enhancements for L2 U2N Relay

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, regarding to service continuity enhancements for L2 U2N relay, some agreements involving general principle were achieved [1]:

Agreements:

Proposal 1 (modified)	For i2i path switch procedure, introduce a new measurement event based on individual thresholds i.e., Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2.  FFS if we also have an event Z2: Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE, and in this case if/how to compare SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE.

Proposal 3 For i2i scenario, re-use the SL-RSRP or SD-RSRP measurement quantities for path switching.

Proposal 4 (modified)	For i2i scenario, serving/candidate U2N relay UEs, when SL-RSRP is unavailable, SD-RSRP is used as the measurement quantity.  Wording can be revisited if it is determined that L2IDs for U2U and U2N are always different (so that candidate U2N relay UEs would never have SL-RSRP available).

Proposal 5 For i2d path switch scenario, re-use the existing T304 timer

Proposal 6 For d2i and i2i path switch scenarios, re-use the existing T420 timer.

In this contribution, we continue to discuss the leftover issues.

Discussion

  - RRM measurement

Measurement event(s)

In last meeting, only one measurement event Z1 that Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2 was agreed. But event Z2 is FFS. To discuss the feasibility of event Z2, two questions needs to be clarified:

- Question 1: If the remote UE already has a connection with a good quality relay UE, whether it needs to find another candidate relay UE?

- Question 2: Whether and how to compare the SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE?

For question 1, the legacy event A3 is compared between the serving cell and the candidate cell, therefore if it is possible, change to a better relay which has better channel condition can have benefit for the PC5 link reliability and stability.

And for question 2, since the serving relay UE may not always be active exchange of data, the SD-RSRP can be used for the serving side; but the SL-RSRP cannot be measured for the candidate side. Therefore, if it is not reasonable to compare between the SL-RSRP and the SD-RSRP, the event Z2 can be used if the serving and the candidate have the same measured quantity, e.g. SD-RSRP.

Proposal 1: Introduce the measurement events Z2 (Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE) for indirect-to-indirect path switching, at least for the same measured quantity (SD-RSRP) of serving and the candidate.

To evaluate a single target node, the legacy Event Y2 introduced in R17 could be reused for indirect-to-indirect path switching: Candidate L2 U2N Relay UE becomes better than threshold.

Proposal 2: The legacy Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can also be used for indirect-to-indirect path switching.

  - Which node to decide the target U2N relay UE

For the node chosen to decide the path switching, the procedure of legacy HO can be used for reference:

- In XnAP, the HANDOVER REQUEST message will be sent to the target NG-RAN node and carry Target Cell Global ID of the target cell;

- In the inter-node message of HandoverPreparationInformation nested in the HANDOVER REQUEST message, a list of recommended best candidate cell(s) on each frequency of this NG-RAN node for which measurement information was available can be optionally included, together with the measurement result of each cell;

- Based on all the information received from the source NG-RAN node and the information got in the target node itself, e.g. the load of different cells, the target NG-RAN node could choose any cell under the node to include it in the HO command.

Figure 1 Legacy inter-gNB handover procedures

It is the RAN3 agreement that “Source gNB decides to trigger path switching for the L2N remote UE”. Based on the path switching decision, 2 issues should be made clear step by step:

- Which node should decide the new path i.e., either a cell or a relay UE;

- Which information should be sent from the source gNB to the target gNB.

For 1), all measurement results which can be measured by the UE are reported to the source gNB. The measurement object of frequency can be tied to only one report configuration which includes only one measurement event, therefore the source gNB could deduce the best candidate target cell/relay UE reported by the UE. If more than one measured event or a periodical measurement is report to the source gNB, it is reasonable the source gNB decide the target gNB and send the HANDOVER REQUEST message to the target gNB. The WA provided by RAN3 is also said that “Source gNB selects the target path type (direct or indirect)” [2]. Therefore, the source gNB is responsible to choose the target gNB(s) and a best target cell or a best target relay UE (which means the target path type) belongs to the gNB. The best target cell or the best target relay UE belongs to the gNB can be transmitted via XnAP which is RAN3 scope.

Observation 1: The source gNB is responsible for choosing the target gNB and the target path type, i.e. a best target cell or a best target relay UE belongs to the target gNB.

Meanwhile, from RAN2 point of view, to align with the legacy procedure, a set of candidate relay UEs could be transmitted to the target gNB, together with the legacy candidate cells included in the inter-node message of HandoverPreparationInformation. After the target gNB received the HANDOVER REQUEST message, the target gNB could choose a cell or a relay UE with lower traffic load based on the load information count by the target gNB itself and the information received from the source gNB, and to decide whether the cell itself or a relay UE under the cell is appropriate for the new path of the UE.

Proposal 3: The target gNB is responsible for deciding the final target cell or the final target relay UE under the cell.

For 2), based on the proposal above, the transmitted information between gNBs can be enhanced by:

- In XnAP, a target relay UE ID can be included, together with the legacy Target Cell Global ID;

- In the inter-node message of HandoverPreparationInformation, a set of candidate relay UE ID(s) and the related measurement result can be included, together with the legacy field of candidateCellInfoList.

Proposal 4: The candidate relay UE ID(s) and the related measurement result(s) can be sent to the target gNB via inter-node message, besides the legacy candidate cell information.

  - Lossless data delivery

For the source indirect link, the data will be sent based on the PC5 link between the remote and the relay UE, and the Uu link between the relay UE and the gNB. For DL, the data that have been acked by the relay UE to the source gNB may not been transmitted to remote UE; and for UL, data that have been acked by the relay UE to the remote UE may not been transmitted to the gNB. Therefore if legacy data forwarding procedure is performed between the source gNB and the target gNB, the packets which are not exactly transmitted between the remote UE and the source gNB will not be accurately reported to the target gNB.

It is RAN3 agreement [3] to wait for RAN2 progress about the lossless data delivery during path switch. The summary about this issue in last meeting proposed to reuse Rel-17 DL/UL lossless delivery using PDCP status reports with no specification impact [4]. In R17 there has the agreement that “The DL/UL lossless delivery during the path switch is done according to the PDCP status report” indeed, but this agreement has not been captured in the specification of TS38.323 actually [5]:

## 5.5 Data recovery For AM DRBs, when upper layers request a PDCP data recovery for a radio bearer, the transmitting PDCP entity shall: - perform retransmission of all the PDCP Data PDUs previously submitted to re-established or released AM RLC entities in ascending order of the associated COUNT values for which the successful delivery has not been confirmed by lower layers, following the data submission procedure in clause 5.2.1. After performing the above procedures, the transmitting PDCP entity shall follow the procedures in clause 5.2.1.

Therefore, it is suggest RAN2 to discuss how to solve the problem of data loss due to legacy data forwarding involving source indirect link.

Observation 2: The R17 agreement of “The DL/UL lossless delivery during the path switch is done according to the PDCP status report” has not been captured in the specification actually.

Proposal 5: RAN2 to discuss the solution for the problem of data loss during path switch.

  - Path switching failure

Similar as HO failure in Uu interface, it is possible for a remote UE suffers path switching failure. The path switching failure maybe occurred due to the following causes:

Cause 1: the remote UE does not receive the path switching command from the source gNB/source relay UE in time, because the source PC5 link is failure before receiving the path switching command;

Cause 2: the remote UE failed to establish the PC5 link with the target relay UE, because the channel condition between target relay UE and the remote UE is changed during that long time period of path switching preparation;

Cause 3: the IDLE/INACTIVE state target relay UE failed to establish the Uu link with the target gNB, because the channel condition between target relay UE and the gNB is changed during that long time period of path switching preparation.

The path switching failure due to the three causes above could be existed in R17 intra-gNB indirect-to-direct and direct-to-indirect path switching, and the probability of failure maybe raised for R18 inter-gNB indirect-to-direct and direct-to-indirect path switching and the indirect-to-indirect path switching.

Therefore for simplification, the legacy UE action of chooses a cell/relay for re-establishment could be considered.

Proposal 6: The remote UE performs RRC re-establishment in a cell or a relay UE when the path switching failure.

  - Context Release in source relay UE

For the procedure of indirect to direct path switching within the same gNB in R17, it is described in TS38.300 [6] that:

6. The gNB sends RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

Figure 2 Procedure for indirect to direct path switching (intra-gNB)

In R17, there is only one gNB and the gNB can decide when to release the source path.

For the inter-gNB indirect-to-direct and the indirect-to-indirect path switching in R18, since the source and the target network node can be different gNBs, it is necessary to discuss in which node to decide the release of the source connection. There are two possible methods:

- Option 1: The source gNB can reuse the legacy R17 RRCReconfiguration message to indicate the U2N Relay UE to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC.

- Option 2: After the remote UE connects to the target U2N relay, the target gNB could indicate to the source gNB, and the source could ask the source U2N relay to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC.

For simplicity and to align with R17 network behavior, it is suggest reusing option 1 for R18 that is the source gNB can reuse the legacy R17 RRCReconfiguration message to indicate the source U2N Relay UE to release the UE context of the remote UE, even before the remote UE connects to the target relay UE.

Proposal 7: The source gNB can send the RRCReconfiguration message to the source U2N Relay UE to release the remote UE context at any time after the source gNB sends the RRCReconfiguration message with sync to the remote UE, without the target gNB indication.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: Introduce the measurement events Z2 (Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE) for indirect-to-indirect path switching, at least for the same measured quantity (SD-RSRP) of serving and the candidate.

Proposal 2: The legacy Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can also be used for indirect-to-indirect path switching.

Observation 1: The source gNB is responsible for choosing the target gNB and the target path type, i.e. a best target cell or a best target relay UE belongs to the target gNB.

Proposal 3: The target gNB is responsible for deciding the final target cell or the final target relay UE under the cell.

Proposal 4: The candidate relay UE ID(s) and the related measurement result(s) can be sent to the target gNB via inter-node message, besides the legacy candidate cell information.

Observation 2: The R17 agreement of “The DL/UL lossless delivery during the path switch is done according to the PDCP status report” has not been captured in the specification actually.

Proposal 5: RAN2 to discuss the solution for the problem of data loss during path switch.

Proposal 6: The remote UE performs RRC re-establishment in a cell or a relay UE when the path switching failure.

Proposal 7: The source gNB can send the RRCReconfiguration message to the source U2N Relay UE to release the remote UE context at any time after the source gNB sends the RRCReconfiguration message with sync to the remote UE, without the target gNB indication.

Reference

- RAN2-119bis-e-Positioning-Relay-2022-10-19-2000_EOM.docx

- RAN3_117bis-e_agenda_20221018_EOM1.doc

- RAN3_117-e_agenda_20220825_EOM1.doc

- R2-2210782	 Summary of AI 8.9.3, Service Continuity Enhancements Ericsson

- TS 38.323 Packet Data Convergence Protocol (PDCP) specification

- TS38.300 NR and NG-RAN Overall Description; Stage 2