3GPP TSG-RAN WG2 Meeting #120	 R2- 2211285

Toulouse FR, 14th – 18th Nov 2022

Agenda Item:	7.2.3

Souce:	MediaTek Inc.

Title:	Discusson on epoch time

Document for:		Discussion and decision

1 Introduction

In RAN2-119bis-e, RAN2 has agreed to capture the RAN1 agreement of epoch time:

Agreements:

- Add the following clarification to the field description of epochTime:

For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received.

2.    In case of HO/CHO, the SFN indicated by epochTime is the frame nearest to the frame where RRC reconfiguration message is received

RRC CR [1] was agreed in principal.

However, we see some ambiguities in current definition of epoch time. In this discussion paper, we discuss the cases of ambiguity and propose corresponding solutions.

2 Discussion

Serving cell epoch time

For serving cell, the epoch time was described in [1] as follows:

”For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. ”

The SI messages are transmitted within periodically occurring time domain windows (referred to as SI-windows). Within the SI-window, the corresponding SI message can be transmitted a number of times. UEs can successfully decode SI message in the different position of SI window, thus UEs can have different SI received time(SFN).There is a case where UEs have different SI received time may have different interpretations of epoch time. See Figure 1, UE1 and UE2 decode SI message in different position of SI window. If the epoch time(T2) indicated in the SIB31 minus 1024 SFN (T1) is located in the duration of SI window which carries the SIB31,  the UE2 can identify T2 as the epoch time, however UE1 which decodes the SI message before T1 will identify T1 as the epoch time because T1 can fit the serving cell epoch time description: “the next upcoming SFN after the frame where the message indicating the epochTime is received”.

Figure 1 different interpretations of serving cell epoch time

Observation 1: UEs decode SI message in different position of SI window can have different interpretations of serving cell epoch time.

To fix this ambiguity, the description of serving cell epoch can be modified to:” For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the last frame of SI window where the message indicating the epochTime is received.” This change rules out any SFNs in the middle of SI window, hence the UE1 and UE2 will have the same interpretation of serving cell epoch time. See Figure 2.

Figure 2 Same interpretations of epoch time after change the description of serving cell epoch time

Proposal 1: Change the serving cell epoch time description to:” For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the last frame of SI window where the message indicating the epochTime is received.”

Neighbor cell epoch time

Neighbor cell ephemeris in SIB31 was indicated by RAN4 LS[2] but has not yet agreed by RAN2. If it is agreed, the same ambiguity can happen. Referring to the RAN1 agreement from RAN1#110, the neighbor cell epoch is described as :” For neighbor cell if EpochTime is indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame nearest to the frame where the message indicating the Epoch time is received.”

UE1 and UE2 decode the SI message in different position of SI window. See figure 3, according the above description, it is possible that UE1 and UE 2 have different interpretations of neighbor cell epoch time.

Figure 3 Different interpretations of neighbour cell epoch time in SIB31

Observation 2: UEs decode SI message in different position of SI window can have different interpretations of neighbor cell epoch time.

Same as serving cell epoch time, as long as long the various reception time of SI message was replaced by a fixed position of SI window, the ambiguity can be fixed.

Proposal 2: If neighbor cell epoch time in SI is agreed, the neighbor cell epoch time description can be: ”For neighbor cell if EpochTime is indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame nearest to the last frame of SI window where the message indicating the Epoch time is received.”

For the case of neighbour cell epoch time in HO/CHO message, there is same ambiguity exists. Due to RLC segmentation/HARQ retransmission/RLC retransmission/NW resource allocation, the time of receiving RRCConnectionReconfiguration message varies. See Figure 4, UE may have different interpretations of neighbour cell epoch time in HO/CHO based on different time of decode success. The later epoch time(T2) could be very wrong.

Figure 4 Different interpretation of neighbour cell epoch time in HO/CHO message

Observation 3: UE can have different interpretations of neighbor cell epoch time in HO/CHO message due to different reception time.

It is rather harder to find a fixed reference point to inference epoch time, only NW implementation can ensure that when UE receives epoch time UE can inference the right epoch time. As an example, if such a retransmission carrying HO/CHO will indicate a different epoch time than it should be, NW stop this retransmission.

Proposal 3: It is up to NW implementation to fix different interpretation of neighbor cell epoch time in HO/CHO message due to different reception time.

3 Conclusions

Observation 1: UEs decode SI message in different position of SI window can have different interpretations of serving cell epoch time.

Proposal 1: Change the serving cell epoch time description to:” For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the last frame of SI window where the message indicating the epochTime is received.”

Observation 2: UEs decode SI message in different position of SI window can have different interpretations of neighbor cell epoch time.

Proposal 2: If neighbor cell epoch time in SI is agreed, the neighbor cell epoch time description can be: ”For neighbor cell if EpochTime is indicated explicitly by a SFN and subframe number, the UE considers this frame to be the frame nearest to the last frame of SI window where the message indicating the Epoch time is received.”

Observation 3: UE can have different interpretations of neighbor cell epoch time in HO/CHO message due to different reception time.

Proposal 3: It is up to NW implementation to fix different interpretation of neighbor cell epoch time in HO/CHO message due to different reception time.

4 References

- R2-2211020, Miscellaneous corrections to TS 36.331 for IoT NTN.

- R2-2211169, Reply LS on measurement gap enhancements for NTN (R4-2217175; contact: Apple)