3GPP TSG-RAN WG2 Meeting #120	 R2-2211290

Toulouse FR, 14th – 18th Nov 2022

Agenda Item:	8.6.4

Souce:	MediaTek Inc.

Title:	On Enhancements to discontinuous coverage

Document for:	    Discussion and Decision

# 1 Introduction

In Rel-17 IoT NTN, discontinuous coverage was introduced. As a satellite moves on a specified orbit, for example in case of a NGSO satellite, the satellite beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the satellite. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g., a sparse satellite constellation deployment. In this document, we discuss an enhancement of discontinuous coverage.

In this contribution, we discuss a further enhancement for discontinuous coverage, as indicated in IoT-NTN Work Item, approved at RAN Plenary #96e [1].

# 2 Discussion

In NTN, before connection establishment, UE needs to re-acquire SIB31 and perform GNSS measurement. The time taken varies from several seconds to tens of seconds depends on the different GNSS status (cold start, warm start). And the connection establishment procedure itself was prolonged due to the UE-eNB RTT.

The time of coverage of earth moving cell depends on the footprint size of the satellite. In IoT NTN, the maximum footprint size for LEO is 1000km, then the maximum coverage time is about 130 seconds. It can be much less when it comes to real network.

Therefore, we can see that it is very possible that when UE try to initiate a connection establishment, the remaining time of coverage is too short to allow connection establishment to complete.

Observation 1: In earth moving cell, it is very possible that when UE try to initiate a connection establishment, the remaining time of coverage is too short to allow connection establishment to complete.

If the UE can predict the time of losing coverage, UE can check whether the remaining time of current cell’s coverage it long enough to accommodate a connection establishment. If it can’t, we propose that UE is allowed to not initiate the connection establishment to save power consumption.

For quasi-earth fixed cell, t-service in SIB3 is provided, it indicates the time when cell stop provide coverage. However, for earth moving cell, there is no information in Rel-17 for UE to predict the time of losing coverage. Footprint information of current cell can be provided for this purpose.

Proposal 1: For earth-moving cell, the serving cell footprint information is broadcast for determining the time of loss of coverage of current cell.

Proposal 2: UE calculates the time of losing coverage before entering RRC connected mode. If the remaining time of current cell’s coverage is too short to accommodate a connection establishment, UE is allowed to not initiate the connection establishment.

# 3 Conclusions

Observation 1: In earth moving cell, it is very possible that when UE try to initiate a connection establishment, the remaining time of coverage is too short to allow connection establishment to complete.

Proposal 1: For earth-moving cell, the serving cell footprint information is broadcast for determining the time of loss of coverage of current cell.

Proposal 2: UE calculates the time of losing coverage before entering RRC connected mode. If the remaining time of current cell’s coverage is too short to accommodate a connection establishment, UE is allowed to not initiate the connection establishment.

# 4 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.