3GPP TSG-RAN WG2 Meeting #120                                                R2-2211309

Toulouse, France, 14th Nov – 18th Nov, 2022

Source:	CATT

Title:	Discussion on introducing satellite assistance information for neighbour cells in SIB31

Agenda Item:	7.2.3

Document for:	Discussion and Decision

Introduction

In last meeting, introducing satellite assistance information for neighbour cells in SIB31 was discussed, but no conclusion was achieved. In this contribution, we give some discussion on this issue.

Discussion

In Rel-17 NR NTN, NTN-specific SIB19 is introduced, and in addition to serving cell NTN configuration, the neighbour cell configuration is also included in SIB19. The neighbour cell configuration can be used to help UE keep time/frequency synchronization with the neighbour cell SSBs, or help UE adjust SMTC correctly. So companies suggested to introducing neighbor assistance information for neighbour cells also in IoT NTN specific SIB31.

During the discussion of last meeting on this topic [1], the main concern of the companies, who were negative to introduce neighbor assistance information for neighbour cells also in SIB31, is it is too late at this Rel-17 stage to do this, which is an optimization and can be discussed in Rel-18. Another concern was the TB size restriction of IoT specific SIB.

We tend to agree with concern above to introduce neighbor assistance information for neighbour cells in SIB31, and we have an assumption that the coverage of IoT NTN may be not continuous in the early deployments. Furthermore, it seems in NB-IoT it has not been introduced measurement enhancement like SMTC4list or propDelayDiffReportConfig defined in NR NTN, the acquisition of which needs the satellite assistance information of the neighbor cells. So the benefit is not clear to introduce the satellite assistance information of the neighbor cells in IoT-NTN now. So we also think we can discuss this issue in Rel-18 if necessary.

Proposal 1: In Rel-17, RAN2 not to discuss the issue of introducing satellite assistance information for neighbour cells in SIB31.

Conclusion

In this contribution, we give some discussion on the issue of introducing satellite assistance information for neighbour cells in SIB31, and the following proposal is given:

Proposal 1: In Rel-17, RAN2 not to discuss the issue of introducing satellite assistance information for neighbour cells in SIB31.

Reference

- R2-2210848 Report of [Offline-107][IoT NTN] RRC corrections (Huawei)