3GPP TSG-RAN WG2 Meeting #120	R2-2211340

# Toulouse, France, November, 2022

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0574 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NTN Stage-2 correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - UE can ignore NTN neighbor cell frequency for measurement if its assistance information is absent in SIB19. However, for TN frequency which does not have assistance infomration in SIB19, UE still needs to perform measurements for NTN-TN mobility. - In NGSO scenarios, UE needs to autonomously pre-compensate the frequency Doppler shift when performing neighbour cell measurement, based on UE position and neighbour cell ephemeris information. This is missing in the current stage-2 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To correct that ignoring measurement only applies to NTN neighbor cell frequency. - Add the missing part on frequency Doppler shift pre-compensation for neighbor cell measurement. Impact analysis Impacted functionality: Rel-17 NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE’s measurement behavor is not correct and frequency Doppler shift pre-compensation for neighbor cell measurement is missing in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.14.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based on R2-2211046 which is in-principle agreed at RAN2#119bis-e. |  |  |  |  |  |  |  |  |

1st Change

16.14.3.3	Measurements

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities;

-	measurement gaps based on multiple SMTCs;

-	assistance information (e.g., ephemeris, Common TA parameters) provided via system information for UE to perform measurement on neighbour cells in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED..

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information (e.g. ephemeris, common TA parameters).

UE assistance information is in the form of the service link propagation delay difference(s) between the serving cell and neighbour cell(s).

When the assistance information of an NTN neighbour cell frequency is absent in SIB19, the NTN neighbour cell frequency can be ignored by the UE when performing measurements. To perform neighbour cell measurements, the UE autonomously pre-compensates the frequency Doppler shift by considering UE position and neighbour cell’s assistance information.

In the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE:

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving cell and a distance threshold.

Measurement rules for cell re-selection with timing information and location information are specified in clause 5.2.4.2 in 38.304 [10].