3GPP TSG RAN WG2 Meeting #120                                                         R2-2211378

# Toulouse, France, 14th-18th November 2022

Agenda item:	8.5.2.1

Source:	Intel Corporation

Title:	DRB mapping for XR specific requirements

Document for:	Discussion and decision

# Introduction

This document discusses the potential target requirements for RAN2 to provide differentiated handling of XR traffic over the air. These requirements are later evaluated for the four alternatives, captured in TR 38.835 [1], on how the DRB mapping may be done for XR traffic considering the options under consideration by SA2 in TR 23.700-60 [2].

# Discussion

## Modeling requirements for RAN2 to evaluate

We understand that RAN2 should agree on the desirable target requirements to be met over the air for Rel-18 XR traffic. The following points summarizes the ones that seem to be considered by companies:

- Requirement (A) Different QoS handling for each type of PDU set

  - This requirement (A) might be very broad for RAN2 to do a complete evaluation of the foreseen impact in RAN/UE.

  - From SA2 side, this type of PDU set seems to be based on its priority or importance. SA2 also concluded in TR 23.700-60 [2] that this PDU set importance info. would be visible to RAN.

  - From RAN2 point of view, the key focus should be on how to provide the differentiated QoS handling for each type of PDU set over the air interface. Some RAN2 companies have also discussed inter-related requirements as part of RAN2#119bis documents and when providing their views during the drafting efforts of this TR 38.835[1]. These views are summarised below in which they are also mapped to the following concrete target requirements below:

    - Requirement (A.1) Different discard rules for each type of PDU set

      - From SA2 side, some solutions, for the Key Issues #4 & #5 on PDU Set based QoS framework, explain how the primary use of the PDU set importance is for preferential frame discarding under specific circumstances (e.g. congestion) as captured in TR 23.700-60 [2]; for example:

        - Sol. 24 explains that the “PDU Set Importance” could be determined based on the “I” and “D” bits in the RTP header extension which are used to identify “Independent” and “Discardable” frames.

        - Sol. 12 explains that the “PDU Set Importance” could be determined based on the NAS unit header in which the NRI field could be used for deriving the “PDU Set Importance”, such that, “NRI with the value of [0,3] indicates the relative transport priority of the RTP packet, 0 indicates the NAL unit can be discarded without risking the integrity of the reference pictures, while the RTP packets of an I frame should have the NRI value of 3”).

      - From RAN2 contributions, it also seems desirable to study mechanisms that could selectively discard specific PDU sets to be used under congestions scenarios. SA2 also concluded that “PDU Set Integrated Indication” can be used to indicate whether all PDUs are needed for the usage of PDU Set by application layer. We discuss some of these enhancements in [3].

For Rel-18 XR, new discard rules associated to the PDU set are proposed by companies in RAN2 and SA2.

    - Requirement (A.2) Different reliability rules for each type of PDU set

      - From SA2 side, SA2#153e XR email discussion captures companies’ views to ”Q2. How to deliver PDU Set importance information to RAN” in which the email disc. Rapporteur suggested as a potential way forward that SA2 should “Continue to discuss on the key concerns of each option, e.g. impacts on RAN scheduling for option 2”, where option 2) includes option 2.1) new sub-QoS flow concept or 2.2) new information in GTP-U header.

      - The solution associated to this requirement (A.2) is purely within RAN2 domain, i.e., out of SA2 scope, although SA2 should be aware if RAN2 targets to address it for Rel-18. Some RAN2 contributions explained that it is desirable to study mechanisms that could selectively use different levels of reliability or robustness required for specific PDU sets to be used under specific scenario, e.g., in [4]-[11].

For Rel-18 XR, new reliability rules (e.g., scheduling, prioritization, or robustness) associated to the PDU set are proposed by companies in RAN2 and SA2.

- Requirement (B) In-sequence delivery to upper layers of different types of PDU sets.

  - From SA2 side, SA2#153e XR email discussion captures companies’ views to ”Q2. How to deliver PDU Set importance information to RAN” in which the email disc. Rapporteur suggested as a potential way forward that SA2 should “Continue to discuss on the key concerns of each option, e.g. whether mis-ordering could happen for option 1)”, where option 1) refers to the usage of different QoS flows. It is explained that it may cause out-of-order problems as RAN may bind the QoS Flows with different PDU set importance to different DRBs.

  - In current NR operation, packets are normally provided in-sequence to upper layers, and out-of-order delivery can be configured to PDCP if upper layers can handle this (e.g. some protocols can handle the reordering like SCTP). For XR, some companies claimed that certain decoders might also be able to handle packets when they are not in order. However, it might be challenging to generalize this, i.e., whether all XR applications can handle packets out of order. To conclude on whether in-sequence delivery is not required for XR, SA2/SA4 input seems needed, i.e. SA2/S4 should be contacted to ask whether it is acceptable that AS layer cannot guarantee in-sequence delivery to upper layers for XR traffic. Meanwhile, we suggest for RAN2 to continue its study assuming Rel-18 XR might need to maintain the same NR principle, i.e., AS layer is capable to deliver of packets in-sequence to upper layers.

For Rel-18 XR, the concern of providing in-sequence delivery to upper layers for different types of PDUs set are raised by companies in RAN2 and SA2.

In summary, RAN2 should perform a complete evaluation considering all target/potential requirements.

- To agree on the following target/potential requirements to be fulfilled to enable Rel-18 XR awareness in RAN:

  - The differentiated QoS handling for each type of PDU (requirement (A)) should further be evaluated considering different discard rules for each type of PDU set (requirement (A.1)) and different reliability rules for each type of PDU set (requirement (A.2)).

  - New requirement needs to be evaluated for the in-sequence delivery to upper layers of different types of PDU sets (requirement (B))

    - RAN2 asks SA2 and SA4 whether it is acceptable that AS layer cannot guarantee that packets of one XR traffic stream that have different type of PDU sets are provided in sequence to upper layers (i.e., whether it is acceptable to always leave the handling of the re-ordering, when/if required, to upper layers, e.g. to the XR Application).

- If Proposal 1 is agreed, to confirm that Rel-18 XR solutions should meet these requirements (A.1), (A.2) and (B) captured in Proposal 1.1 and Proposal 1.2.

## Further analysis of the DRB mapping for XR traffic

Figure 1 depicts the 4 possible alternatives depending on the mapping of PDU sets onto QoS flows is done in the NAS layer and how QoS flows are mapped onto DRBs in the AS layer as it is captured TR 38.835 [1].

Figure 1.  Mapping Alternatives as captured in TR 38.835 [1]

Table 1 summarizes the evaluation explained in TR 38.835 [1], and Table 2 analyses the foreseen RAN/UE impacts if the new requirements explained in previous section are supported by any of the four alternatives under discussion in TR 38.835 [1] on how the DRB mapping may be done for XR traffic. A symbol is added to categorize the foreseen level of impact:  for none/minimum,  for medium, and  for large.

Table 1. Summary details of each mapping alternative  currently explained in TR 38.835 [1]

| Points to consider from RAN evaluation point of view | Alternatives |  |  |  |
| --- | --- | --- | --- | --- |
|  | 111 | NN1 | N11 | N1N |
| (CN model) Types of PDU set & QoS flow | 1:1 | N:N | N:1 | N:1 |
| (RAN model) QoS flow & DRB | 1:1 | N:1 | 1:1 | 1:N |
| (X) Is DRB/QoS flow mapping feasible with current spec.? | Yes | Yes | Yes | Not without changes (due to sub-QoS flow) |
| (A) Different QoS handling for each type of PDU sets [unclear if this includes (A.1), (A.2) from Table 2] | It is possible with current spec. | Each type of PDU sets in a DRB needs some way of differentiation | Each type of PDU sets in a DRB needs some way of differentiation | It is possible with current spec. |

Table 2. Impacts analysis from RAN side of each mapping alternative for the new identified requirements

| Points to consider from RAN evaluation point of view | Alternatives |  |  |  |
| --- | --- | --- | --- | --- |
|  | 111 | NN1 | N11 | N1N |
| (A.1) Different discard rules for each type of PDU sets | It is possible with current spec. (*) | To be defined based on the type of PDU set within a DRB level | To be defined based on the type of PDU set within a DRB level | It is possible with current spec. (*) |
| (A.2) Different reliability/robustness rules for each type of PDU sets | It is possible with current spec. (*) | To be defined based on the type of PDU set within a DRB level | To be defined based on the type of PDU set within a DRB level | It is possible with current spec. (*) |
| (B) In-sequence delivery to upper layers of different types of PDU sets | New re-ordering across different DRBs is required | It is possible with current spec. | It is possible with current spec. | New re-ordering across different DRBs is required |

(*) Some changes might be required if new solutions/enhancements are defined in Rel-18 XR on those areas

Current analysis explained in TR 38.835 [1] and summarized in Table 1 could lead to the conclusion that alternative 111 is the most preferable as it has no RAN impact while being able to provide different QoS for each type of PDU set. However, in our understanding, this analysis is incomplete as it has not considered all requirements associated to the AS modeling specific to XR traffic.

TR 38.835 does not consider all the requirements associated to the modelling in AS layer specific to XR traffic and is therefore incomplete and can lead to a misleading conclusion.

The analysis of the new/updated requirements shown in Table 2 explain how all alternatives have some level of impacts from AS layer point of view, for example:

- For alternatives NN1 and N11, a new mechanism needs to be defined in AS layer to differentiate packets of the same DRB that require different priority in order to provide them with the optimum and differentiated operation over the radio. This may be done, e.g., using different logical channels for the same DRB with different priority with additional mechanism to split packets after PDCP to the different logical channels based on the priority requirements.

- For alternatives 111 and N1N, a new re-ordering mechanism might be required across different DRBs, e.g., common sequence number (SN) required across packets of those different DRBs.

Alternatives NN1 and N11 (which use a single DRB to convey different types of PDU sets) require changes to legacy operation in order to allow distinguishing and to provide differentiated handling of the packets with different priority over the radio using a single DRB and which belong to the same XR traffic stream.

Alternatives 111 and N1N (which use different DRBs to convey each type of PDU sets) require changes to legacy operation in order to provide in-sequence delivery to upper layers (i.e. a reordering mechanism might be required across different DRBs) and potentially even to provide differentiated handling.

All mapping alternatives captured in TR 38.835 have some level of AS layer impacts for them to support all the requirements here discussed (A.1, A.2 and B).

In summary, we suggest for RAN2 to perform a complete evaluation of all the alternatives based on the requirements understanding that all options are feasible with certain level of impact.

- RAN2 updates the analysis of the 4 mapping alternatives to also evaluate the foreseen impacts for each potential/target XR specific requirement and uses Table 1 and Table 2 as baseline to be included/considered when updating section 5.1.2 of TR 38.835.

## XR awareness in UE side

For the XR awareness on UE side, RAN2 mainly conclude that UL should also be considered as part of Rel-18 XR study; e.g., RAN2 agreed the following:

- 	RAN2 assumes that PDU Set based parameters and PDU Set related information may be used for better support of XR services. RAN2 can consider both UL and DL directions.

- 	XR awareness discussion in RAN2 should consider PDU set characteristics and how to use the information available on those (for UL and/or DL). Can also consider how to handle data bursts.

- From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL:

  - Semi-static information (from CN to RAN): At least PSER and PSDB.

  - Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

- RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN.

SA2 captured in the conclusion section of Key Issues #4 & #5 in TR 23.700-60 [2] that UL handling/discussion is left to RAN WGs.

- “NOTE: Further PDU Set handling for Uplink will be studied and led by RAN WG. SA2 can align with RAN’s progress and decision for Uplink, if any.”

In our understanding, RAN2 can assume that information/mechanisms similar to the DL may be visible/available at UE (in order to also discuss potential UL enhancements. Considering TR 38.835, TR 23.700-60 and our analysis in [13], this information is summarized in the following two points:

- Semi-static kind of XR information (i.e. sent over control plane): PDU-Set Delay Budget (PSDB), PDU-Set Error Rate (PSER), Traffic parameters (e.g. burst periodicity), jitter information associated with each periodicity (e.g. range) and PDU set integrity indication and/or discard rules.

- Dynamic kind of XR information (i.e. sent over user plane): PDU set information (including identifier, start/end, size and PDU SN within the PDU set), end of a data burst and type of PDU set (e.g. PDU set importance).

After SA2/RAN2 work is more stable, RAN2 could also check with SA2/CT1/SA4 on whether they have any input on whether any specification impact is foreseen/required by upper layers to expose this kind of information to UE AS. Another alternative is that it is left up to UE implementation how a UE is aware of this “PDU set” related information.

For XR study, SA2 focuses on DL although RAN2 plans to consider both (DL and UL side).

In summary, RAN2 can initiate its study under these assumptions before knowing the exact PDU set related information that may be visible or known to RAN and UE AS.

- RAN2 confirms and informs SA2/SA4 that the XR related information (for PDU set and data burst) applicable to DL are also considered applicable to UL XR traffic (unless it is stated otherwise). Based on TR 38.835 and TR 23.700-60, this XR information is: semi-static kind of XR information (which can be sent over control plane and includes PDU-Set Delay Budget (PSDB), PDU-Set Error Rate (PSER), Traffic parameters (e.g. burst periodicity), jitter information associated with each periodicity (e.g. range) and PDU set integrity indication and/or discard rules), and dynamic kind of XR information (which can be sent over user plane and includes PDU set information (including identifier, start/end, size and PDU SN within the PDU set), end of a data burst and type of PDU set (e.g. PDU set importance)).

# Conclusion

The observations captured are the following:

Observation 1.	For Rel-18 XR, new discard rules associated to the PDU set are proposed by companies in RAN2 and SA2.

Observation 2.	For Rel-18 XR, new reliability rules (e.g., scheduling, prioritization, or robustness) associated to the PDU set are proposed by companies in RAN2 and SA2.

Observation 3.	For Rel-18 XR, the concern of providing in-sequence delivery to upper layers for different types of PDUs set are raised by companies in RAN2 and SA2.

Observation 4.	TR 38.835 does not consider all the requirements associated to the modelling in AS layer specific to XR traffic and is therefore incomplete and can lead to a misleading conclusion.

Observation 5.	Alternatives NN1 and N11 (which use a single DRB to convey different types of PDU sets) require changes to legacy operation in order to allow distinguishing and to provide differentiated handling of the packets with different priority over the radio using a single DRB and which belong to the same XR traffic stream.

Observation 6.	Alternatives 111 and N1N (which use different DRBs to convey each type of PDU sets) require changes to legacy operation in order to provide in-sequence delivery to upper layers (i.e. a reordering mechanism might be required across different DRBs) and potentially even to provide differentiated handling.

Observation 7.	All mapping alternatives captured in TR 38.835 have some level of AS layer impacts for them to support all the requirements here discussed (A.1, A.2 and B).

Observation 8.	For XR study, SA2 focuses on DL although RAN2 plans to consider both (DL and UL side).

The proposals captured are the following:

Modeling requirements for RAN2 to evaluate

Proposal 1.	To agree on the following target/potential requirements to be fulfilled to enable Rel-18 XR awareness in RAN:

Proposal 1.1.	The differentiated QoS handling for each type of PDU (requirement (A)) should further be evaluated considering different discard rules for each type of PDU set (requirement (A.1)) and different reliability rules for each type of PDU set (requirement (A.2)).

Proposal 1.2.	New requirement needs to be evaluated for the in-sequence delivery to upper layers of different types of PDU sets (requirement (B))

Proposal 1.2.1.	RAN2 asks SA2 and SA4 whether it is acceptable that AS layer cannot guarantee that packets of one XR traffic stream that have different type of PDU sets are provided in sequence to upper layers (i.e., whether it is acceptable to always leave the handling of the re-ordering, when/if required, to upper layers, e.g. to the XR Application).

Proposal 2.	If Proposal 1 is agreed, to confirm that Rel-18 XR solutions should meet these requirements (A.1), (A.2) and (B) captured in Proposal 1.1 and Proposal 1.2.

Further analysis of the DRB mapping for XR traffic

Proposal 3.	RAN2 updates the analysis of the 4 mapping alternatives to also evaluate the foreseen impacts for each potential/target XR specific requirement and uses Table 1 and Table 2 as baseline to be included/considered when updating section 5.1.2 of TR 38.835.

XR awareness in UE side

Proposal 4.	RAN2 confirms and informs SA2/SA4 that the XR related information (for PDU set and data burst) applicable to DL are also considered applicable to UL XR traffic (unless it is stated otherwise). Based on TR 38.835 and TR 23.700-60, this XR information is: semi-static kind of XR information (which can be sent over control plane and includes PDU-Set Delay Budget (PSDB), PDU-Set Error Rate (PSER), Traffic parameters (e.g. burst periodicity), jitter information associated with each periodicity (e.g. range) and PDU set integrity indication and/or discard rules), and dynamic kind of XR information (which can be sent over user plane and includes PDU set information (including identifier, start/end, size and PDU SN within the PDU set), end of a data burst and type of PDU set (e.g. PDU set importance)).

# Reference

- R2-2210814, Study on XR enhancements for NR, Rel-18 3GPP TR 38.835 V0.3.0, October 2022.

- TR 23.700-60 V1.2.0, Study on XR (Extended Reality) and media services, October 2022.

- R2-2211380, Packet discard for XR traffic, Intel Corporation, November 2022.

- R2-2211379, Enhancements to provide differentiated XR handling, Intel Corporation, November 2022.

- R2-2209414, On mapping PDU Sets for XR, Futurewei, November 2022.

- R2-2209644, PDU-set to DRB mapping for XR, ZTE Corporation, Sanechips, November 2022.

- R2-2209668, Discussion on QoS support with PDU Set granularity, Xiaomi Communications, November 2022.

- R2-2209468, Prioritization of XR traffic, CATT, November 2022.

- R2-2209778, Enhancements for Traffic Prioritization in XR, Apple, November 2022.

- R2-2210201, Handling of XR PDU sets in RAN, Huawei, HiSilicon, November 2022.

- R2-2210202, Discussion about XR-awareness impacts on LCP, Huawei, HiSilicon, November 2022.

- S2-2209979, LS on XR and Media Services, LSin, To RAN1, RAN2, RAN3, From SA2, October 2022.

- R2-2211382, Information in RAN for XR traffic and congestion, Intel Corporation, November 2022.