3GPP TSG RAN WG2 Meeting #120-e                                                   R2-2211380

# Toulouse, France, 14th-18th November 2022

Agenda item:	8.5.2.3

Source:	Intel Corporation

Title:	Packet discard for XR traffic

Document for:	Discussion

# Introduction

The following agreements were taken in RAN2#119bis-e meeting related to the topic of XR awareness information and packet discard for XR traffic [1]:

RAN2 119bis-e Agreements

- SDAP maps each data packet in a PDU set to a single PDCP SDU, as in legacy (i.e. each PDU is only mapped to a single SDU). - HARQ and RLC re-/transmissions for XR traffic are done as in legacy (i.e. they are not based on XR PDU sets). - For UE transmitter, the PDCP discard should be performed per PDU set basis. - For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set. - Wait for SA2/SA4 to clarify this (considering AL-FEC for XR packet discard). No need for LS at this point.

In this document, we discuss the topic of packet discard from RAN2 perspective, while considering the XR traffic characteristics and PDU set parameters, and also taking into account the progress made in the SA2/SA4 WGs thus far.

# Discussion

## Status Recap for SA2/SA4 and RAN2

### Status in RAN2

In the last RAN2 meeting #119bis-e, the topic of packet discard was discussed and several companies contributed to this topic. Companies brought forth proposals on discard criteria (e.g., delay based or dependency based), discard configuration (e.g., network configured or up to UE implementation), and other miscellaneous aspects, such as, discard relation/impact on ROHC context packets, on application layer FEC, or on L2 procedures (e.g. RLC AM window stalling, PDCP gap, HARQ retransmissions, etc.).

In the last RAN2 meeting 119bis-e, three important aspects related to discard in uplink were agreed upon (with reference included in previous section 1). Firstly, that the UE transmitter carries out the discard operation. Secondly, discard is performed at the PDCP layer (or by the PDCP entity for SDU level discard). And thirdly, discard is performed on a per PDU set basis. With respect to the point just mentioned (i.e. performing packet discard at PDU set level), a straightforward requirement could be to be able to identify PDUs that belong to a PDU set, which is also in line with the agreement made in RAN2 119bis-e that “Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed”.

- From RAN2#119bis agreements, it can be claimed that UE’s knowledge of the PDUs belonging to a PDU set allows that for UL traffic, the discard is performed by the UE transmitter on a per PDU-set basis.

With this RAN2 recap, we now move to discuss the progress made in the SA2/SA4 thus far which could be useful for RAN2 progress.

### Status in SA2/SA4

In the last SA2 meeting #153e, SA2 reached some conclusions on key issues 4 and 5, highlighting for this discussion the PDU set integrated packet handling and differentiated PDU set handling. The conclusions made in SA2 were for downlink communication, however, a note was added in the TR 23.700-60 that “PDU Set handling for Uplink will be studied and led by RAN WG”.

- PDU Set QoS parameters: It was agreed in principle to provide PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) while detailed definitions are still FFS, and PDU Set Integrated Indication for whether all PDUs are needed for the usage of PDU Set by application layer.

- PDU Set Information on User Plane: Parameters of PDU set Identifier, PDU Sequence Number (SN) within a PDU Set, and PDU set importance, among others, were agreed that they may be visible to RAN to support PDU set handling.

- QoS parameters of PSDB and PSER, along with information on PDU set identification and SN could be available in RAN via the user plane to support PDU set handling.

SA4 has updated the TR 26.926 for XR Release 18, from which we mention below some points that can be of relevance to RAN2 in this topic:

- “Slices” are considered lost beyond a maximum latency parameter. Receiver configuration includes loss detection through sequence numbers. Packets are dropped if late.

- Content delivery modelling for Application Layer FEC (AL-FEC) is for further study.

- SA4 considers latency as a criterion for packets to be dropped or considered lost as captured in the latest updates of TR 26.926.

Considering the status recap from RAN2 and SA2/SA4, we now discuss RAN2 related aspects for PDU discard.

## Discard Criteria

Based on the discussion thus far in RAN2 and SA2, following aspects could be considered as criteria for packet discard in uplink.

- Packets are discarded based on a maximum latency constraint. This maximum latency constraint, max_delay, could be based on the PSDB value. Since the gNB is aware of the PSDB value as a QoS parameter based on the above captured SA2 conclusion [TR 23.700-60], this max_delay value can be configured by the gNB per PDU set (e.g at the beginning of PDU set transmission), and is considered the maximum time duration before which the gNB must receive all packets in a PDU set considering all factors contributing towards end-to-end latency.  The UE can track the elapsed time duration and can discard packets if they are likely to not meet the maximum latency constraint of max_delay. If those PDUs of a given PDU set cannot meet that max_delay, UE transmitter could treat them as “unnecessary”.

- Packets are discarded based on the type of PDU set. For the case of XR traffic, PDU sets may carry different kind of content (e.g. I/B/P frames, or slices/tiles within an I/B/P frame etc.). An XR application may require correct decoding of the I-frame to enable the decoding of sub-sequent P-frames, but it may be able to sustain loss of some P or B frames.  In this case, under certain conditions, e.g congestion, the transmitter could discard PDU sets identified as lower priority/importance to consequently prioritize more important PDU sets e.g corresponding to an I-frame.

- Packets are discarded based on the QoS parameter of PDU Set Integrated Indication, such that if any PDU for a PDU set is lost/corrupted/delayed, for which is it indicated by the CN that all PDUs are needed for the usage of PDU Set by application layer, then remaining PDUs in that PDU set are considered unnecessary to be transmitted and can be discarded by UE transmitter. Here it is for the UE to determine/consider a packet as lost/corrupted/delayed for example, based on maximum latency requirement, or after it has exhausted HARQ retransmissions etc.

Note that above-mentioned criteria can be considered separately or in conjunction with one another, e.g criterion 3 which relates to the integrated handling of a PDU set may come in effect after certain PDUs are considered lost/unnecessary due to expiration of their delay budget, as explained in criterion 1. Figure 1 illustrates these three scenarios based on different discard criteria, where each discard criterion is considered independently for simplicity.

Figure 1: PDU discard under different discard criteria

The terms “lost” or “unnecessary” have both been used for discarded or discardable packets in this discussion. Our understanding is that any packet which meets one or multiple the discard criteria listed above is considered unnecessary and can be discarded e.g if delay budget requirement based on PSDB is violated, the PDUs for that PDU set are considered unnecessary before transmission, or may be considered lost if initial transmission is already attempted, or if one PDU of a PDU set is lost/corrupted during transmission over air, then the remaining PDUs of that PDU set are considered unnecessary by UE to transmit if integrated handling for that PDU Set is enabled.

- PDUs are considered unnecessary or lost if they violate the discard criteria, such as, the maximum delay budget requirement, if they are of low importance and discarded due to congestion, or if they belong to a PDU set for which integrated indication is flagged but other PDUs of that PDU set already result in failed transmissions (i.e. remaining PDUs would be considered unnecessary).

- Discard criteria considered by UE transmitter are: (1) a maximum latency constraint for a PDU set, (2) type/importance of PDU set, and/or (3) requirement for integrated handling for a PDU set.

## Types of Discard

From RAN2 perspective, data PDUs that are identified to be discarded, at the transmitter, could be in two stages of transmission. It is possible that the transmitter decides to discard certain PDUs (e.g unnecessary PDUs when some PDUs in the PDU set are lost), or certain PDU sets (e.g those which have low importance and are discarded in lieu of high importance PDU sets) before they are transmitted. It is also possible that the transmitter decides to discard certain PDUs after a transmission attempt has been made e.g when discard happens due to violating maximum latency constraint, even if part of the PDU set is already transmitted, the PDU set is considered to be lost/unnecessary as the latency requirement (based on PSDB) cannot be met.

- Data packets could be discarded at the transmitter side during the following stages: (1) before any transmission is attempted, and (2) after transmission attempt(s) have been made.

- Discard before transmission attempt:  In this case, data PDUs which are to be discarded have not been transmitted over the air interface, and therefore if they were dropped by the transmitter, their associated L2 (PDCP/RLC) SNs could be reused for new packets. Moreover for packets that have not been transmitted over the air, they could be discarded as PDCP SDUs. In this case, existing SDU discard mechanism could be reused as baseline.

- Existing SDU discard mechanism could be used as baseline, if PDUs of a given PDU set are discarded before they are initially transmitted.

- Discard after transmission attempt: In this case, such data PDUs may already have been transmitted over the air interface but not successfully received in which case, if they were dropped by the transmitter, the associated L2 SN cannot be reused for another packet by the transmitting entity. The point to note here is that even for packets or RLC PDUs for which delivery has already been attempted by MAC, they could be discarded without attempting retransmission if e.g delay timer expires. This is different from current specification, where only those RLC PDUs/SDUs can be discarded which have not yet been submitted to the lower layers [TS 38.322]. For high volume of XR packets to be transmitted with large number of packets possibly in the transmitter’s buffer, it could be beneficial to discard packets even if they have already been submitted to lower layers (MAC layer) for transmission. In this case, however, RAN2 needs to discuss how to support PDU discard, e.g at PDCP or RLC layer. Discard at this level however could result in gaps in the PDCP and/or RLC sequence numbers (SNs) since they cannot be reused as discussed later in section 2.4.1.

- Packet discard is allowed after packets are already submitted to lower layers (MAC layer).

## Potential impacts of PDU discard

### SN gaps for PDCP/RLC PDU level discard

An obvious implication of packet discarding in RAN2 for the case when such packets are already submitted to lower layers (MAC layer) is that discarding related/dependent PDCP/RLC PDUs could result in gaps in the PDCP/RLC SNs since the SN of the dropped PDCP/RLC PDU cannot be reused for new packets, as it was previously explained. RAN2 can therefore discuss whether/how this potential issue of PDCP/RLC SN gaps could be addressed e.g some feedback from the transmitter to the receiver (UE to gNB for UL or gNB to UE for DL) on the gap SNs to avoid reordering delays at the receiver side.

- If Proposal 3 is agreed, RAN2 to address PDCP/RLC SN gaps which may result from the discard of XR packets (e.g some feedback from the transmitter to the receiver on the gap SNs to avoid reordering delays at the receiver side).

### Impacts to BSR

The UE transmitter may decide to discard packets based on any or multiple discard criteria, as previously explained. A problem which may arise in the case of PDU discard is in relation to feedback and BSR from the UE side. In the event of autonomous packet discard by the UE, the network might have outdated BSR information. Given the usual high bit rate of XR traffic, and also expected large data volumes, it could be useful for the network to know of the reduction in UE’s buffered data volume when UL data is discarded from its buffers. This could be beneficial for resource allocation, such that if the network has not allocated UL grants yet, and it is informed of the reduced data volume, it can perform future scheduling and resource planning accordingly. Therefore when data packets are discarded, a BSR can be triggered to report to the network of the reduction in data volume in the UL buffer for the case when packets are discarded at the transmitting entity i.e UE in uplink. This could potentially avoid the gNB unnecessarily allocating UL resources. This will need minor updates of current BSR e.g., to trigger a BSR when packet discard is performed. Further details are provided on this in our companion paper [2].

- RAN2 considers UE feedback to the gNB (e.g BSR) due to XR packet discard from UE’s buffers.

# Conclusion

The observations and proposals are captured as follows

Current Status in RAN2 and SA2/SA4 WGs

Observation 1.	From RAN2#119bis agreements, it can be claimed that UE’s knowledge of the PDUs belonging to a PDU set allows that for UL traffic, the discard is performed by the UE transmitter on a per PDU-set basis.

Observation 2.	QoS parameters of PSDB and PSER, along with information on PDU set identification and SN could be available in RAN via the user plane to support PDU set handling.

Observation 3.	SA4 considers latency as a criterion for packets to be dropped or considered lost as captured in the latest updates of TR 26.926.

Discard Criteria

Observation 4.	PDUs are considered unnecessary or lost if they violate the discard criteria, such as, the maximum delay budget requirement, if they are of low importance and discarded due to congestion, or if they belong to a PDU set for which integrated indication is flagged but other PDUs of that PDU set already result in failed transmissions (i.e. remaining PDUs would be considered unnecessary).

Proposal 1.	Discard criteria considered by UE transmitter are: (1) a maximum latency constraint for a PDU set, (2) type/importance of PDU set, and/or (3) requirement for integrated handling for a PDU set.

Types of Discard

Proposal 2.	Existing SDU discard mechanism could be used as baseline, if PDUs of a given PDU set are discarded before they are initially transmitted.

Proposal 3.	Packet discard is allowed after packets are already submitted to lower layers (MAC layer).

Potential Impacts of PDU Discard

Proposal 4.	If Proposal 3 is agreed, RAN2 to address PDCP/RLC SN gaps which may result from the discard of XR packets  (e.g some feedback from the transmitter to the receiver on the gap SNs to avoid reordering delays at the receiver side).

Proposal 5.	RAN2 considers UE feedback to the gNB (e.g BSR) due to XR packet discard from UE’s buffers.

# References

- R2-22xxxxx, Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online, October 2022.

- R2-2211383 Enhancements to Buffer Status Reporting for XR Traffic, RAN2 120-e.

- TR 23.700-60 (v1.2.0), Study on XR (Extended Reality) and media services, SA2, Rel-18, October 2022.

- TR 26.926 (v1.2.0),  Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems, Rel-18, August 2022.