3GPP TSG RAN WG2 Meeting #120                                                   R2-2211383

# Toulouse, France, 14th-18th November 2022

Agenda item:	8.5.4.1

Source:	Intel Corporation

Title:	Enhancements to Buffer Status Reporting for XR Traffic

Document for:	Discussion

# Introduction

In the last RAN2 meeting #119bis-e, the topic of XR specific capacity improvements was discussed and the following relevant agreements were made [1].

- Introduce new BS table(s) to reduce the quantization errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). - Delay information consists of at least “remaining time”. - RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. - If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information.

This document discusses possible enhancements to buffer status reporting for XR traffic. Particularly, we discuss how BSR could be enhanced for XR traffic for the following cases: (1) QoS aware scheduling and (2) efficient resource allocation in the event of PDU discard.

# Discussion

## BSR Enhancements for QoS Aware Scheduling

### Definition of remaining time

Based on SA2 discussion and TR 23.700-60 [3], awareness of XR traffic characteristics which includes PDU and PDU set related information, such as PDU set delay budget (PSDB), PDU set integrated handling indication, PDU set importance, size of a PDU set, boundary indicators e.g first and last PDU of a PDU set etc. may be available in RAN for downlink traffic. It can be assumed that such information (or part of it) may also be available in uplink Error! Reference source not found.. Based on the delay budget requirement for PDUs or PDU set, the UE can be aware of the tolerable delay for data stored in the UL buffer and can assess the remaining time after which the data stored in the UL buffer may be unnecessary to transmit i.e. it will not meet the delay budget requirement. The UE can assess delay information on the remaining time before expiration of a maximum delay value which e.g. could be some pre-configured timer that is set based on PSDB value associated to any PDU set or for each PDU set.

- Remaining time is defined per PDU set as the tolerable delay after which the PDUs of a PDU set which are still stored in the UL buffer may be unnecessary to transmit i.e. they will not meet the delay budget requirement. The maximum value of this remaining time defined per PDU set is less than or equal to the corresponding PSDB.

- UE can determine remaining time before expiration of preconfigured timer based on the maximum delay tolerable by a PDU set (i.e based on PSDB value).

### Reporting of remaining time

If the UE and gNB are cognizant of PSDB value per PDU set, especially for delay-sensitive data in the UL buffer, the UE could enhance L2 procedures. Such delay information could be conveyed dynamically e.g BSR sent for each PDU set or per groups of PDU sets, e.g aggregate across all PDU sets of a logical channel group, or it could be semi-static with delay information shared periodically with the periodicity value related to XR traffic arrival rate etc, or the delay report could only be triggered if the remaining time for any given PDU set falls below a certain threshold criterion.

- Delay information can be reported dynamically per PDU set, semi-statically based on traffic arrival periodicity, or based on pre-configured threshold criterion.

It must be noted that at any given time there may be multiple PDU sets stored in the UL buffer, and each PDU set can have its own PSDB value. It was agreed in the last meeting that “If we have delay information, it needs to distinguish how much data is buffered for which delay value.” The UE is aware of the PSDB value associated with each PDU set, and it can be assumed that for UL transmission, the UE may have similar mechanism of PDU set identification as in downlink in TR 23.700-60. It was discussed in the last meeting’s online session, whether dynamic reporting of delay information may indeed be needed if both the UE and the gNB can be aware of the PSDB value since the PSDB value is a QoS parameter. The reporting of PSDB value in uplink is not required, however, the gNB may not have a guaranteed way of keeping track of all PDU-sets that the UE has started transmitting, and whether the remaining budget for data stored in the UL buffer is limited/expiring. It can be argued that the PDU set may be transmitted altogether, hence all PDUs in the PDU set face similar delay circumstances. However, based on the UL buffer and the size of the PDU set, it is also possible that the PDU set is transmitted sequentially but over different bursts (i.e. different occasions on time). Depending on whether the PDUs of a data burst arrive simultaneously in the AS or not, the gNB may not be able to reliably track the PDU set transmission progress (e.g how many PDUs and delay budget remains for any given PDU set).

- gNB cannot reliably keep track of remaining time especially if multiple PDU-sets are undergoing transmission or are stored in the UL buffer, or if a PDU set is transmitted over multiple data bursts.

Since the definition of remaining time for PDUs or data stored in the UL buffer will vary according to the PSDB value of their associated PDU sets, it is therefore appropriate to consider dynamic reporting of the delay information. However reporting delay information of remaining time per PDU set could result in significant overhead, since also considering that at any given time, multiple PDU sets may be stored in the UL buffer, RAN2 therefore needs to decide on an aggregation level over which individual delay values (or remaining time) per PDU set can be reported e.g over all PDUs/PDU-sets stored in the UL buffer, or per sub-group of PDU-sets in which case it could be defined as the worst-case or average value, that is the smallest or mean value of remaining delay budget among PDU-sets in that sub-group.

- Delay information based on the remaining time can be determined/reported by UE at some aggregation level e.g. per PDU set, or per logical channel group.

The buffer status report can be used to convey this aggregated delay information to the gNB. An appropriate aggregation level could be per logical channel group (LCG), such that the delay information is reported as the mean or worst-case value across multiple PDU sets of a given LCG.

- Delay information can be sent using a BSR.

### Impacts to L2 processing

Having up to date information on the remaining time for data in the UL buffer can have benefit for the gNB to perform resource scheduling and prioritizing accordingly when required e.g. for important/priority data or delay sensitive data. It could also be useful for the UE to decide whether the remaining packets of PDU set in the UL buffer are to be discarded upon exhausting HARQ retransmissions and considering certain packets as unsuccessfully delivered, or if the remaining time is insufficient for successful transmission, the UE may decide to skip UL grants if it is capable of doing so. Given the large data volume usually associated with XR transmission, the absence of user data could potentially be detected by the gNB (due to low received signal power on the skipped PUSCH), in which case it could become aware of a discarded-packet situation.

- Other L2 procedures could be optimized based on received delay information. For example, during resource scheduling by gNB or for UE to trigger the discard of packets unable to meet the remaining time requirement of a PDU set.

To reflect delay in BSR, a new BSR mapping table for delay values similar to Table 6.1.3.1-1 and Table 6.1.3.1-2 in TS 38.321 could be created. A new MAC CE for enhanced BSR could be defined to carry delay information for XR traffic, where this delay information is provided per logical channel group as the worst case value, that is the smallest value of remaining delay budget of data carried by any given logical channel in a particular logical channel group. Alternatively, this value could be defined as the mean or average value for the remaining time.

- A new MAC CE for enhanced BSR is defined to carry delay information for XR traffic, where this delay information is provided per logical channel group.

## BSR Enhancements for Resource Efficient Packet Discard

### Impact of Packet discard on resource allocation

The topic of PDU discard is discussed in our companion paper [2]. If some PDUs within a PDU set are lost/corrupted/delayed during UL transmission, the transmitting entity (i.e. the UE) may decide to discard remaining PDUs within the PDU set to avoid unnecessary transmission e.g if the PDU set integrated indication is flagged, or PDU sets could be discarded based on their type (high/low importance). This decision could be made by the transmitter (i.e. UE) for example, based on some delay budget related timer, or the type of PDU set. A problem in relation to feedback and BSR from the UE side, which could arise in the case of PDU discard, is that network might have outdated BSR information when UE discards packets without knowledge of the network. In some cases, the network may have allocated UL grants for packets that are discarded by UE before gNB has received an updated BSR. In other cases, the network might have not allocated a grant yet but it can still be useful to know about a reduction in UE’s buffered data volume e.g. for future scheduling, resource usage/planning. An example of such scenario is shown in Figure 1 below.

Figure 1: Updated BSR after PDU discard

- Network has outdated BSR information of a given UE when the UE transmitter discards packets autonomously i.e. without knowledge of the network.

- It is beneficial for the UE to report to the network about a reduction in the data volume of the UE’s buffer e.g. in the event of PDU discard for UL data which was previously reported in a BSR to the network.

### Enhancements to BSR

To avoid/minimize resource wastage in the event of packet discard, RAN2 can discuss methods of efficient buffer status reporting of XR traffic in uplink. A new BSR triggering condition can be introduced such that when UE performs the discard of packets, it sends an updated BSR to the gNB with the updated data volume in the buffer (after subtracting the discarded packets’ volume), i.e the UE reports a reduction in UL data volume via a BSR. Depending on the size of UL grant following a scheduling request, it is also possible that the UE sends few packets of the PDU set along with the BSR and learns of transmission failure for some (or all) of the transmitted PDUs. This could trigger the discard of all the remaining packets in the UL buffer that are related to that PDU set and it is also possible that there may not be data left in the UL buffer for that LCG.

In summary, when UE performs PDU discard, this can trigger the transmission of BSR or other form of indication that may convey UE’s feedback such as on the data status or amount of data in the UE side (e.g, a new MAC CE). It is also possible that some additional condition(s) might need to be met in connection with the discard of the data for the UE to trigger the BSR, e.g a threshold might apply to the discarded data, i.e. UE needs to discard at least certain amount of data before BSR can be triggered to report reduction in UL buffer volume.

- PDU discard at UE transmitter triggers a BSR to report reduction in UL buffer data volume.

  - Whether an additional threshold condition is needed on the amount of discarded data before which the UE can trigger a BSR to update on its reduction of data volume in UE’s buffers can be decided in the WI phase.

# Conclusion

The observations and proposals are captured as follows:

Definition of remaining time

Proposal 1.	Remaining time is defined per PDU set as the tolerable delay after which the PDUs of a PDU set which are still stored in the UL buffer may be unnecessary to transmit i.e. they will not meet the delay budget requirement. The maximum value of this remaining time defined per PDU set is less than or equal to the corresponding PSDB.

Proposal 2.	UE can determine remaining time before expiration of preconfigured timer based on the maximum delay tolerable by a PDU set (i.e based on PSDB value).

Reporting of remaining time

Observation 1.	Delay information can be reported dynamically per PDU set, semi-statically based on traffic arrival periodicity, or based on pre-configured threshold criterion.

Observation 2.	gNB cannot reliably keep track of remaining time especially if multiple PDU-sets are undergoing transmission or are stored in the UL buffer, or if a PDU set is transmitted over multiple data bursts.

Proposal 3.	Delay information based on the remaining time can be determined/reported by UE at some aggregation level e.g. per PDU set, or per logical channel group.

Proposal 4.	Delay information can be sent using a BSR.

Impacts to L2 processing

Observation 3.	Other L2 procedures could be optimized based on received delay information. For example, during resource scheduling by gNB or for UE to trigger the discard of packets unable to meet the remaining time requirement of a PDU set.

Proposal 5.	A new MAC CE for enhanced BSR is defined to carry delay information for XR traffic, where this delay information is provided per logical channel group.

Impact of Packet discard on resource allocation

Observation 4.	Network has outdated BSR information of a given UE when the UE transmitter discards packets autonomously i.e. without knowledge of the network.

Proposal 6.	It is beneficial for the UE to report to the network about a reduction in the data volume of the UE’s buffer e.g. in the event of PDU discard for UL data which was previously reported in a BSR to the network.

Enhancements to BSR

Proposal 7.	PDU discard at UE transmitter triggers a BSR to report reduction in UL buffer data volume.

Proposal 7.1.	Whether an additional threshold condition is needed on the amount of discarded data before which the UE can trigger a BSR to update on its reduction of data volume in UE’s buffers can be decided in the WI phase.

# References

- R2-22xxxxx, Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online, October 2022.

- R2-2211380 Packet discard optimizations for XR traffic, RAN2 120-e.

- TR 23.700-60 (v1.2.0), Study on XR (Extended Reality) and media services, SA2, Rel-18, October 2022.

- R2-2211382, Information in RAN for XR traffic and congestion, RAN2 120-e.