3GPP TSG RAN WG2 Meeting #120	R2-2211394

Toulouse, France, Nov. 14 – 18, 2022

Agenda Item:	8.5.4.1 (Feedback enhancements)

Source: 	Samsung

Title: 	Discussion on BSR enhancements for XR

Document for:	Discussion & decision

Introduction

Considering the agreements in previous RAN2 meeting [1], i.e.,

1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). Delay information consists of at least “remaining time”. 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

BSR enhancement is captured as one of the potential solutions for capacity improvements, and RAN2 can further discuss details on BSR enhancements based on the agreements from the last meeting.

Specifically, the BSR enhancements have been considered in terms of 1) introducing new BS table(s) to reduce the quantisation errors, and 2) considering delay information useful for XR. In this document, we discuss BSR enhancements in light of the aforementioned two directions.

Discussion

New BS Table

In the current spec, the buffer size field of BSR is used to report an index representing a numeric buffer size interval that covers the actual buffer size. Current BS tables, both for short and long BSRs, are designed by exponential functions, such that the BS interval becomes wider—the granularity becomes lower—as the index (and hence, the BS) increases.

Note that the motivation of introducing new BS table(s) arises from the fact that XR applications may entail large UL data bursts, e.g., AR with UL video traffic, such that the XR UL traffic amount may be reported by a BS index indicating a low granularity BS interval.

On the other hand, some other applications, e.g., web browsing, may not incur substantial UL data burst. It means that these applications may not necessarily require a newly designed BSR table(s), and can be served with legacy BS tables without noticeable performance degradation.

Observation 1: For the applications that may not generate large UL data burst, legacy BS tables can be used without noticeable performance degradation.

Intuitively, when introducing a new BS table, we may face extra BSR overhead caused by the enlarged buffer size fields if the new BS table contains more BS indices than legacy BS tables. Also, if we keep the number of indices but only cover part of the BS range of interest with the new BS table, the new BS table may not support the case when the buffer size reported is out of the BS range of interest. As a result, the new BS table should be used on-demand for the LCG(s) or LCH(s) that entails large UL data bursts.

Based on the discussion above, we would like to propose the following.

Proposal 1: RAN2 is kindly asked to consider that gNB may configure UE in terms of LCG(s) or LCH(s), for which the new BS table may be used.

Delay Information

One of the main characteristics of XR traffic is the tight delay requirement, which also applies to UL PDU sets. As indicated in [2], UL PDU set should be delivered from UE to gNB within namely 5G-AN PSDB. However, when a BSR is received, the gNB has no way to determine how urgently the buffered UL traffic should be scheduled. It may negatively affect system capacity, especially when the gNB has to decide how to allocate UL resources to multiple UEs for multiple PDU sets, given that the total UL resource requested by the UEs is more than that gNB can provide.

It can be considered that informing the gNB of PSDB can improve system capacity via delay-aware scheduling. However, unlike DL case, where gNB is aware of when a PDU set has arrived at gNB, and hence, can infer the level of urgency of the PDU set, in UL case, gNB has little knowledge on when a PDU set has been generated and arrived at UE when conducting UL scheduling. It means that the gNB may not infer the level of urgency with only PSDB for UL PDU sets, which may result in delay-agnostic UL scheduling.

Observation 2: With legacy BSR and possibly PSDB, gNB has little knowledge on how urgently the buffered UL PDU set(s) should be scheduled.

More specifically, gNB may try to meet the delay requirements of PDU sets for as many UEs as possible, to maximize system capacity, such that more urgent PDU set is served before less urgent one. It necessitates the premise that the gNB should be aware of the level of urgency of PDU sets, which can be expressed by how long the remaining delivery time of the PDU set is. It is not a constant or semi-static parameter, in that the remaining delivery time of a PDU set gradually decreases, and hence, the level of urgency of the PDU set gradually increases, as time goes on. Put differently, UE should report the most up-to-date remaining delivery time of PDU set to gNB to support better UL scheduling. Considering its dynamic characteristics, remaining delivery time of PDU set is better to be reported via MAC CE to enable timely reporting. We can consider either introducing the remaining delivery time of PDU set when discussing BSR format or newly define a MAC CE for that purpose. The remaining delivery time can be reported with respect to the most urgent PDU set for LCH(s) or for LCG(s).

Based on the discussion above, we would like to propose the following.

Proposal 2: RAN2 is kindly asked to consider reporting remaining delivery time of PDU set via BSR or new MAC CE, considering its dynamic characteristics.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: For the applications that may not generate large UL data burst, legacy BS tables can be used without noticeable performance degradation.

Proposal 1: RAN2 is kindly asked to consider that gNB may configure UE in terms of LCG(s) or LCH(s), for which the new BS table may be used.

Observation 2: With legacy BSR and possibly PSDB, gNB has little knowledge on how urgently the buffered UL PDU set(s) should be scheduled.

Proposal 2: RAN2 is kindly asked to consider reporting remaining delivery time of PDU set via BSR or new MAC CE, considering its dynamic characteristics.

References

- R2-2210802, Report on LTE legacy, DCCA, MUSIM, Slicing, 71 GHz, XR and QoE, Vice Chairman (Nokia).

- 3GPP TR 23.700-60 v1.2.0, Study on XR (Extended Reality) and media services.