3GPP TSG RAN WG2 Meeting #120	                                                       R2-2211403

# Toulouse, France, November 14th- 18th, 2022

Agenda item:		8.9.4

Source:			Intel Corporation

Title:		Path management for Multi-path Relaying

Document for:	 	Discussion

# Introduction

In the last meeting [RAN2#119bis-e], discussion on multipath study continued with emphasis on path addition/release and control plane aspects of the two scenarios. The set of agreements made in the meeting is available in annex. In this contribution we aim to discuss some of the open aspects related to path management for multi-path relaying.

# Discussion

## Path addition/modification/release concepts

### Change of Relay UE and cell change for Remote UE in scenario 1

Path management aspects for scenario 2 are still under discussion while they are more stable for scenario 1. While we agreed that the remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB, we need to decide whether this is achieved via separate release-and-add procedure or single switch procedure similar to indirect-indirect path switching. Considering the amount of signalling overhead involved in releasing PC5 connection (if upper layer also releases the connection), we think that it is best to support a single switch procedure.

- In scenario 1 for remote UE operating in multi-path, change of relay UE under the same gNB is handled via single switch procedure using indirect-indirect path switching with service continuity as baseline. Details considered during WI phase.

The support of cell handover for remote UE under the same gNB was discussed for both scenario 1 and 2 in the last RAN2 meeting 119bis-e. Supporting cell change for the direct path is in general, legacy operation. Considering scenario 1, it is a corner case that the remote UE has to perform inter-cell handover while the relay UE does not. Companies in RAN2 do not prefer to consider group handover situation, where the relay UE also performs handover along with the remote UE to a different cell. It should be possible to support this case using a single procedure of legacy cell change/handover procedure along with additional minimal gNB configuration considering that the Remote UE is in multi-path but some companies had concern about it. We think that we can agree to consider supporting this change using a single procedure unless a problem is identified during the WI phase.

Agreement:

The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

- In scenario 1 for remote UE operating in multi-path, the remote UE can change the direct path to a different cell of the same gNB via a single procedure unless a problem is identified. Details considered during WI phase.

### Addition/Release for scenario 2

In the last RAN3 meeting 117bis-e, RAN3 updated their previous agreement for scenario 2 as “Whether to add direct path, after the establishment of the indirect path is pending to RAN2 decision”, that is, RAN3 has left the decision of whether the addition of direct path is supported when indirect path is already established for scenario 2 multi-path up to RAN2. From the last RAN2 meeting 119bis-e, the following FFS was captured as highlighted below:

Proposal 1-2C: Whether to support the following case can be further discussed for Scenario 2.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.   The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

In scenario 2, initial access of Remote UE without gNB/default configuration through the UE-UE link via indirect path/Relay UE is not possible and direct path establishment is necessary to enable indirect path configuration. Therefore, we think that the remote UE configured only on the indirect path and adding direct path thereafter should not be supported.

- In scenario 2, indirect path cannot be configured until direct path is available.

  - The addition of direct path under the same gNB after configuration of indirect path (i.e. B in FFS Proposal 1-2C in [1]) is not considered feasible.

At the same time, even if we agree that SRB1 and SRB2 can be configured only on the direct path, then naturally, the direct path cannot be released in scenario 2 upon configuration of multipath.

- In scenario 2, for the Remote UE configured with multipath, releasing the direct path (i.e. D in FFS Proposal 1-2C in [1]) is not supported.

#### Change of Relay UE and cell change for Remote UE in scenario 2

When the Remote UE in scenario 2 is configured to support multi-path by the gNB, there was discussion about whether the serving cell of Remote UE can be changed while the serving Relay UE for the indirect path can be maintained. Depending on whether SRB1/SRB2 is allowed on indirect path as well as our consideration that the direct path is always available, we think that the serving cell of the Remote UE should not be changed with only indirect path remaining at any point during multipath.

- In scenario 2, for the Remote UE configured with multipath, changing the serving cell for the direct path while keeping the serving relay UE for the indirect path (i.e. E in FFS Proposal 1-2C in [1]) is not supported.

In scenario 2, for the case on whether the serving relay UE can be changed by the remote UE for the indirect path while keeping the direct path under the same gNB, we already made an agreement that the relation between remote UE and relay UE is pre-configured or static. Given this, we do not think that the change of relay UE when multipath is already enabled, is feasible/necessary. Since this link is out of 3GPP scope, we think change of the relay UE should also be out of 3GPP scope.

- In scenario 2, the remote UE configured with multipath changing to a new Relay UE for the indirect path while keeping the direct path under the same gNB (i.e. G in FFS Proposal 1-2C in [1]) is out of 3GPP scope and not supported.

# Conclusion

In this contribution we discuss path management for multipath relaying considering scenario 1 and scenario 2 and make the following observations and proposals:

- In scenario 1 for remote UE operating in multi-path, change of relay UE under the same gNB is handled via single switch procedure using indirect-indirect path switching with service continuity as baseline. Details considered during WI phase.

- In scenario 1 for remote UE operating in multi-path, the remote UE can change the direct path to a different cell of the same gNB via a single procedure unless a problem is identified. Details considered during WI phase.

- In scenario 2, indirect path cannot be configured until direct path is available.

  - The addition of direct path under the same gNB after configuration of indirect path (i.e. B in FFS Proposal 1-2C in [1]) is not considered feasible

- In scenario 2, for the Remote UE configured with multipath, releasing the direct path (i.e. D in FFS Proposal 1-2C in [1]) is not supported.

- In scenario 2, for the Remote UE configured with multipath, changing the serving cell for the direct path while keeping the serving relay UE for the indirect path (i.e. E in FFS Proposal 1-2C in [1]) is not supported.

- In scenario 2, the remote UE configured with multipath changing to a new Relay UE for the indirect path while keeping the direct path under the same gNB (i.e. G in FFS Proposal 1-2C in [1]) is out of 3GPP scope and not supported.

# References

[1] RAN2#119bis-e Chair notes

# Annex

Agreements:

Proposal 1-1A (modified): The following cases are to be supported for Scenario 1.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

C.	The remote UE operating in multi-path releases the indirect path;

D.	The remote UE operating in multi-path releases the direct path;

G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

Proposal 1-1B (modified): The following case is to be not supported for Scenario 1 as a group mobility scenario.

F.	The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

Agreement:

The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

Agreements:

Proposal 1-2A: The following cases are proposed to be supported for Scenario 2.

A.	The remote UE configured only on the direct path adds the indirect path under the same gNB;

C.	The remote UE configured with multi-path releases the indirect path;

Proposal 1-2B: The following case is proposed to be not supported for Scenario 2.

F.	The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

Proposal 1-2C: Whether to support the following case can be further discussed for Scenario 2.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.