3GPP TSG-RAN WG2 Meeting #120                                             　    　　　　　　 　　　　	R2-2211426

Toulouse, France, November, 2022

Source: 	KDDI Corporation

Title:	Considerations on XR jitter handling

Tdoc Type:	Discussion

Agenda Item:	8.5.3.1 DRX enhancements

Document for:	Discussion and decision

Release:	Rel-18

Introduction

RAN2 have been discussing jitter information which can be used for XR aware scheduling and power saving and made some progress with the following agreement [1][2]. In this contribution, we share our considerations on detailed definition of jitter information.

RAN2#119-e

- RAN2 to focus on the following issues for power saving, as well necessary parameters XR-awareness to support such enhancements, i.e.: - DRX enhancements to address the issues of DRX cycle mismatch and jitter; - Identify necessary parameters from CN for XR-awareness for power saving.

RAN2#119bis-e

- Capture in TR that traffic parameters and Jitter are semi-static info.

R2-2210814     TR 38.835 v030

Semi-static information provided by the CN: - The PDU-Set Delay Budget (PSDB); - The PDU-Set Error Rate (PSER); - Traffic parameters (e.g. periodicity); - Jitter information (e.g. range).

Discussion

SA2 discussion status

SA2 captures the jitter related study result as follows, and they assume three types of jitter [3]

- end-to-end jitter

- CN part transmission jitter

- air interface transmission jitter

- 7.1 Evaluation for Key Issue #7 There are four solutions proposed to address the Key Issue #7 i.e. about policy enhancements for jitter minimization in clause 5.7: Sol#9, Sol#30, Sol#31 and Sol#32. Sol#9 proposes a user plane solution by adding new GTP-u header with sequence number of PDU set. The proposed PDU Set Delay budget is more related to delay budget. In this solution, there are no jitter specific mechanism introduced, thus this solution need not be evaluated under KI#7 but under KI#4. The following list shows some principles of solutions #30, #31, 32: - Maximizing the reuse of 5GS mechanisms with extensions, when needed, to assist the AF to request of QoS monitoring and report of jitter to AF: - Solution #30 propose that in order to determine whether jitter minimization is needed or not, the 5GS needs to expose its jitter performance to AF in order to let the AF decide whether further minimization is needed or not. - Solution#31 This solution proposes that the 5GS have a knowledge of the end-to-end jitter and as well as the air interface transmission jitter and CN part transmission jitter, which can help PCF generate some policy to minimize the jitter in different part of 5GS. - Solution#32 also share same architectural framework that AF needs to understand the jitter of 5GS. - Maximizing the reuse of 5GS mechanisms with extensions, when needed, allow AF and 5GC to interaction i.e. support AF to provision jitter requirements between AF and 5GC: - Both solution#31 and solution#30 propose AF to provision jitter requirements to 5GC NF i.e. NEF/PCF. Solution#31 proposed that PCF calculate the jitter value for RAN part and the part between NG-RAN and PSA UPF and decide what kind of policy enhancement can be implemented to reduce the jitter. - Solution#32 provides new/updated Periodicity and Periodicity Jitter with the XRM stream to the NEF/PCF. Regarding to jitter derivation and usage of jitter, solution#30 and solution#31 considers packet level parameter and solution#32 considers PDU set level parameter. In split of this, the QoS framework on requesting and reporting of jitter, as well as AF provisioning of jitter requirements to 5GC are common.

We think that those three jitters captured in SA2 should be also captured in RAN side Technical Report. But at the same time, we don’t think it’s feasible to have further conclusions on detailed definition of those jitters in this WI phase.

Proposal 1:

RAN2 agree to capture the SA2 defined jitter parameters, end-to-end jitter, CN part transmission jitter, air interface transmission jitter.

Proposal 2:

RAN2 agree to discuss detailed definition of those SA defined jitters in the WI phase with the coordination with SA2 and SA5.

Detailed jitter information

RFC3550 as a reference

RFC 3505 has a jitter relevant definition “relative transit time“ and “interarrival jitter” as follows [4]. Even though the XR application data may be difference from it to some extent but we expect that those definition can be used as a reference for developing 3gpp XR jitter definition.

interarrival jitter: 32 bits An estimate of the statistical variance of the RTP data packet interarrival time, measured in timestamp units and expressed as an unsigned integer. The interarrival jitter J is defined to be the mean deviation (smoothed absolute value) of the difference D in packet spacing at the receiver compared to the sender for a pair of packets. As shown in the equation below, this is equivalent to the difference in the "relative transit time" for the two packets; the relative transit time is the difference between a packet's RTP timestamp and the receiver's clock at the time of arrival, measured in the same units. If Si is the RTP timestamp from packet i, and Ri is the time of arrival in RTP timestamp units for packet i, then for two packets i and j, D may be expressed as D(i,j) = (Rj - Ri) - (Sj - Si) = (Rj - Sj) - (Ri - Si) The interarrival jitter SHOULD be calculated continuously as each data packet i is received from source SSRC_n, using this difference D for that packet and the previous packet i-1 in order of arrival (not necessarily in sequence), according to the formula J(i) = J(i-1) + (¦D(i-1,i)¦ - J(i-1))/16 Whenever a reception report is issued, the current value of J is sampled. The jitter calculation MUST conform to the formula specified here in order to allow profile-independent monitors to make valid interpretations of reports coming from different implementations. This algorithm is the optimal first-order estimator and the gain parameter 1/16 gives a good noise reduction ratio while maintaining a reasonable rate of convergence [22]. A sample implementation is shown in Appendix A.8. See Section 6.4.4 for a discussion of the effects of varying packet duration and delay before transmission.

Downlink jitters/Uplink jitters

- End-to-end jitter(relative transit time)

This metrics is relative transit time measured at Application layer of AF in network side and application resided in UE side. For downlink transmission case, if AF in network side transmit packet i at SAF(i) and packet j at SAF(j) accordingly and application in UE side receive packet i at RApp(i) and packet j at RApp(j), then the relative transit time is considered as DAF-App(i,j) = (RApp(j) - RApp(i)) - (SAF(j) - SAF(i)) = (RApp(j) - SAF(j)) - (RApp(i) - SAF(i)). For uplink transmission, RApp(i) and RApp(j) is packet arrival time at network side,  SAF(i) and SAF(j) is packet transmit time at UE side. The relative transit time is calculated with the same way as downlink transmission case.

- CN part transmission jitter(relative transit time)

This metrics is relative transit time measured at Application layer of AF in network side and PDCP of gNB, the boundary between gNB and UPF. For downlink transmission case, if AF in network side transmit packet i at SAF(i) and packet j at SAF(j) accordingly and PDCP in gNB receive packet i at RgNB(i) and packet j at RgNB(j), then the relative transit time is considered as DAF-gNB(i,j) = (RgNB(j) – RgNB(i)) - (SAF(j) - SAF(i)) = (RgNB(j) - SAF(j)) - (RgNB(i) - SAF(i)). For uplink transmission, RAF(i) and RAF(j) is packet arrival time at network side,  SgNB(i) and SgNB(j) is packet transmit time at UE side. The relative transit time is calculated with the same way as downlink transmission case.

- Air interface transmission jitter(relative transit time)

This metrics is relative transit time measured at air interface, the boundary between gNB and UE. For downlink transmission case, if gNB transmit packet i at SgNB(i) and packet j at SgNB(j) accordingly and UE receive packet i at RUE(i) and packet j at RUE(j), then the relative transit time is considered as DgNB-UE(i,j) = (RUE(j) – RUE(i)) - (SgNB(j) - SgNB(i)) = (RUE(j) – SgNB(j)) - (RUE(i) – SgNB(i)). For uplink transmission, RgNB(i) and RgNB(j) is packet arrival time at network side,  SUE(i) and SUE(j) is packet transmit time at UE side. The relative transit time is calculated with the same way as downlink transmission case.

Figure 1 Downlink relative transit time

Figure 2 Uplink relative transit time

Jitter metrics

CN or OAM or other nodes collect considerable number of samples of relative transit time D and generate jitter information such as range, average, minimum value and maximum value.

Proposal 3:

RAN2 agree to capture the above consideration on jitter detailed jitter information

Static vs Dynamic

In RAN2#119bis-e, jitter information was agreed to be captured as a semi-static information in TR. Here, we share additional consideration on semi-static jitter info in terms of how those information is organized and indicated to gNB.

Semi-Static

OAM may collect the performance measurements data including packet delay [5], and those collected packet delay measurements can be used for organizing the detailed jitter information discussed in the previous section2.2. In this case, OAM or other CN nodes can figure out jitter information such as relative transit time D, min D, max D, average D specifically form the collocated packet delay measurements data. After that, OAM or other CN nodes can provide those jitter information to gNB when PDU session being established. gNB and UE can keep using those static jitter information until the PDU session ends.

We also think that in some cases where jitters can fluctuate dynamically, dynamic jitter information is also beneficial, and provide our views as below.

Dynamic

Because of the RAN load status and other reasons, jitters can fluctuate dynamically. For those cases, dynamic jitter information may be beneficial. In this case, gNB or UE monitor packet delay and calculate jitter information continuously and periodically. One possible solution for this continuous monitoring and calculation would be according to the formula

J(i) = J(i-1) + (|D(i-1,i)| - J(i-1))/K, K is a scaling factor

Whenever gNB or UE get new sample data of relative transit time D, then the value J is updated. Detailed calculation mechanism can be discussed in the WI phase.

Proposal 4:

RAN2 agree to capture the static and dynamic jitter information discussed above

Text proposal for TR38.835

Proposal 5:

RAN2 discuss and finalize the jitter related statements in the TR using the following TP as a baseline.

- 5.1.1 General In both uplink and downlink, XR-Awareness contributes to optimizations of gNB radio resource scheduling and relies at least on the notions of PDU set and Data Burst (see TR 23.700-60 [9]): a PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice), while a Data Burst is a set of data PDUs generated and sent by the application in a short period of time. NOTE: A Data Burst can be composed of multiple PDUs belonging to one or multiple PDU Sets. In order to handle PDUs efficiently in both UL and DL, the following information would be useful: Semi-static information provided by the CN: - The PDU-Set Delay Budget (PSDB); - The PDU-Set Error Rate (PSER); - Traffic parameters (e.g. periodicity); - Jitter information (e.g. range). Dynamic information: - The PDUs belonging to a PDU set (this includes the means to determine at least the PDU set boundaries); - The PDUs belonging to a Data Burst. When a certain number of PDUs of a PDU set are known to be required by the application layer to use the corresponding unit of information (for instance due to the absence or limitations of error concealment techniques, see TR 26.926 [6]), as soon as the number of PDUs known to be lost exceeds this number, the remaining PDUs of that PDU set are no longer needed by the application and may be subject to discard operation (see subclause 5.3.2). NOTE 1: This depends on the application and it cannot always be assumed that the remaining PDUs and/or dependent PDU sets are not useful and can safely be discarded. NOTE 2: In case of Forward Error Coding (FEC), active discarding of PDUs when assuming that a large enough number of packets have already been transmitted for FEC to recover without the remaining PDUs is not recommended as it might trigger an increase of FEC packets (see S4aV220921 [14]). CN have a knowledge of the end-to-end jitter, air interface transmission jitter and CN part transmission jitter. CN may provide those information to gNB, and gNB can use those jitter information for XR aware radio resource scheduling and/or DRX optimization aims to achieve power saving. Jitter information may include metrics (range, average, min, max) of relative transit time at certain layer/function. Jitter information may be generated at CN using past OAM performance measurements including packet delay, and those semi-static jitter information is provided to gNB when PDU session being established, and gNB can keep using those semi-static jitter information until the PDU session ends. In some cases where jitters can fluctuate dynamically due to RAN load status and other reasons, dynamic jitter information may be also beneficial. In those cases, gNB or UE may monitor jitter performance and generate jitter information continuously and periodically during PDU session is maintained.

Conclusion

Proposal 1:

RAN2 agree to capture the SA2 defined jitter parameters, end-to-end jitter, CN part transmission jitter, air interface transmission jitter.

Proposal 2:

RAN2 agree to discuss detailed definition of those SA defined jitters in the WI phase with the coordination with SA2 and SA5.

Proposal 3:

RAN2 agree to capture the above consideration on jitter detailed jitter information

Proposal 4:

RAN2 agree to capture the static and dynamic jitter information discussed above

Proposal 5:

RAN2 discuss and finalize the jitter related statements in the TR using the following TP as a baseline.

Reference

[1] R2-2210802	Report from session on LTE legacy, 71 GHz, DCCA, Multi-SIM, RAN slicing, QoE and XR Vice Chairman (Nokia)	Report

[2] R2-2210814    TR 38.835 v030    Nokia (Rapporteur)        draft TR            Rel-18   38.835  0.3.0     FS_NR_XR_enh

[3] 3GPP TR 23.700-60 V1.2.0 (2022-10) Study on XR (Extended Reality) and media services

(Release 18)

[4] RFC3550 RTP: A Transport Protocol for Real-Time Applications https://www.rfc-editor.org/rfc/rfc3550

[5] 3GPP TS 28.552 V18.0.0 (2022-09)	Management and orchestration; 5G performance measurements