3GPP TSG-RAN WG2 Meeting #120                                                        R2-2211442

Toulouse, France, 14th – 18th Nov, 2022

Source:              CATT

Title:	             Further consideration on XR-specific capacity improvement

Agenda Item:	8.5.4.2

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e, RAN2 reached below agreements on CG enhancement:

1: Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting. 2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

And in RAN1#110bis-e, RAN1 agreed:

Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration Conclusion No further discussion in RAN1 for Rel-18 XR to extend the support of legacy single DCI scheduling multi-PDSCHs for FR2-2, to other SCS in FR1/FR2-1. Conclusion The capacity gain performance results in R1-2208661, R1-2209658 and R1-2209198 corresponding to enhancements based on multi-PDSCH scheduling by a single DCI are captured in XR SI TR Conclusion Study on enhancement for CBG based HARQ-ACK feedback reporting is down-priorotized in RAN1 XR SI. Conclusion The following proposed enhancements techniques to improve XR capacity performance are down-priorotized in RAN1 XR SI: - (P3-5-3) Study on PHR enhancement based on XR traffic arrival periodicity or UL pose periodicity. - (P3-5-4) Study mechanism of packet dropping based on the PDB requirement, to avoid resource waste due to the out-of-date packets. Agreement - For further study the mechanisms to enable HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission for CA operation on TDD cells, consider at least the following: - Capacity performance evaluation results - Complexity analysis and RAN2 impact Conclusion - Study of soft HARQ-ACK and Delta MCS in RAN1 XR SI for improving XR capacity is down-priortized. - Note: The corresponding capacity gain performance results in R1-2210003, R1-2208377 and R1-2203607 are captured in XR SI TR. Conclusion - Study on enhanced CQI based on CBG transmission, and study on enhanced CQI based on DMRS for improving XR capacity are down-priortized in RAN1 XR SI. - Note: The corresponding capacity gain performance results in R1-2208402 and R1-2209536 are captured in XR SI TR. Conclusion Study on Cooperative MIMO via DL interference probing based on SRS enhancement for improving XR capacity is down prioritized in RAN1 XR SI. Note: The corresponding capacity gain performance results in R1-2208377 are captured in XR SI TR. Conclusion No consensus to continue study on differentiation of XR multiple flows based on CG enhancement in RAN1 XR SI. Conclusion No consensus to continue study of multi-bits SR mechanisms for capacity improvement of XR traffic in RAN1 XR SI.

According to above agreements, XR-specific CG enhancements needs to be discussed. And we consider XR-specific capacity improvement in DL further.

Discussion

2.1 CG enhancement

The various streams (video, audio, pose/control) that form the XR traffic are all periodic [1], so CG should be considered naturally. However, because of the characteristics of jitter and data variation of the video stream, legacy CG has lower capacity than DG based on RAN1 evaluation [2]. Note that in the comparison, UE is always on for PDCCH monitoring when DG is applied. And for the dynamic UL scheduler to always provide the right UL grant size at the right time, matching the PDU set arrival time and size, it would require the UE to send an SR/BSR for each PDU set, thus adding complexity, power consumption and latency. CG has at least 2 advantages: UE power saving and low-latency transmission for the periodic UL services. So, since the power saving requirement is at least as important as the capacity requirement for XR, CG should be adopted with some enhancements.

Observation 1: Unlike dynamic grants, configured grants are best suited to serve the UL XR traffic, while providing the best latency and power saving performance.

Although RAN2 agreed “Current CG configurations can be reused for UL XR traffic”, obviously current CG periodicities are not appropriate to the non-integer periodicities of XR traffic such as 30/60/90/120 fps. In current specification, the CG periodicity can be “periodicityExt-r17 INTEGER (1..40960)” [symbols]. The value range of CG periodicity is flexible enough for services with integer periodicity. The question is the periodicities of XR video streams are non-integer and the deviation between the time occasion of CG and arrival time of service data will increase with time lapse. Taking the service periodicity 16.66ms as an example, if the CG periodicity is configured as 16ms starting from SFN0, the time occasion of CG resources will be 0, 16, 32, 48, 64, 80, 96…[ms]; and the service data will arrive at 0, 16.66, 33.33, 50, 66.66, 83.33, 100…[ms]. It will exceed the end-to-end latency requirement which can be 5ms for cloud gaming and then the CG resource allocation will be useless.

To match the non-integer XR service periodicity, the simplest way is to introduce non-integer CG periodicity and UE will calculate the time occasion of CG as INT (periodicity *N). Considering both cross-slot periodicity and SFN wrap-around issues have already been addressed since R16, such approach should be straightforward.

Proposal 1: Non-integer CG periodicity should be introduced and UE will calculate the time occasion of CG as INT (periodicity*N).

Similar to DL XR-specific traffic, the UL XR-specific traffic has the characteristic of periodical data generation and variable packet size. The UL configured grant has a fixed amount of resource allocation at each periodicity. The reserved resource allocation could be based on average packet size or minimum packet size of XR service with consideration of the system resource utilization. The larger the resource is allocated for CG, the less number of CG users is supported. So it is challenging to use CG for NR without enhancement to support XR services with the high data rate, low latency and high reliability.

There are two CG solutions for XR services: multiple CG configurations and multi-CGO in a period. The comparison is shown in Table 1.

Table 1 The comparison between multiple CG configurations and multi-CGO in a period

|  | Multiple CG configurations | Multi-CGO in a period |
| --- | --- | --- |
| Explanation | Multiple CG configurations are configured for a XR service with jitter and variable packet size. And multiple CG configurations should be activated to transmit a PDU set which cannot be accommodated in one CG resource. | A CG configuration in which multi-CGO is allocated in each period is configured for a XR service. One CG is activated to transmit a PDU set. |
| Specification impact | Existing: The mechanism of multiple CG configurations is supported now. Specification change: - For the large-scope packet size variation and support of multiple XR services, the number of CG configurations should be extended. - One DCI activating multiple CG configurations should be considered. | Existing: Multi-CGO in a period is supported in unlicensed band now. Specification change: - Multi-CGO is applied to licensed band. - The time-based HARQ process ID determination for the multi-CGO should be considered. |
| How to avoid wasting of the unused CG PUSCH occasion(s) | PDU set level BSR [3] makes the gNB be aware of the unused CG PUSCH occasions. |  |

As analysed above, to cope with the XR characteristics, any CG mechanism needs to be enhanced. Considering this is the last meeting in SI stage, we propose capturing both multiple CG configurations and multi-CGO in TR38.835 and further discuss them in WI stage.

Proposal 2: RAN2 considers both multiple CG configurations and multi-CGO in the TR and further discuss them in WI stage.

2.2 The dynamic PDSCH allocation introduced in a SPS period

RAN2 agreed in RAN2#119 that “RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR”. This agreement is aimed to SPS mechanism itself, such as periodicity, multiple-PDSCH, etc. Other DL capacity improvement solution related to but not restricted to SPS could be considered.

The C-DRX can be used for UE power saving while serving the UE with robust data. Both C-DRX and SPS can be used for UE power saving. The SPS provides semi-persistent resource allocation, allowing the UE to receive PDSCH without any scheduling assignment and regardless DRX ON or OFF. The SPS PDSCH and DRX can be configured together for UE with periodic services, such as XR service. However, the current combination of C-DRX and SPS would not be appropriate for the XR services due to the jitter and variable burst size, specifically for the XR video traffic. Some enhancement is needed.

Observation 2: As in legacy, the combination of C-DRX and SPS can be used for UE power saving, but some enhancement is needed for XR services.

During DRX ON period, UE should monitor PDCCH for dynamic scheduling. During DRX OFF period, UE should receive SPS resource allocation without PDCCH monitoring. The enhancement of SPS is to introduce a dynamic downlink assignment embedded in an SPS PDSCH when an XR burst is larger than the pre-allocated SPS resource or when the XR burst experiences delay jitter. In both cases, an additional PDSCH resource is needed, and a DL assignment MAC CE transmitted in an SPS occasion is proposed, to handle this without PDCCH monitoring.

As shown in Figure 1,

- Case 1: XR burst can be accommodated in the SPS resource appropriately. This is the ideal case as other robust periodic services, such as VoIP.

- Case 2: To cope with jitter, gNB should allocate DG in SPS occasion via DL assignment MAC CE. Otherwise, the XR burst can be transmitted only when the next DRX cycle is started which will decrease the DL transmission performance.

- Case 3: When the size of arrived XR burst is larger than the SPS resource, gNB can allocate subsequent DL resource via DL assignment MAC CE easily. So the XR burst can be transmitted within PDSB especially for the time sensitive XR service.

Figure 1: Complementing an SPS resource with additional dynamic assignment embedded in a MAC CE transmitted in the SPS resource

Proposal 3: RAN2 considers DL assignment MAC CE to improve the capacity while minimizing the UE power consumption.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Observation 1: Unlike dynamic grants, configured grants are best suited to serve the UL XR traffic, while providing the best latency and power saving performance.

Proposal 1: Non-integer CG periodicity should be introduced and UE will calculate the time occasion of CG as INT (periodicity*N).

Proposal 2: RAN2 considers both multiple CG configurations and multi-CGO in the TR and further discuss them in WI stage.

Observation 2: As in legacy, the combination of C-DRX and SPS can be used for UE power saving, but some enhancement is needed for XR services.

Proposal 3: RAN2 considers DL assignment MAC CE to improve the capacity while minimizing the UE power consumption.

The corresponding TP is attached in the Annex.

References

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR, V17.0.0

- R1-2208953, NR enhancement for XR capacity improvement, CATT

- R2-2211441, Further consideration on BSR, CATT

- R2-2210814, TR 38.835, v030

Annex: TP to TR 38.835 (R2-2210814)

5.3		Capacity Improvements Techniques

5.3.1	Physical Layer Enhancements

5.3.2	Layer 2 Enhancements

In order to enhance the scheduling of uplink resources for XR, the following improvements are envisioned:

-	New BS table(s) to reduce the quantisation errors in BSR reporting (e.g. for high bit rates);

-	Delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay.

Editor's Note: FFS if dynamic reporting of the uplink delay (e.g. via BSR) is needed, or whether PSDB is sufficient.

Furthermore, when, for a PDU set, the number of PDUs known to either be lost or associated to discarded SDUs, exceeds a threshold (see subclause 5.1.1), all remaining PDUs of that PDU set could be discarded at the transmitter to free up radio resources.

NOTE: this means that the granularity of the discard operation at PDCP in the transmitter should be the PDU set.

Editor's Note: the discard operation could also be captured in 5.2.2.

To leverage XR-specific capacity improvement while saving power, enhancements on SPS and CG are envisioned:

- For SPS, DL assignment MAC CE in SPS occasion (especially in DRX inactive period) is introduced to guarantee low latency transmission of the variable sized XR data,

- For CG,

  - Non-integer CG periodicity can be introduced to align the CG occasion with XR data arrival time. UE calculates the time occasion of CG as INT (periodicity*N).

  - Both multiple CG configurations and multi-CGO in a period can be considered to cope with the delay jitter and data variation. The enhancements on Rel-17 mechanisms can be studied in stage3.