3GPP TSG RAN WG2 Meeting #120          	      	                	               R2- 2211455

Toulouse, FR, 14th – 18th Nov, 2022

Agenda Item:	8.16.2

Source:	Intel Corporation

Title:	General aspects of AI/ML air interface and RAN2 impact

Document for:	Discussion and Decision

# Introduction

In RAN2 #119bis-e meeting, following agreements on general aspects of AI/ML air interface RAN2 impact were agreed:

#### General Assume that R2 will reuse terminology defined by R1 to the extent possible/reasonable Observation: the collaboration levels definitions doesn’t really clarify what is required, more work is needed #### Model transfer and format R2 assumes that for the existing (under discussion) AI/ML use cases, proprietary models may be supported and/or open format may be supported (and maybe RAN2 doesn’t have to further elaborate on this assumption). R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS. R2 assumes that a model is identified by a model ID. Its usage is FFS. General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

In this contribution, we continue discuss RAN2 impact of different collaboration level.

# Discussion

During RAN1 #109e meeting, following three network-UE collaboration levels are agreed to be studied:

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

Before discussing general framework of supporting AI/ML over air interface, it is important to understand where and how the AI/ML model is deployed in the system of different network/UE collaboration level, which highly impacts the analysis of specification impact.

RAN1 further agreed with following terminology during RAN1 #109e meeting:

UE-side (AI/ML) model; An AI/ML Model whose inference is performed entirely at the UE. Network-side (AI/ML) model; An AI/ML Model whose inference is performed entirely at the network. One-sided (AI/ML) model; A UE-side (AI/ML) model or a Network-side (AI/ML) model. Two-sided (AI/ML) model; A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa..

Based on above terminologies and agreements, the network-UE collaboration levels and model deployment options can be summarized as below:

Option 1: Level x + one-sided model (either network-side model or UE-side model)

Option 2: Level y + one-sided model (either network-side model or UE-side model)

Option 3: Level y + two-sided model

Option 4: Level z + two-sided model

Proposal 1: RAN2 assume following network-UE collaboration levels and deployment options for further study on AI/ML air interface:

1) Option 1: Level x + one-sided model (either network-side model or UE-side model)

2) Option 2: Level y + one-sided model (either network-side model or UE-side model)

3) Option 3: Level y + two-sided model

4) Option 4: Level z + two-sided model

## Level x RAN2 Impact

During RAN1 #110bis-e meeting, following agreements on level x and data collection were agreed:

Agreement Clarify Level x/y boundary as: - Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Based on the above agreement, level x may have specification impact which is unrelated to AI/ML collaboration. Based on our understanding, for level x, even though the AI/ML operation is implementation-based, data collection for the purpose of model training and model inference may still need to be supported. However, whether and what information is collected for model training/inference as part of data collection needs to be studied case by case. RAN2 should wait and see if there’s any specification impact of level x after RAN1 finish use case study.

Observation 1: Data collection for model training and model inference for level x implementation-based AI/ML operation may be needed.

Proposal 2: RAN2 should wait for RAN1 progress on data collection to support level x for each use case before concluding RAN2 impact of level x.

## Level y and Level z Common RAN2 impact

In AI/ML life cycle management, following aspects/components are considered:

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- Model registration

- Model deployment

- Model configuration

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

- Model monitoring

- Model update

- Model transfer

- UE capability

As agreed in RAN1 #110bis-e meeting:

Working Assumption - Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not. - Note: other procedures than model transfer/delivery are decoupled with collaboration level y-z - Clarifying note: Level y includes cases without model delivery.

It is straightforward to see that, from 3GPP specification point of view, except model transfer, all other LCM components are commonly supported in level y and level z.

Observation 2: Except model transfer, all other LCM components are commonly supported in Level y and Level z.

Additionally, as mentioned earlier, RAN1 further agreed on the terminology of one-sided model (including UE-sided model and Network-sided model) and two-sided model. Different types of model deployment may lead to different signalling design impact to LCM. In the following section, we mainly discuss what are the signalling impact of different types of model deployment.

### Model Registration

Model Registration is seen as a procedure to request service for a specific use case.

There are two kinds of model registration: 1) network registers model from upper layer; 2) UE registers model from the network. When UE performs model registration, it may serve for two purposes, one is to request AI/ML service from network to the UE for UE-sided model or two-sided model, another is to request AI/ML operation is used for certain use case when network-sided model is considered.

Observation 3: The UE may perform model registration to request AI/ML service from the network or request network to perform AI/ML for a certain use case.

For the first registration type, it can be achieved by implementation. In this section, we mainly focus on how UE can register model from the network and analyse RAN2 impact.

Based on our understanding, AI/ML model for different use cases can be treated as a service for the UE. The UE can register and select proper AI/ML model based on broadcast information by the network. A similar procedure as multicast/broadcast interest indication can be considered for AI/ML service/model registration. The network can broadcast AI/ML service(s) it can support in the SIB information, and the UE indicates its interested service(s) by sending interest indication message to the network.

Proposal 3: A new SIB information is used by the network to broadcast its supported AI/ML use case(s). The UE indicates its interested service(s) via interest indication message. FFS on the details carried in SIB and interest indication.

### AI/ML related UE Capability

Compared with legacy physical layer processing, AI/ML model inference may have certain performance shift. If the AI/ML model is purely trained/inferenced at the UE side, e.g. UE-sided model, the network needs to be able to control whether the UE is using legacy approach or using AI/ML model. To allow network has the visibility of UE’s capability of AI/ML, the UE needs to report what use case it can support by AI/ML.

Proposal 4: UE should report supported AI/ML use case(s) to the network.

### Model Monitoring

Following agreements were reached during RAN1 #110bis-e meeting:

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

It can be observed that the model monitoring can be summarized in following two aspects:

1) monitoring wireless system performance: it can be defined per use case, considering UE throughput, packet delay, etc. For example, system performance KPIs in above RAN1 agreement.

2) monitoring model performance feedback: this includes inference accuracy, model bias, model variance, confidence level, drift detection of input/output data, etc. This information is generated by model inference function. For example, inference accuracy, AI/ML signalling overhead, computation and memory cost, power consumption, etc in above RAN1 agreement.

Observation 4: model performance feedback and wireless system performance are considered as input for model monitoring.

Therefore, the RAN2 impact comes when model performance feedback and wireless system performance needs to be exchanged over the air interface. For wireless system performance, similar as L2 measurement, the network is already able to observe each UE’s performance, as defined in TS38.314 [1]. This measurement is independent from where model inference is located.

Observation 5: As already supported by L2 measurement, network can monitor wireless system performance of AI/ML operation based on its own measurement or measurement report from UE, no matter AI/ML model inference is performed at UE side or network side.

As agreed by RAN1, model monitoring is used to serve the purpose of model activation, deactivation, selection, etc.

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

As agreed in RAN1 CSI feedback and Beam management use case, except UE-side model, it is more suitable to let the network be responsible for model monitoring, as it can reduce the overhead for controlling model selection, activation, etc.

CSI use case In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching BM use case Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Therefore, following scenarios are considered:

Option 1: UE-sided one-sided model and model monitoring at the UE side

Option 2: network-sided one-sided model and model monitoring at the network side

Option 3: two-sided model and model monitoring at the network side

Option 4: two-sided model and model monitoring at the UE side and report to the network

Since model inference and model monitoring are located in the same side/entity in Option 1 and Option 2, the model performance feedback can be exchanged within the same entity without specification impact. In Option 3/4, since the (part of) model inference function is located at a different entity, the model performance feedback should be reported by the UE towards the network.

Similar as measurement report, this model performance feedback can either report periodically or triggered by event.

RAN2 may consider adding other scenarios depending on further progress in RAN1.

Proposal 5: For UE-sided model and two-sided model, the UE reports its model performance feedback (e.g. inference accuracy, model bias, etc) to the network periodically or event-triggered. FFS on the detail information of model performance feedback. FFS on other scenarios depending on further progress in RAN1.

### Model selection/activation/deactivation/switching/fallback

Following agreements are achieved in RAN1 #110bis-e meeting on model selection and related procedure:

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

For network-sided model, the network should make the decision, where the model selection and related procedure can be done by implementation without specification impact.

Considering model selection/activation/deactivation/fallback/switching procedures may not be not frequently updated, therefore, RRC signaling can be considered to support those procedures. As captured in RAN1 above agreements, the main specification impact comes from UE-sided and two-sided model, where following signalling aspects should be considered by RAN2:

Scenario 1: UE-sided/two-sided model, network-side model monitoring, network initiated

Scenario 2: UE-sided/two-sided model, UE-side model monitoring, UE-initiated decision and request to network

Scenario 3: UE-sided/two-sided model, UE-side model monitoring, event triggered

Scenario 4: UE-sided/two-sided model, UE-side model monitoring, UE’s decision report to the network

Observation 6: RRC Signalling can used to support model selection /activation /deactivation /fallback /switching.

Proposal 6: RAN2 to study below four scenarios of RRC signalling impact:

1) Scenario 1: UE-sided/two-sided model, network-side model monitoring, network initiated

2) Scenario 2: UE-sided/two-sided model, UE-side model monitoring, UE-initiated decision and request to network

3) Scenario 3: UE-sided/two-sided model, UE-side model monitoring, event triggered

4) Scenario 4: UE-sided/two-sided model, UE-side model monitoring, UE’s decision report to the network

## Level z Specific RAN2 Impact (model transfer)

As agreed by RAN1, the main difference between level y and level z is whether model transfer is transparent to 3GPP or not. Therefore, model transfer specification impact is specifically studied to support level z.

### Multiple Model

It is possible that multiple AI models are used for the same functionality/use case. As agreed in RAN2 #119bis-e meeting, model ID is used to identify different models. Hence multiple model IDs are used to represent different models for the same functionality/use case.

Proposal 7: For the same functionality (use case), multiple models use different model IDs.

### Control Plane vs. User Plane

During RAN2 119bis-e meeting, RAN2 reached following agreements:

- General FFS: AIML Model delivery to the UE may have different options, Control-plane (multiple subvariants), User Plane, can be discussed case by case.

It is observed that whether to use control plane or user plane for model transfer depends on where the model is generated and stored.

If it’s generated by upper layer (e.g. application layer or 3rd party), AI/ML model can be treated as service packets, where user plane can be used.

If the model is stored in OAM, a control plane solution as Rel-17 QoE can be reused. In Rel-17 QoE, a new SRB is designed to transfer QoE container from the network to the UE with a low priority. Moreover, considering the large size of QoE container, RRC segmentation is also supported. For AI/ML model deliver, same mechanism can be used to deliver AI/ML model in control plane.

Observation 7: control plane can also support models with large size with the support of RRC Segmentation. Whether to use control plane or user plane for model transfer depends on the location of model generation and storage.

Considering AI/ML model may not be frequently updated, the transition of AI/ML model can be delivered with low priority. Therefore, if user plane solution is used, a new radio bearer can be designed for model delivery, e.g. machine learning radio bearer. Otherwise, if control plane solution is used, a new low priority SRB is used to carry AI/ML model. The priority of this new SRB can be the same as SRB4 for Rel-17 QoE.

Proposal 8: RAN2 to check with RAN1 on the assumption the location of model generation and storage, and consider following two options for model transfer:

1) If control plane solution is used, a new low priority SRB (same priority level as SRB4) is used for model transfer;

2) If user plane solution is used, a new type of radio bearer (e.g. machine learning radio bearer, MLRB) is used for model transfer.

### Meta Info for Model Description

It was agreed in RAN2 #119bis-e meeting that some meta information can be used to describe model information during model transfer:

- R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS.

Recalling that a new SIB is proposed to broadcast what AI/ML use case a network can provide. Together with the use case information, meta information for model description can also be broadcasted together. The meta information is mainly to let UE understand the complexity of AI/ML model, so that the UE can select and register a suitable AI/ML model based on its own capability.

Another example is when UE selects a model and request ACK from the network, it can also include the model description together with the request. With such information, network can decide whether to grant it or not.

As agreed in RAN1 for performance evaluation, following aspects can be considered as meta information:

- Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

For pre-/post-processing, since it cannot use accurate data to describe its complexity, the network can simply indicate “high”, “low”, etc as the computational complexity.

Proposal 9: Meta Information as model description includes model inference complexity (e.g. FLOPs), level of pre-/post processing, model size.

### AI/ML related UE Capability

Different from legacy UE capability, to support AI/ML, it requires UE has specific capability to be able to process the received AI/ML model. For example, to successful receive the model, the UE should have enough memory to store the trained model, therefore, the UE should report its supported maximum model size.

Additionally, the UE should also be able to compile and execute the received AI/ML model. To support that, the network needs to understand the supported AI/ML structure by the UE, e.g. CNN, LSTM, RNN, etc.

Performing AI/ML model inference may require a lot of power consumption at UE side. It is better for the UE to report its maximum battery capacity and its current battery status to the network, so that the network may consider a proper duration for model activation and deactivation.

Proposal 10: UE reports following AI/ML capability of the UE to the network:

- maximum model size,

- supported model structure,

- maximum battery capacity,

- current battery status, etc.

### Model Update

After running a certain time, the deployed model at the UE side may need to be updated. Following mechanism for model update can be considered from RAN2 point of view:

- Timer based

For UE-sided model, the network may configure a timer to let UE update its model upon timer expires. Alternatively, if the model is received from the network, the network can send an update AI/ML model to the UE upon timer expires. For the later case, it can be done by implementation.

- Model information based (e.g. Model confidence level/loss)

The main motivation to update model at one side is to maintain the high-accuracy level of AI/ML model and keep the inference result accurate. Therefore, the model update can be triggered based on the model performance, e.g. model confidence level, or loss function.

Confidence level is calculated at the model inference node, by comparing prediction results and the real value (real future input to model inference). If the confidence level is lower than certain threshold, a model update should be triggered to avoid using the wrong/inaccurate predicted results/actions. This could also be triggered by either network or UE.

- Performance feedback based

As discsussed earlier, performance feedback may trigger model selection/activation/etc. Similarly, it can also trigger model update.

Proposal 11: RAN2 to study signaling and protocol impact assuming following model update mechanism:

1) timer based model udpate 2) model information (e.g. confidence level/loss) based model udpate 3) performance feedback based.

# Conclusion

In this contribution, we analyze the potential RAN2 impact with following observation and proposal:

General

Proposal 1: RAN2 assume following network-UE collaboration levels and deployment options for further study on AI/ML air interface:

1) Option 1: Level x + one-sided model (either network-side model or UE-side model)

2) Option 2: Level y + one-sided model (either network-side model or UE-side model)

3) Option 3: Level y + two-sided model

4) Option 4: Level z + two-sided model

Level X

Observation 1: Data collection for model training and model inference for level x implementation-based AI/ML operation may be needed.

Proposal 2: RAN2 should wait for RAN1 progress on data collection to support level x for each use case before concluding RAN2 impact of level x.

Level Y/Z common aspects

Observation 2: Except model transfer, all other LCM components are commonly supported in Level y and Level z.

Model registration

Observation 3: The UE may perform model registration to request AI/ML service from the network or request network to perform AI/ML for a certain use case.

Proposal 3: A new SIB information is used by the network to broadcast its supported AI/ML use case(s). The UE indicates its interested service(s) via interest indication message. FFS on the details carried in SIB and interest indication.

Level Y/Z common AI/ML related UE capability

Proposal 4: UE should report supported AI/ML use case(s) to the network.

Model monitoring

Observation 4: model performance feedback and wireless system performance are considered as input for model monitoring.

Observation 5: As already supported by L2 measurement, network can monitor wireless system performance of AI/ML operation based on its own measurement or measurement report from UE, no matter AI/ML model inference is performed at UE side or network side.

Proposal 5: For UE-sided model and two-sided model, the UE reports its model performance feedback (e.g. inference accuracy, model bias, etc) to the network periodically or event-triggered. FFS on the detail information of model performance feedback. FFS on other scenarios depending on further progress in RAN1.

Model selection/activation/deactivation/fallback/switching

Observation 6: RRC Signalling can used to support model selection /activation /deactivation /fallback /switching.

Proposal 6: RAN2 to study below four scenarios of RRC signalling impact:

1) Scenario 1: UE-sided/two-sided model, network-side model monitoring, network initiated

2) Scenario 2: UE-sided/two-sided model, UE-side model monitoring, UE-initiated decision and request to network

3) Scenario 3: UE-sided/two-sided model, UE-side model monitoring, event triggered

4) Scenario 4: UE-sided/two-sided model, UE-side model monitoring, UE’s decision report to the network

Level Z

Model ID

Proposal 7: For the same functionality (use case), multiple models use different model IDs.

Control plane vs. user plane

Observation 7: control plane can also support models with large size with the support of RRC Segmentation. Whether to use control plane or user plane for model transfer depends on the location of model generation and storage.

Proposal 8: RAN2 to check with RAN1 on the assumption the location of model generation and storage, and consider following two options for model transfer:

1) If control plane solution is used, a new low priority SRB (same priority level as SRB4) is used for model transfer;

2) If user plane solution is used, a new type of radio bearer (e.g. machine learning radio bearer, MLRB) is used for model transfer.

Meta info

Proposal 9: Meta Information as model description includes model inference complexity (e.g. FLOPs), level of pre-/post processing, model size.

Level Z AI/ML related UE capability

Proposal 10: UE reports following AI/ML capability of the UE to the network:

- maximum model size,

- supported model structure,

- maximum battery capacity,

- current battery status, etc.

Model update

Proposal 11: RAN2 to study signaling and protocol impact assuming following model update mechanism:

1) timer based model udpate 2) model information (e.g. confidence level/loss) based model udpate 3) performance feedback based.

# References

- TS38.314, Layer 2 Measurements

# Annex (Functional Framework)

## Option 1 (Level x + One-sided Model)

## Option 2 (Level y + One-sided Model)

## Option 3 (Level y + Two-sided Model)

## Option 4 (Level z + Two-sided Model)