3GPP TSG-RAN WG2 Meeting #120	R2-2211470

# Toulouse, France, 14 – 18 November, 2022

Agenda Item:	6.6.3

Source:	Ericsson

Title:	On HARQ process offset

Document for:	Discussion, Decision

# 1	Introduction

One still unsettled issue regarding SDT is the question of whether harq-ProcID-Offset can be configured for SDT. This was discussed in the offline discussion [1] where there was no consensus. In the last RAN2 meeting the issue was not discussed. In this contribution we present our view of this issue.

# 2	Discussion

In the specifications, there are two parameters which can be used to configure an offset for HARQ process IDs for configured grant i.e. harq-ProcID-Offset and harq-ProcID-Offset2. The field description in 38.331 for these parameters is

Both can be configured with the same value range , INTEGER (0..15). As is also clear from the field description that harq-ProcID-Offset is for shared spectrum, i.e. NR-U. Furthermore, in the formulas for deriving harq process ID for a CG transmission occasion in SDT in [3], the parameter harq-ProcID-Offset has no effect, only harq-ProcID-Offset2 will impact the harq process id for a CG transmission occasion. This means that even if it would be configured, it would make no difference when it comes to the operation of CG-SDT.

Configuration of harq-ProcID-Offset will not impact the HARQ process ID derivation for CG-SDT.

The parameter harq-ProcID-Offset2 can be used the introduce more flexibility especially when multiple CG-SDT configurations are used. This parameter is applicable to CG-SDT. Therefore, we suggest RAN2 to close this issue without further actions.

No spec change is needed with respect to harq-ProcID-Offset.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Configuration of harq-ProcID-Offset will not impact the HARQ process ID derivation for CG-SDT.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No spec change is needed with respect to harq-ProcID-Offset.

# 4 References

- 3GPP R2-2208703, Report for Rel-17 Small data and URLLC/IIoT.

- 3GPP TS 38.331 V17.2.0 (2022-09), Radio Resource Control (RRC) protocol specification (Release 17)

- 3GPP TS 38.321 V17.2.0 (2022-09), Medium Access Control (MAC) protocol specification (Release 17)