3GPP TSG-RAN WG2 Meeting #120		R2-2211495

Troulouse, France, 14th Nov. – 18th Nov. 2022

Source:	vivo

Title:	Uplink XR Traffic Information for Power Saving

Agenda Item:	8.5.3.2

Document for:	Discussion and Decision

- Introduction

In XR SID [1], the objective of XR specific power saving is as follows:

Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

In RAN2#119bis-e meeting, RAN2 discussed the XR Specific Information that is considered to be useful for RAN power saving. And the following agreement has been reached:

1. RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings. 2. RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN.

Meanwhile, an LS[2] was received which includes the latest SA2 agreement on the XR Specific Information that may be helpful for the core network to provide RAN for power saving.

In this contribution, we provide our views on what traffic and QQoS-relatedinformation on uplink traffic should be provided to RAN for UE power savings and how the information may be provided to RAN.

- Discussion

According to the LS from SA2[2], the following traffic and QQoS-relatedinformation[3] is concluded to be useful for RAN for power saving and can be provided from CN to RAN.

Conclusions 8.X Interim Conclusion for Key Issue #8 The following information, to be provided to the NG-RAN at PDU Session Establishment/Modification via an NGAP Message, is taken as baseline for normative work: - Periodicity for UL and DL traffic of the QoS Flow. In addition to integer periodicity values, non-integer values associated to, e.g., 45FPS, 60 FPS, 90FPS, 120FPS, shall be supported. Such information shall be exchanged by re-using/extending the TSCAI/TSCAC definitions in TS 23.501 clause 5.27.2.1. Editor's Note: If the PDU sets with the Periodicity (e.g., 45FPS, 60 FPS, 90FPS, 120FPS) are mapped into different QoS Flows, it is FFS whether the same Periodicity still exists and whether the same Periodicity can be used for the UL and DL traffic of each QoS Flow. NOTE 1: The above information can be provided to the 5GC by the AF via an NEF API. The 5GC can further derive, or be configured, with such information. Editor's Note: How 5GC derives the above information is FFS. - Traffic jitter information associated with each periodicity. Editor's Note: How 5GC derives the above information is FFS. The AF may provide assistant information to 5GC and assist the UPF to detect the end of Data Burst: NOTE 2: Whether this assistant information based on indication of only one media unit within each data burst or based on RTP extension header or both can be discussed in normative work. The following information for DL traffic, to be provided to the NG-RAN with in-band signaling via GTP-U header, is taken as baseline for normative work: - Optional, End of Data Burst indication in the header of the last PDU of the Data Burst. NOTE 3: It is assumed that the PDU with the End of data burst indication is received by the NG-RAN after all other PDUs of the Data Burst. The UPF detects the end of a Data Burst the and marks the End of Data Burst indication over GTP-U based on information provided by the AS in the PDU (e.g., “End” in the RTP extended header when only one media unit (e.g. NAL Unit) within each data burst).

Among the above information agreed, we think the counterpart of only some information of uplink traffic is useful for power saving.

- For UL XR traffic, useful information for power saving:

  - Periodicity for UL traffic of the QoS Flow： this can be helpful for e.g., configuring the periodicity of CDRX and PDCCH monitoring for dynamic UL grant.

  - End of Data Burst indication: this can be helpful for gNB, e.g., to indicate the UE to dynamically skip PDCCH monitoring once the last PDU of the burst is received.

- For UL XR traffic, useless information for power saving:

  - Traffic jitter information associated with each periodicity: in our understanding, the uplink jitter can be neglected since the UL data is generated by UE itself.

Proposal 1: Among the traffic information agreed to be provided from CN to RAN for power saving, the counterpart of uplink traffic which is useful for power saving includes: periodicity for UL traffic of the QoS Flow, end of Data Burst indication. UL traffic jitter information is not useful for power saving.

In addition, we think the following information of uplink traffic is helpful for power saving:

- Start time of the first PDU of a PDU set and PDU set size (number of bits): this can help gNB scheduling with a UE power efficient way, e.g., gNB can give up following UL scheduling for the remaining data of a certain PDU set when the PSDB of the PDU set has exceeded.

- PDU set identity and relationship information among PDUs within the same PDU set:  UE can use this information for early PDU dropping.

Proposal 2: the following information of uplink traffic is useful for power saving: start time of the first PDU of a PDU set and PDU set size (number of bits), PDU set identity and relationship information among PDUs within the same PDU set.

Among the above information of uplink traffic which is considered useful for power saving, periodicity for UL traffic of the QoS Flow has been agreed in SA2 to be provided to RAN from CN, PDU set identity and relationship information among PDUs within the same PDU set is used by UE itself; other information needs to send from UE to gNB. The details are following:

- End of Data Burst indication: UE can send an indication to gNB when the last PDU of a data burst in UL buffer has been sent to gNB. The indication can be a UCI or MAC CE.

- Start time of the first PDU of a PDU set and PDU set size (number of bits):  current BSR is used to indicate the UL buffer status to gNB. Start time and size of PDU set are some kinds of enhanced information related to the buffer status. Hence, we think this information can be reported by extending the current BSR.

Proposal 3: UE sends an indication to gNB when the last PDU of a data burst in UL buffer has been sent to gNB. FFS whether the indication is a UCI or MAC CE.

Proposal 4: Start time and size of PDU set are reported by extending the current BSR. Details are FFS.

- Conclusion

In this contribution, we provide our views on what traffic and QoS related information on uplink traffic should be provided to RAN for UE power savings and how the information may be provided to RAN.

Based on the discussion, we have the following proposals:

Proposal 1: Among the traffic information agreed to be provided from CN to RAN for power saving, the counterpart of uplink traffic which is useful for power saving includes: periodicity for UL traffic of the QoS Flow, end of Data Burst indication. UL traffic jitter information is not useful for power saving.

Proposal 2: The following information of uplink traffic is useful for power saving: start time of the first PDU of a PDU set and PDU set size (number of bits), PDU set identity and relationship information among PDUs within the same PDU set.

Proposal 3: UE sends an indication to gNB when the last PDU of a data burst in UL buffer has been sent to gNB. FFS whether the indication is a UCI or MAC CE.

Proposal 4: Start time and size of PDU set are reported by extending the current BSR. Details are FFS.

- References

- RP-220285, Study on XR Enhancements for NR, Nokia

- S2-2209979 LS on XR and Media Services, SA2

- S2-2209939 Key Issue #8: Evaluation and conclusion, Qualcomm Incorporated, Huawei, Nokia, Nokia Shanghai Bell, Tencent, Tencent Cloud, Vivo