3GPP TSG-RAN WG2 Meeting #120		R2-2211496

Toulouse, France, 14th Nov. – 18th Nov. 2022

Source:	vivo

Title:	Discussion on feedback enhancements for XR-specific capacity improvements

Agenda Item:	8.5.4.1

Document for:	Discussion and Decision

- Introduction

In RAN2#119bise [1], there were the following agreements on BSR enhancements for XR:

1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). Delay information consists of at least “remaining time”. 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

According the agreements, RAN2 agreed to refine the granularity for buffer status level report and to discuss whether to carry delay information in BSR. In this contribution, we will discuss the BSR enhancements considering the tradeoff among BSR contents, BSR overhead and potential benefits from BSR enhancements. In addition, we will also share our view on BSR triggering.

- Discussion

  - Buffer status level granularity enhancements

In previous discussion, we observed that the main motivation to improve the existing buffer status level granularity is to conquer the quantization error in case of large video data volume in R15/16/17. The easiest way is to introduce the new quantization table and use more bits to indicate the buffer status level in the BSR.

- RAN2 support to use more bits (> 8bits) to report the buffer status level according new quantization table with finer granularity.

For a UE with XR service, this UE can also have associated signaling/data traffics with relatively low data volume, for instance, the RRC signaling, the voice data associated with the video traffic for XR. Furthermore, there could be the other low data volume traffics coexisting with XR traffic. In this case, if BSR is triggered, both LCGs with low data volumes and LCGs with high data volume should be reported if there are available data for both low data rate service traffic (e.g. XR audio data, associated RRC signaling, other simultaneous background traffic) and high data rate traffic (e.g. XR video data). To ensure the accuracy of low data volume report, the buffer status level accuracy for low data volume case should not be degraded due to the refinement of the granularity of the high buffer status level.

- The granularity for low data volume quantization should be at least as good as in R15/16/17.

If large buffer status level field (> 8 bits) are used for all LCGs with new BSR, regardless of the maximum data volumes respectively for the LCGs, it will always generate a much larger BSR size than the legacy BSR, even though the actual buffer size of all/some LCGs are rather small. For these LCGs with low data volume, using new quantization table and more buffer status level bits does not help the capacity enhancement since the existing quantization table is good enough. To optimize the tradeoff between the BSR overhead and expected benefit to use long buffer size indication, one can consider to allow the UE to conditionally determine if to use finer granularity. The detail criteria and configurations on the conditions can be left for Stage3.

- RAN2 to support the method for UE to determine the quantization table from more than one quantization tables, e.g. based on certain UE configuration or indication from NW.

If the UE is allowed/configured to determine the quantization table per LCG, the UE can select the proper quantization table and report the buffer status level with the proper number of bits. This can avoid all LCGs have to use large buffer status level field if some of the LCGs to be reported needs to use large buffer status level field.

- RAN2 to support per LCG quantization table configuration for BSR report.

  - Delay information report in BSR

As XR video traffic is delay sensitive and the total delay budget is rather small than other traffics such as eMBB, quick report of the delay information with respect to the data in the transmission buffer of the UE is required. Delay information of buffered data in the transmission buffer of the UE can help the gNB to determine the available delay budget for the buffered data. It is expected that delay information in BSR can meet this requirement. From this perspective, we propose:

- Delay information of the queued data in the transmission buffer of the UE should be reported in BSR.

There is still one issue on whether/how to report the delay information for one/all LCGs. Considering the overhead of delay information report and the expected benefit, it is not optimal to always report the delay information for all LCGs once BSR is reported. For one instance, for the LCGs whose LCHs does not carry delay sensitive traffic, the gNB has no interest to know the exact delay information. For another instance, for one LCG with delay sensitive data, if all the data of an LCG in the transmission buffer still have large enough delay budget for the gNB to perform scheduling, it is not necessary to inform the related delay information to the gNB as there is no urgent need of UL grant for this LCG.

- RAN2 to support per LCG delay information report.

When one LCG whose delay information should be reported, there is one issue one how to report the delay information. To optimize the delay information report, proper quantization for delay information report is required. The quantization table could be either predefined or provided via RRC signaling, wherein the former can save signaling overhead while the latter can provide flexibility for report in case of different e2e delay restrictions.

- RAN2 to support to introduce delay quantization table for delay information report.

  - New BSR triggering

To avoid frequently triggering of BSR with delay information and/or always include delay information for an LCG into the BSR, conditions for BSR report with delay information has to be defined or preconfigured. With the conditions, it is expected that the UE only report the BSR with delay information only when this delay information can really help the UL scheduling by the gNB. The conditions for BSR report with delay information in different situations (e.g. various processing delay performance of gNB and UE) could be different, wherein the exact conditions and the possible related configurations could be left for WI phase.

- RAN2 to support the delay triggered BSR report, FFS on the exact trigger conditions.

In Rel-15/16 NR, UL data arrives at a UE’s L2 buffer(s), and the UE requests UL-SCH resources for the UL data via SR/BSR mechanisms. In the BSR mechanism, several types of BSR are defined, i.e. ‘Regular BSR’, ‘Periodic BSR’ and ‘Padding BSR’. The volume of UL data is reported by a BSR MAC CE when at least one BSR has been triggered and not cancelled, and there are UL-SCH resources available for a new transmission and the UL-SCH resources can accommodate the BSR MAC CE plus its subheader.

The current regular BSR is triggered by the data arrival of an LCH with higher priority than all the LCHs with data available. The buffer change of the LCHs with already available data relies on the periodic BSR. However, the periodic BSR is only triggered by the expiration of the periodicBSR-Timer. If there is continuous data arrival for one LCH, the gNB cannot know the exact buffer status for the LCH according to the existing BSR mechanism as no new BSR will be triggered except the periodic BSR. Continuous UL data arrival of UL XR traffic is probably the real case as the application layer may take considerable time to perform the video encoding and deliver the encoded packets to the RAN layers gradually during the video encoding process. From this perspective, new triggering mechanisms to enable the gNB to be timely aware of the UE buffer change should be studied. One possible improvement could be that, for example, if a new packet or a new set of packets belonging to a logical channel /QoS flow /stream satisfying pre-defined requirement(s), or, the amount of data of the logical channel /QoS flow /stream pending for transmission exceeds a pre-defined threshold, or all data of a frame/burst has arrived, a BSR can be triggered. As a result, the gNB can know the UE buffer status change in time, which can further facilitate timely and efficient UL scheduling.

- RAN2 to study the following BSR triggering enhancements:

  - Timely buffer status report due to subsequent data arrival of an LCH;

  - Timely buffer status report due to data arrival for a whole burst or PDU set.

- Conclusion

In this contribution, we have discussed the XR specific feedback enhancements based on the XR traffic characteristics. Based on the discussions and the observations, we have the following proposals:

Proposal 1	RAN2 support to use more bits (> 8bits) to report the buffer status level according new quantization table with finer granularity.

Proposal 2	The granularity for low data volume quantization should be at least as good as in R15/16/17.

Proposal 3	RAN2 to support the method for UE to determine the quantization table from more than one quantization tables, e.g. based on certain UE configuration or indication from NW.

Proposal 4	RAN2 to support per LCG quantization table configuration for BSR report.

Proposal 5	Delay information of the queued data in the transmission buffer of the UE should be reported in BSR.

Proposal 6	RAN2 to support per LCG delay information report.

Proposal 7	RAN2 to support to introduce delay quantization table for delay information report.

Proposal 8	RAN2 to support the delay triggered BSR report, FFS on the exact trigger conditions.

Proposal 9	RAN2 to study the following BSR triggering enhancements:

-	Timely buffer status report due to subsequent data arrival of an LCH;

-	Timely buffer status report due to data arrival for a whole burst or PDU set.

- References

- RAN2#119bise, Chairman notes