3GPP TSG-RAN WG2 Meeting #120		R2-2211497

Toulouse, France, 14th Nov. – 18th Nov. 2022

Source:	vivo

Title:	Discussion on scheduling enhancements XR-specific capacity improvements

Agenda Item:	8.5.4.2

Document for:	Discussion and Decision

- Introduction

In RAN2#119bise [1], there were the following agreements with respect to the scheduling enhancements for XR:

1: Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting. 2: RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

In the meanwhile, in RAN1#110e[2] there were the following related agreements and conclusions.

Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration Conclusion No further discussion in RAN1 for Rel-18 XR to extend the support of legacy single DCI scheduling multi-PDSCHs for FR2-2, to other SCS in FR1/FR2-1.

In this contribution, we will discuss XR scheduling enhancements including CG scheduling and assistant information report from UE, based on the respective related agreements from RAN1 and RAN2.

- Discussion

  - Configured grant (CG) scheduling enhancements

Considering the tight delay budget of XR service, CG scheduling could be considered as a beneficial approach for uplink scheduling as the SR transmission procedure in case of dynamic scheduling could be avoided. However, as the burst size of XR traffic varies, in order to provide enough transmission capacity for the various burst sizes, the network has to configure a set of CGs corresponding to one burst in order to provide enough capacity for the burst with large size. In this way, some CGs in the CG cluster may be redundant if the size of the corresponding burst is small, which may cause resource waste from system capacity point of view. Figure 1 illustrates the redundant CGs in relation to burst size variations.

Figure 1 Illustration of Recyclable CGs

- If only CG is used for XR data transmission, the network has to provide large enough capacity for various burst size. The provided capacity may be redundant for some small burst(s) and cause resource waste.

According the agreements from RAN1, CG recycling is already considered as one option for CG enhancement. With CG recycling, the redundant CGs can be recycled and the corresponding radio resources can be rescheduled for other data transmissions. In this way, the redundant capacity providing via configured scheduling can be avoided.

- With CG recycling, the gNB can reschedule the radio resources corresponding to the recycled CGs for other data transmission so that the resource waste due to the CG overproviding can be avoided.

Considering the potential benefits from CG recycling, we propose:

- RAN2 to support the CG recycling in case of CG based UL XR data transmission.

After determining the CG(s) to be recycled within one set of CGs, the gNB still needs certain time to perform UL scheduling in order to utilize the radio resources corresponding to the recycled CGs. From this perspective, it is expected that the gNB can determine the recycled CG(s) within one set of CG as soon as possible. As the gNB does not know the exact amount of data to be transmitted within a CG cluster, certain assistant information from UE is necessary for the serving gNB to determine the recycled CG(s). For one instance, the UE can determine amount of data (e.g. the data burst containing one whole XR frame) that is required to be transmitted within a CG cluster and report such information to the gNB as soon as possible, either using a MAC CE or CG-UCI associated with the PUSCH at CG occasions. Figure 2 illustrates that the UE reports assistant information for CG recycling once it has determined the amount of data to be transmitted in the current CG cluster. For another instance, the UE can report the CGs to be used or not used. The gNB can then determine the recycled CGs according the received assistant information.

Figure 2 Assistant information from UE to gNB for CG Recycling

- RAN2 to support the following assistant information from UE to gNB for CG recycling:

  - Indication of unused/used CGs; or

  - BSR triggering upon entire UL burst/PDU set arrival.

In our companion paper, simulation results with respect to CG recycling with considerable capacity gain are observed in some cases[3].

  - Other enhancements

There are also some other CG enhancements, e.g. CG enhancement considering the non-integer XR burst periodicity, which may lead mismatch between XR burst arrival and CG configuration. This part is expected to be discussed in RAN1, and RAN2 may discuss the potential impacts upon RAN1’s input.

- RAN2 to wait for RAN1 input regarding time granularity mismatch between XR burst arrival and CG configuration.

The UE may have timely information regarding the UL XR data charateristics which can fascilitate the related UL scheduling by the gNB. For example, the application layer may predict the amount of data and the arrival time for the coming frame based on the achieved raw picture data before the encoding/compression completion and inform this information to RAN protocol layers and these information could be used to improve the SR/BSR procedure. For instance, the UE can be allowed to trigger SR request based on the predicted data arrival time notified by the application layer. The UE may also report the related information to gNB.

- RAN to support scheduling enhancement based on predicted the UL XR data arrival (e.g. predicted arrival time and amount of data) with the assistance of UE Application layer.

- Conclusion

In this contribution, we have discussed the scheduling enhancement for XR capacity. Based on the discussions, we have the following observations:

Observation 1	If only CG is used for XR data transmission, the network has to provide large enough capacity for various burst size. The provided capacity may be redundant for some small burst(s) and cause resource waste.

Observation 2	With CG recycling, the gNB can reschedule the radio resources corresponding to the recycled CGs for other data transmission so that the resource waste due to the CG overproviding can be avoided.

Based on the discussions and the observations, we have the following proposals:

Proposal 1	RAN2 to support the CG recycling in case of CG based UL XR data transmission.

Proposal 2	RAN2 to support the following assistant information from UE to gNB for CG recycling:

a.	Indication of unused/used CGs; or

b.	BSR triggering upon entire UL burst/PDU set arrival.

Proposal 3	RAN2 to wait for RAN1 input regarding time granularity mismatch between XR burst arrival and CG configuration.

Proposal 4	RAN to support scheduling enhancement based on predicted the UL XR data arrival (e.g. predicted arrival time and amount of data) with the assistance of UE Application layer.

- References

- 3GPP RAN2#119bis, Chairman notes

- 3GPP RAN1#110e, Chairman notes

- R1-2211025, Discussion on XR specific capacity enhancements, 3GPP RAN1#111