3GPP TSG-RAN WG2 #120	R2-2211502

# Toulouse, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1462 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38.321 on IUC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following issues are idenfitied to be addressed for IUC. 1.Issues for IUC trigger - IUC information MAC CE can be triggered by either a condition or an explicit request which is indicated by sl-IUC-Condition and sl-IUC-Explicit respectively, however both parameters are not used for IUC trigger in clauses 5.22.1.9 and 5.22.1.10. - sl-TriggerConditionRequest is used in RRC to indicate the trigger condition of an explicit request from UE-B to UE-A. however this parameter is not used for IUC request MAC CE in clause 5.22.1.9. - sl-TriggerConditionCoordInfo is used in RRC to Indicate the trigger condition of inter-UE coordination information from UE-A to UE-B when it is triggered by a condition, however this parameter is not used for IUC information trigger in clause 5.22.1.10. 2. In clause 5.22.1.1, the terminologies UE-B and UE-A are referred in the Note 3B2, however, there is no definition for the two terminologies. NOTE 3B2: When UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or different UE-As, when UE-B has own sensing results, it is up to UE-B implementation to use the preferred resource set in its resource (re)selection for transmissions to the UE A providing the preferred resource set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1st change: in clause 5.22.1.9, includes texts for checking the value of sl-IUC-Explicit and sl-TriggerConditionRequest for triggering IUC request MAC CE. 2nd change: in clause 5.22.1.10, includes texts for checking the value of sl-IUC-Explicit and sl-IUC-Condition when determing whether IUC reporting procedure can be triggered. In addition, when the Sidelink Inter-UE Coordination reporting procedure is triggered by a condition, includes texts for checking the value of sl-TriggerConditionCoordInfo. 3rd change: in Note 3B2, removes terminologies of UE-B and UE-A. Impact analysis Impacted 5G architecture options: NR standalone Impacted functionality: SL IUC Inter-operability: If the network implements the changes but not the UE, the UE and the nework may have misaligned interpretations on IUC trigger conditions. If the UE implements the changes, but not the network, the UE and the nework may have misaligned interpretations on IUC trigger conditions. If one UE implements the change but not another UE, there ismisaligned between UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the changes are not approved, there may be misaligned interpretations on IUC trigger conditions between UEs and between UEs and the gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.1, 5.22.1.9. 5.22.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< First group of changes start >

< Unmodified parts omitted >

#### 5.22.1.9	IUC-Request transmission

The Sidelink Inter-UE Coordination Request (SL-IUC Req) transmission procedure is used to trigger a peer UE to transmit Sidelink Inter-UE Coordination Information as specified in clause 8.1.4 of TS 38.214 [7] when sl-IUC-Explicit

is configured with value enabled by RRC (see clause 6.3.5 of TS 38.331 [5]). If sl-TriggerConditionRequest is configured with vlalue 0, the Sidelink Inter-UE Coordination Request can be triggered the UE’s implementation. If sl-TriggerConditionRequest is configured with vlalue 1, the Sidelink Inter-UE Coordination Request can be triggered only when the UE has data to be transmitted to the peer UE (see clause 6.3.5 of TS 38.331 [5]).

#### 5.22.1.10	IUC-Information Reporting

The Sidelink Inter-UE Coordination Information (SL-IUC Info) reporting procedure is used to provide a peer UE with inter-UE coordination information as specified in clause 8.1.4 of TS 38.214 [7]. The Sidelink Inter-UE Coordination reporting procedure can be triggered by reception of a Sidelink Inter-UE Coordination Request MAC CE when sl-IUC-Explicit

is configured with value enabled by RRC or a condition when sl-IUC-Condition is configured with value enabled by RRC (see clause 6.3.5 of TS 38.331 [5] and clause 8.1.4 of TS 38.214 [7]). When the Sidelink Inter-UE Coordination reporting procedure is triggered by a condition, if sl-TriggerConditionCoordInfo is configured with value 0, the Sidelink Inter-UE Coordination reporting procedure is triggered by the UE’s implementation while if sl-TriggerConditionCoordInfo is configured with value 1, the Sidelink Inter-UE Coordination reporting procedure is triggered only when the UE has data to be transmitted together with the inter-UE coordination information to the peer UE.

RRC configures the following parameter to control the SL-IUC Information reporting procedure:

-	sl-LatencyBoundIUC-Report, which is maintained for each PC5-RRC connection.

The MAC entity maintains an sl-IUC-ReportTimer for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection. sl-IUC-ReportTimer is used for an SL-IUC Information reporting UE to follow the latency requirement signalled from an IUC-Information triggering UE. The value of sl-IUC-ReportTimer is the same as the‎ latency requirement of the SL-IUC Information in sl-LatencyBoundIUC-Report configured by RRC.

The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers:

1>	if the SL-IUC Information reporting has been triggered by an SL-IUC Request MAC CE (and/or an SCI) and not cancelled:

2>	if the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting is not running:

3>	start the sl-IUC-ReportTimer.

2>	if the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting expires:

3>	cancel the triggered SL-IUC Information reporting.

2>	else if the MAC entity has SL resources allocated for new transmission and the SL-SCH resources can accommodate the SL-IUC Information MAC CE and its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate a Sidelink Inter-UE Coordination Information MAC CE as defined in clause 6.1.3.35;

3>	stop the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting;

3>	cancel the triggered SL-IUC Information reporting.

< Unmodified parts omitted >

< First group of changes end >

< Second group of changes start >

#### 5.22.1.1	SL Grant reception and SCI transmission

< Unmodified parts omitted >

NOTE 3B2:	When the UE receives both a single preferred resource set and a single non-preferred resource set from the same peer UE or different peer UEs, when the UE has own sensing results, it is up to the UE implementation to use the preferred resource set in its resource (re)selection for transmissions to the peer UE  providing the preferred resource set.

< Unmodified parts omitted >

< Second group of changes end >