3GPP TSG-RAN2#120										     R2-2211526

Toulouse, France, 14-18 Nov’22

Source: 			ZTE Corporation, Sanechips

Title: 	PDU-set prioritization for XR

Agenda item:	8.5.2.2

Document for: Discussion and Decision

# Introduction

In this contribution we discuss how prioritization between PDU-sets can be implemented for XR traffic.

# Handling of inter-PDU set priority/dependency

Different PDU-sets may be associated with different priorities. Further there may be some dependency between PDU-sets. For instance, I frames in the XR traffic are generally associated with higher priority and are associated with a overall lower PSER. The P and B frames are of lower priority and may be associated with higher PSER. Further, there are dependencies between different PDU-sets. For instance, I frames are required at the application layer to be able to decode the dependent P and B frames.

The inter-PDU set priority within the UE can be handled using the LCP procedure. With this approach, different PDU sets are assumed to be mapped to different logical channels. The priority handling between PDU-sets then can reuse the existing LCP procedure in UL.

Proposal 1: For inter-PDU set priority handling in UL, the existing LCP procedure can be used as baseline

However, the existing LCP procedure does not take into account the dependencies between different logical channels. We need to discuss how the LCP procedure should be enhanced to take into account the dependencies between different PDU-sets. As an example if an I frame is lost over the air (i.e. NACK is received), then the LCP procedure should prioritize the retransmissions of the I frame over transmission of any associated P or B frames. RAN2 should consider enhancements to the LCP procedure to enable handling of such inter-PDU-set dependencies within the XR traffic.

Proposal 2: If SA2 agree to specify the mechanisms for inter-PDU set dependencies, RAN2 can discuss the enhancements to LCP procedure for inter-PDU set dependency handling

Proposal 3: The LCP procedure should be enhanced to prioritize the retransmissions of the unacknowledged higher priority PDU-sets over the transmission of PDU-sets which are dependent on these unacknowledged PDU-sets

# Conclusion

The following proposals are made in this contribution:

Proposal 1: For inter-PDU set priority handling in UL, the existing LCP procedure can be used as baseline

Proposal 2: If SA2 agree to specify the mechanisms for inter-PDU set dependencies, RAN2 can discuss the enhancements to LCP procedure for inter-PDU set dependency handling

Proposal 3: The LCP procedure should be enhanced to prioritize the retransmissions of the unacknowledged higher priority PDU-sets over the transmission of PDU-sets which are dependent on these unacknowledged PDU-sets

# References

- S2-2201087, Key issue on traffic characteristics of XR and media service, SA2#149-e

- 3GPP TR 23.700-60, V0.3.0, Study on XR (Extended Reality) and media services