3GPP TSG-RAN2#120	   	 					 			R2-2211530

Toulouse, France, November 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Feedback enhancements for XR capacity

Agenda item:	8.5.4.1

Document for: Discussion and Decision

# Introduction

In RAN2#119bis-e meeting, the following agreements has been agreed[3]:

- 1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase). - Delay information consists of at least “remaining time”. - 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase. - If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

In this contribution, we will further discuss the remaining issue of UE feedback enhancements for XR capacity.

# BSR reporting enhancements

For XR service with jitter, dynamic scheduling is necessary.  Since the UL dynamic scheduling depends on the BSR reporting from UE, the accuracy of buffer status level reported impacts the resource scheduling efficiency. The more accurate the report is, the more efficiently resources can be used which results in less padding bits, lower scheduling delays and higher XR capacities.

In the current specification, there are two tables which are used by the UE for BSR reporting: one table is 5-bit Buffer Size field used when the short BSR is triggered, and the second table is 8-bit Buffer Size field used for long BSR reporting. In both tables, the larger the index is, the larger the buffer size value granularity becomes. In other words, the higher the index is, the larger the quantization error becomes. Although there is a RAN2 agreement to introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates), considering the large value range of BS levels, it is still not optimal to minimise quantization errors. One option to avoid these errors would be to include multiple BSRs for the same LCG. The actual value of the reported BSR is the sum of these BSR values. With this approach, the UL buffer size with a very fine granularity can be reported by the UE, improving the overall scheduling efficiency. Based on the current specification, a MAC PDU shall contain at most one BSR MAC CE. To report the buffer size level more accurately, it should be allowed that a MAC PDU contains more than one BSR MAC CEs with same LCG, and the reported buffer size of the LCG equals to the sum of buffer size indicated by each BSR MAC CE for that LCG in the MAC PDU.

Proposal 1: To enable finer granularity for BSR, more than one BSR MAC CE for same LCG can be included in one MAC PDU and the actual buffer size value is calculated using all the BSR MAC CEs for the same LCG included in the MAC PDU.

Considering that the table with 8-bit Buffer Size field can indicate finer granularity and larger range of buffer size level than the table with 5-bit Buffer Size field, to report more accurate buffer size level when only one LCG has data available for transmission, long BSR should be used. But based on the current specification, Long BSR(e.g. use the table with 8-bit Buffer Size field) can be reported only when more than one LCG has data available for transmission.

Proposal 2: RAN2 should specify mechanisms to enable Long BSR to be reported even if only one LCG has data available for transmission.

Considering XR service may usually lead large Buffer size, but the buffer size value granularity of the table with 8-bit Buffer Size field is very large when the buffer size is large. For more accurate buffer size reporting, it may also be necessary to introduce a new buffer size table with 8-bit Buffer Size field for the large buffer size range. Once a new buffer size table is introduced, how to identify whether the new buffer size table or the old buffer size table is used should be considered. Usually, there are two options:

Option 1: UE report it’s new table supporting capability, and gNB configures whether the new table or the old table is used.

Option 2: gNB indicate it’s new table supporting capability, and UE differentiate whether the new table or the old table is used by UL LC ID.

Considering that option 2 is more flexible, e.g. UE can decide whether the new table or the old table is used based on the buffer size to be reported, we suggest to use option 2 for the new table identification.
Proposal 3: A new buffer size table with 8-bit Buffer Size field is introduced for the large buffer size range.

Proposal 3a: gNB sends an indication to UE to indicate whether it supports the new buffer size table, and UE use a new UL LC ID to indicate whether the new buffer size table is used for the BSR reporting.

For dynamic scheduling for UL periodic XR service with jitter, BSR reporting. How to allocate the UL grant for the BSR reporting when the burst arrival should be considered. Usually SR will be triggered firstly for BSR reporting. But based on the SR procedure(e.g. SR reporting-->PDCCH with UL grant for BSR-->BSR reporting-->PDCCH with UL grant for data transmission-->UL data transmission), the service delay will be large. In RAN1#109 meeting, the following agreement has been made:

Agreement When DG is used as the baseline scheme, for the performance evaluation scheduling, after SR is triggered, both BSR and UL data can be transmitted using the UL grant after SR. - Companies are encouraged to provide the size of resources by the first UL grant after SR

But the XR frame may always change, and UE and/or AMF may only predict and provide the least size of PDU set, it is impossible to predict the size for each PDU size. Thus, multiple UL data transmissions will be inevitable, which may lead large delay for the PDU set transmission. If the gNB can obtain the buffer size accurately before the first UL grant, the scheduling efficiency will be improved. To cater for the periodic XR traffic with jitter, C-DRX is usually configured and the UL burst may only arrive during the DRX on-duration time. If periodic UL resource(e.g. CG) for BSR reporting is configured during the DRX on-duration time, UE can report the buffer size in time, which can avoid the SR procedure and improve the UL scheduling efficiency. Since the UE will not report BSR outside the DRX on-duration, the periodic UL resource(e.g. CG) for BSR reporting should only be valid during the DRX on-duration time.

Proposal 4: periodic UL resource (e.g. CG) for BSR reporting is configured, which is valid only during the DRX on-duration time.

As for the “remaining time” reporting along with BSR, there are the following issues to be considered:

- UE cannot obtain the PDB and PSDB for the the non-Standardized 5QI case. If the “remaining time” is the remaining delay budget, then the PDB and/or PSDB should be provided to UE. One alternative way is to use PDCP discardTimer to evaluate the “remaining time”, but the PDCP discardTimer is configured per DRB, and the BSR is reported per LCG, thus, more discussion is needed on how UE can obtain a value for the “remaining time”.

- If the BSR is triggered once the PDU arrives at the UE PDCP, the remaining time reporting seems useless and gNB can schedule the UL grant based on the PDB, PSDB and priority. If the BSR is not triggered immediately when the PDU arrives at the UE PDCP, there will be different remaining time for different PDUs in the buffer corresponding to the LCG, which may need multiple MAC CEs to report the BSR and remaining time, and will lead large BSR payload. Further, the BSR may not actually be transmitted at the time when it is generated. So, the remaining time should take into account the actual BSR transmission time instance and report the remaining time accordingly. Otherwise, the remaining time reported will be ambiguous

Proposal 5: If “remaining time” is reported along with BSR, further discussion is needed on the BSR overhead and granularity (e.g. per LCG/Per LC/Per DRB), accuracy of the reporting (considering the reporting time vs time when the BSR is constructed at the UE).

# About LS response on XR and Media Services to SA2

In the LS on XR and Media Services[4], the following questions are provided:

- In KI#3 (Network exposure), SA2 has been studying what information is useful for the purpose of enablement of rate adaptation at application and how that can be exposed by 5GS to the server and agreed the conclusions in TR 23.700-60 clause 8 (see pCR S2-2209977 and S2-2209978). The purpose of rate adaptation is to reduce the influx of data to keep the buffer/queue length level low which gives low latency. Two variants of L4S marking are considered: (1) L4S marking in the NG-RAN node and (2) L4S marking by the PSA UPF based on information provided by NG-RAN. SA2 would like to ask RAN2 and RAN3 feedback on the following questions: - Q1: whether it is feasible for RAN to estimate congestion information per QoS flow, per DRB in downlink and uplink directions. - Q2: whether it is feasible for RAN to estimate congestion information per QoS flow in UL, per DRB in UL without UE impacts.

As for the congestion, gNB usually decides the cell congestion based on the scheduling delay, user data rate available, cell power output etc. Upon detecting that the cell is in congested state, gNB will select UE DRB and/or QoS flow to downgrade the QoS and therefore decrease the cell load. If congestion is detected for a UE/DRB then all QoS flows mapped to the DRB will deemed to be subject to congestion. In which case, the selected UE, DRB and/or QoS flow for QoS downgrading can be considered as in congested state, and gNB can notify the UPF or AF to decrease the data rate and/or frame rate from application. From this point of view, gNB can decide the congestion information per QoS flow, per DRB in downlink and uplink directions without UE impacts. The actual signalling to indicate congestion detected by the gNB to UPF/AF is up to RAN3 design.

Proposal 6: Respond to SA2 LS with both “Yes” for the two questions. E.g. RAN can estimate the congestion information per QoS flow, per DRB in downlink and uplink directions without UE impacts. The actual signalling mechanisms to indicate congestion from RAN to CN is up to RAN3.

# Conclusion

The following observations and proposals are made:

Proposal 1: To enable finer granularity for BSR, more than one BSR MAC CE for same LCG can be included in one MAC PDU and the actual buffer size value is calculated using all the BSR MAC CEs for the same LCG included in the MAC PDU.

Proposal 2: RAN2 should specify mechanisms to enable Long BSR to be reported even if only one LCG has data available for transmission.

Proposal 3: A new buffer size table with 8-bit Buffer Size field is introduced for the large buffer size range.

Proposal 3a: gNB sends an indication to UE to indicate whether it supports the new buffer size table, and UE use a new UL LC ID to indicate whether the new buffer size table is used for the BSR reporting.

Proposal 4: periodic UL resource (e.g. CG) for BSR reporting is configured, which is valid only during the DRX on-duration time.

Proposal 5: If “remaining time” is reported along with BSR, further discussion is needed on the BSR overhead and granularity (e.g. per LCG/Per LC/Per DRB), accuracy of the reporting (considering the reporting time vs time when the BSR is constructed at the UE).

Proposal 6: Respond to SA2 LS with both “Yes” for the two questions. E.g. RAN can estimate the congestion information per QoS flow, per DRB in downlink and uplink directions without UE impacts. The actual signalling mechanisms to indicate congestion from RAN to CN is up to RAN3.

# References

- 3GPP TR 23.700-60, V0.3.0, Study on XR (Extended Reality) and media services

- 3GPP TS 38.321, V17.1.0 (2022-09),Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification

- 3GPP, RAN2-119bise LTE DCCA MUSIM Slicing 71 GHz QoE XR (Tero)_2022-10-19-EOM

- 3GPP, R2-2211138, LS on XR and Media Services (S2-2209979; contact: vivo), SA2, RAN2#120, November, 2022