3GPP TSG-RAN WG2 Meeting #120	R2-2211551

Toulouse, France, 14 – 18 November 2022

Agenda item:	8.12.3

Source:	Nokia, Nokia Shanghai Bell

Title:	IAB node migration with same physical cell resources at logical DUs

WID/SID:	NR_mobile_IAB-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

After RAN2#119bis-e offline discussion R2-2211054 [AT119bis-e][022][eIAB] Dual Cells LS (AT&T), the following agreement was reached regarding full migration with two logical IAB-DUs at the migrating IAB node:

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

No LS is needed

Recall that the two scenarios discussed for full migration were:

-	Alt1: The two logical DUs of the migrating IAB node use the separate physical cell resources.

-	Alt2: The two logical DUs of the migrating IAB node use the same physical cell resources.

In this contribution we discuss explicitly acknowledging that scenario Alt2 may be handled by implementation and is not precluded.

# 2	Discussion

## 2.1	Dual DUs with same physical cell resources

In Rel-17, for Alt2, RAN1 and RAN2 made the following observations/conclusions in reply LSs sent to RAN3:

RAN1 to RAN3 in R1-2108529 (Reply LS on Inter-donor migration):

- When two cells use the same PCI, this may not necessarily require all the UEs to switch to another cell at one time if RAN2 can confirm that the current specification enables a RRC CONNECTED UE remains connected, while observing the change of NCGI, and no change to the PCI.

- When two cells use the different PCIs, this will require all the UEs to perform HO to another cell at one time, which pose a high load to RACH.

RAN2 to RAN3 in R2-2109143 (Reply LS on Inter-donor migration):

- On Q1 (“Whether the current specification enables a RRC CONNECTED UE remains connected, while observing the change of NCGI, and no change to the PCI?”), since NCGI is broadcast via SIB1, the change of NCGI can be achieved by updating SIB1. Therefore, RAN2 has not identified any issues for the case of NCGI change without accompanying PCI change.

- On Q2 (“Is it possible to use same PCI for cell1 and cell2, and support the HO from cell1 to cell2 without new impact to the UE (e.g. a legacy UE)?”), some companies indicate they see no issues with using the same PCI, while some companies raise some concerns including UE behaviour when PCI is not changed

Observation 1: In Rel-17, RAN1 did not preclude the scenario where the logical DUs use the same physical cell resources. In particular, for the scenario where both DUs use the same PCI, RAN1’s only concern was whether UEs could stay in RRC_CONNECTED.

Observation 2: RAN2 already concluded that UEs could remain in RRC_CONNECTED when the logical DUs use the same PCI, i.e. only the NCGI changes.

From the above, we may say that the Alt2 evaluation in RAN2 is not comprehensive enough to entirely preclude it from implementation in Rel-18. There were no critical technical issues raised that would prevent using the same PCI, only some “concerns”. Rather, a key reason why companies wished to press forward with Alt1 in Rel-18 was due to time constraints: in particular, lack of TUs allocated to RAN1 for Rel-18 mobile-IAB.

Observation 3: The scenario where two logical DUs of the migrating IAB node use the same PCI was not ruled out in Rel-18 based on rigorous technical evaluation.

In our view, it might still technically be possible to support Alt2 through implementation as discussed in R2-2210778.

Finally, we emphasize that sharing the same radio resources for the two logical DUs would optimize the resource utilization without the need to split resources between the DUs or to reserve e.g. another carrier just for the migration purposes. Although, RAN2 has agreed to focus on the Alt1 scenario in Rel-18, the usage of this solution could be less resource efficient than Alt2 (assuming Alt2 can be practically developed). Therefore, precluding Alt2 from implementation risks precluding a practical mobile IAB solution that is also beneficial in real network deployment scenarios.

Observation 4: Assuming it can be handled by implementation, the scenario where the two logical DUs of the migrating IAB node use the same physical cell resources could provide practical benefits for real network deployments vs the scenario where the logical DUs use separate physical cell resources.

Noting all the above, we are of the view that the Alt2 scenario should not be precluded from implementation and propose that this is acknowledged.

Proposal 1: RAN2 notes that the scenario where the two logical DUs of the migrating IAB node use the same physical cell resources and PCI is not precluded from implementation.

# 3	Conclusion

Based on the discussion in this contribution we made the following observations:

Observation 1: In Rel-17, RAN1 did not preclude the scenario where the logical DUs use the same physical cell resources. In particular, for the scenario where both DUs use the same PCI, RAN1’s only concern was whether UEs could stay in RRC_CONNECTED.

Observation 2: RAN2 already concluded that UEs could remain in RRC_CONNECTED when the logical DUs use the same PCI, i.e. only the NCGI changes.

Observation 3: The scenario where two logical DUs of the migrating IAB node use the same PCI was not ruled out in Rel-18 based on rigorous technical evaluation.

Observation 4: Assuming it can be handled by implementation, the scenario where the two logical DUs of the migrating IAB node use the same physical cell resources could provide practical benefits for real network deployments vs the scenario where the logical DUs use separate physical cell resources.

And proposed the following:

Proposal 1: RAN2 notes that the scenario where the two logical DUs of the migrating IAB node use the same physical resources and PCI is not precluded from implementation.