3GPP TSG-RAN WG2 Meeting #120                                    	R2-2211575

Toulouse, France: November 14-18, 2022

Agenda item:	7.2.3

Source:	Qualcomm Incorporated

Title:	Discussion on SA2 LS reply on UE capability for IoT NTN

Document for:	Discussion and Decision

# Introduction

In RAN2#119bis-e meeting, RAN2 agreed to stick to the agreement that UE reports its capability depending on where it is connected to TN or NTN cell. Following agreement was made and a LS was sent in [1].

Agreements:

- Send a reply LS to SA2 (Cc: CT1, RAN3) saying RAN2 confirms the preference to go for “option 2”, acknowledging that there might be at least CT1 impact (e.g. for new TAU trigger for UE capability update) and indicating that RAN2 will further discuss at the next meeting whether any enhancements are needed for connected mode mobility (e.g. about the support of RACS for eMTC NTN IoT), implying that additional impacts to other groups might also be expected.

In this document, we provide details to address the FFS aspects of the LS reply [1].

# Discussion

UE capability reporting

As per TS 36.331, the EUTRA capability container either belongs to NTN or TN depending on which network type (TN or NTN) the UE capability is reported.

### 5.6.3 UE capability transfer #### 5.6.3.1 General Figure 5.6.3.1-1: UE capability transfer The purpose of this procedure is to transfer UE radio access capability information from the UE to E-UTRAN. If the UE is NTN capable, the UE reports its E-UTRAN radio access capabilities for the network type (TN or NTN) to which it is connected. If the UE has changed its E-UTRAN radio access capabilities, the UE shall request higher layers to initiate the necessary NAS procedures (see TS 23.401 [41]) that would result in the update of UE radio access capabilities using a new RRC connection. NOTE: Change of the UE's GERAN UE radio capabilities in RRC_IDLE is supported by use of Tracking Area Update.

With this procedure, EUTRAN would be able to know specific features UE implemented in the network type (TN or NTN). RAN2 sent LS to SA2 [3] for possible alignment work as MME will have to store the TN and NTN containers separately and provide the correct one to TN eNB and NTN eNB.

However, SA2 replied in [2] that large MME impact is not preferable but RAN2 agreed to go with following less optimal solution recommended by SA2.

Option#2: UE triggers TAU update with UE capability update needed when moving between TN and NTN.

UE triggers TAU update with UE capability update needed when moving between TN and NTN.

From the above explanation, there will be no concept of such two containers stored in MME. The MME will store just a single container and currently there is no procedure to prevent MME or source eNB from providing NTN UE capability wrongly to TN eNB when UE moves from NTN to TN. This applies to RACS for eMTC.

The solution to this problem is that whenever the UE moves to different network type, it triggers the TAU update with UE capability update indication. It also brings other issues to resolve.

- TN Measurement configuration

- Handover between TN and NTN in eMTC

- RRC reestablishment between TN and NTN in eMTC

- NB-IoT and CP solution-based reestablishment between TN and NTN when UE is using CP CIoT optimization.

- RRC release with redirection to TN

- RACS for eMTC.

The first point to resolve is how the source eNB knows whether the UE supports any TN bands. Since the UE capability reported belongs to only the network where it is reported, the UE will not include any unnecessary band specific UE capabilities. One option is to add following clarification in the specification.

- To enable measurement configuration, release with redirection and handover, the UE may include list of supported band information.

However, it is not simple to include such information because of use of band combination lists with same number of entries and in the same order rules. For example, consider the UE is in NTN networks. It supports NTN bands n255 and n256. It also supports TN band band-1 and band-2 (example shows just 2 out of many TN bands). The following is the way how eMTC UE will report the list of supported bands.

SupportedBandListEUTRA = { band-maxFBI, band-maxFBI, band-1, band-2 }

SupportedBandListEUTRA-v9e0 = { band-256, band-256, FreqBandIndicator-v9e0 not included, FreqBandIndicator-v9e0 not included}

SupportedBandListEUTRA-v1250 = {256QAM supported, 256QAM unsupported, dl-256QAM-r12 not included, dl-256QAM-r12 not included}

In addition to this, we need to add following clarification in the specification.

In this case, the UE only appends E-UTRA TN band entries after NTN band entries in SupportedBandListEUTRA or SupportedBandListEUTRA-v9e0, without including any other UE TN capability associated with the TN bands.

Many other lists associated with the SupportedBandListEUTRA need to be included in the same order with same number of entries.

| SupportedBandListEUTRA Includes the supported E-UTRA bands. This field shall include all bands which are indicated in BandCombinationParameters. | - |
| --- | --- |
| SupportedBandListEUTRA-v9e0, SupportedBandListEUTRA-v1250, SupportedBandListEUTRA-v1310, SupportedBandListEUTRA-v1320 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandListEUTRA (i.e. without suffix). | - |

bandInfoNR-v1610 One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to a E-UTRA band listed in supportedBandListEUTRA except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig.

bandListEUTRA One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA.

| interFreqBandList One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. | - |
| --- | --- |

| mbms-SupportedBandInfoList One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. This list is included only if fembmsMixedCell or fembmsDedicatedCell is included. If mbms-SupportedBandInfoList-v1700 is included, the UE shall include the same number of entries, and listed in the same order, as in mbms-SupportedBandInfoList-r16. | - |
| --- | --- |

However, in our view, it is cleaner to introduce a new IE to report the supported list of bands to avoid breaking “same number of entries” rule as explained above. The other benefit is the target eNB (e.g., legacy TN eNBs or Rel-16 TN eNBs) will be able to determine the UE capability container is NOT usable when supplied by different network type or RAT type or MME.

UE-EUTRA-Capability-v1720-IEs ::= SEQUENCE {

ntn-Parameters-v1720						NTN-Parameters-v1720,

nonCriticalExtension						UE-EUTRA-Capability-v1730-IEs							OPTIONAL

}

UE-EUTRA-Capability-v1730-IEs ::= SEQUENCE {

rf-Parameters-v1730					RF-Parameters-v1730							OPTIONAL,

nonCriticalExtension						SEQUENCE {}							OPTIONAL

}

RF-Parameters-v11730 ::=				SEQUENCE {

basicHOsupportedBandListEUTRA-v1730         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicator-r11

}

Introduce a new IE to report the supported list of bands to avoid breaking “same number of entries” rule in the band combination list and to avoid confusion on applicability of the UE capability container.

Second point to discuss is how UE capability is updated while the UE is in RRC_CONNECTED. Following is the text specified in SA2 specification 23.401 for UE capability update with added draft text (in color font) as an example how SA2 may clarify the UE capability update in NTN scenario.

If the UE's non-UTRAN UE Radio Capability information changes while in ECM-IDLE state (including cases of being in GERAN/UTRAN coverage), the UE shall perform a Tracking Area Update indicating "UE radio capability update" when it next returns to E UTRAN coverage. If the UE moves from a cell of a terrestrial RAT to the same or different satellite RAT and vice versa e.g. from terrestrial NB-IoT to satellite NB-IoT or from satellite WB-E-UTRAN to terrestrial NB-IoT, the UE shall perform a Tracking Area Update indicating "UE radio capability update". When the UE is in ECM-IDLE with AS information stored (as defined in clause 4.11 for User Plane CIOT EPS optimisation), NAS shall trigger AS to establish a new RRC connection and not resume the existing one in order to send Tracking Area Update indicating "UE radio capability update". As a result of this, the access stratum in the UE will discard the AS information and establish a new RRC connection as defined in TS 36.331 [37].

From above, it is clear the UE will not trigger any UE capability update procedure while being in RRC_CONNECTED. To make handover work, network has to implement some change to handle the UE capability update procedure and RACS update procedure. This will have impact to RAN3 and SA2 specification on when UE radio capability container is not supplied, or UE capability ID is not supplied when there is TN NTN mobility.

One option is that we leave this aspect of making basic handover to network implementation. In this case, the source or target gNB needs to detect the handover is TN-NTN handover from implementation. This detection can be done by UE’s inclusion of list of supported bands and band specific parameters in the EUTRAN radio UE capability container as described above for Proposal 1. In addition, the target TN eNB can also check the HO preparation message from source and check the source SIB1 to identify whether the source cell is NTN. Then it is clear, for example, the target TN eNB is not supposed to be using the any UE radio UE capability supplied by NTN eNB or MME if it knows the UE is being handed over from NTN network.

HO preparation message includes source cell info and if TN target eNB knows source is NTN, it can ignore the supplied UE capability and request new UE capability from the UE.

It is also possible the target gNB is not supplied with UE radio capability in this case, as the source NTN gNB knows the target cell is TN and may not supply the UE capability ID or may include empty EUTRAN UE capability container. The empty or absence of EUTRAN UE capability container is indication that target eNB has to retrieve the UE capability from the UE.

In this case the target eNB triggers UE capability enquiry and UE reports the correct UE capability to the target eNB after completing handover with basic configuration as done for RRC connection setup procedure. After the target eNB obtains updated UE capability, it has to upload the updated UE capability to MME. Then MME has to trigger proper procedure to update the UE capability ID if RACS is supported. The MME should replace the possibly stored container with this new updated UE capability container and assign new UE capability ID to UE either using GUTI re-allocation or after UE does TAU in TAU ACCEPT.

We think this is not in the scope of RAN2 discussion but there may be some clarification needed in RAN3 and SA2 specification so they should check this aspect. We should send LS to RAN3 and SA2 informing RAN2 assumption and let them decide if any change in their specification is needed.

RAN2 assumes network implementation can make basic handover between TN and NTN work.

Upon detecting TN-NTN connected mode mobility, RAN2 assumes target eNB may enquire UE capability from UE, upload the updated UE capability to MME and MME may assign new UE capability ID either using GUTI re-allocation or after UE does TAU in TAU ACCEPT.

Send LS to RAN3 and SA2 informing above RAN2 assumption and let them decide if any alignment work is needed in their specification.

# Conclusion

Following observations and proposals are made.

Proposal 1	Introduce a new IE to report the supported list of bands to avoid breaking “same number of entries” rule in the band combination list and to avoid confusion on applicability of the UE capability container.

Observation 1.	HO preparation message includes source cell info and if TN target eNB knows source is NTN, it can ignore the supplied UE capability and request new UE capability from the UE.

Proposal 2	RAN2 assumes network implementation can make basic handover between TN and NTN work.

Proposal 3	Upon detecting TN-NTN connected mode mobility, RAN2 assumes target eNB may enquire UE capability from UE, upload the updated UE capability to MME and MME may assign new UE capability ID either using GUTI re-allocation or after UE does TAU in TAU ACCEPT.

Proposal 4	Send LS to RAN3 and SA2 informing above RAN2 assumption and let them decide if any alignment work is needed in their specification.

# References

[1] R2-2210846, “Response to Reply to LS on UE capability signaling for IoT-NTN”, from RAN2, To SA2.

[2] R2-2209197, “Reply to LS on UE capability signalling for IoT-NTN”, from SA2, To RAN2.

[3] R2-2208928, “LS On UE capability signalling for IoT-NTN”, from RAN2, to SA2.